| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0139 |
| --- | --- |

| Title: | AHG7 Related: Differential coding method for DLT in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li, Ping Wu No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 Kai Zhang*, Jicheng An*, Shawmin Lei† * MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 † MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | +86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn Shawmin Lei +886-3-5670766 ext. 25555 { kai.zhang, jicheng.an, shawmin.lei} @mediatek.com |
| Source: | ZTE Corporation, MediaTek Inc. |  |  |

_____________________________

# Abstract

In HTM8.0, depth loop-up table (DLT) is signalled in the video parameter set (VPS) for depth components. Statistical results show that DLT information takes about 84% of the total coding bits of VPS. To reduce the coding bits of DLT, single-view differential coding is proposed to perform lossless compression on DLT elements for every depth view component. This method was implemented on HTM8.0, and the experimental results demonstrate that compared with HTM8.0 anchors, an average of about 85% DLT bits are saved with a slight BD-rate reduction.

# Introduction

Simplified depth coding (SDC) and depth loop-up table (DLT) were adopted into HTM to improve the coding efficiency of depth components [1]. DLT maps a depth value with a bit depth of X in original input depth components to a table index value with a bit depth of Y. When using DLT, a depth value which is not used in depth components is approximated to the nearest one that already exists in DLT. When the actual used depth values in depth components are no more than 1 << (X – 1), Y will be smaller than X, and converting the original depth values to DLT index values reduces the bit-depth of the input depth components, which provides an average gain of about 0.2% for random access configurations [1].

In HTM8.0, DLTs for depth view components are constructed by pre-analyzing an intra period of original depth pictures and then coded in video parameter set (VPS) extension. Although DLT brings an evident coding gain, it takes large percentage of coding bits in VPS. Using the common text conditions (CTC) for HTM 8.0, the coding bits of DLT and VPS are counted as presented in Table 1. Note that for the test sequences of GT_fly, Poznan_street and Undo-dancer, the number of DLT values are above 128. As the original depth values are represented in 8 bits, no bit-depth reduction can be achieved by using DLT. Therefore, HTM8.0 disables DLT when coding the above mentioned sequences. As observed, DLT related syntax elements take about 84% of the coding bits of VPS.

Table 1.  Number of coding bits of DLT and VPS (HTM8.0, CTC)

| Sequence | View | HTM8.0 Anchor |  |  |
| --- | --- | --- | --- | --- |
|  |  | DLT bits (D0) | VPS bits (V0) | D0/V0 (%) |
| Balloon | 3 | 556 | 2135 | 85.06 |
|  | 1 | 642 |  |  |
|  | 5 | 618 |  |  |
|  | Total | 1816 |  |  |
| Kendo | 3 | 486 | 2093 | 84.76 |
|  | 1 | 638 |  |  |
|  | 5 | 650 |  |  |
|  | Total | 1774 |  |  |
| Newspaper | 4 | 607 | 2245 | 85.79 |
|  | 2 | 622 |  |  |
|  | 6 | 697 |  |  |
|  | Total | 1926 |  |  |
| Poznan_hall2 | 6 | 421 | 1503 | 78.78 |
|  | 7 | 381 |  |  |
|  | 5 | 382 |  |  |
|  | Total | 1184 |  |  |
| Average per Sequence |  | 1675.00 | 1994.00 | 84.00 |

# Proposed method and modified WD

To reduce the coding bits for DLT, a differential coding method is proposed. The modifications on 3D-HEVC WD are detailed as follow.

Table 2.  Modified VPS syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V DepthLayerFlag[ layerId ] should be derived from dimension_id to be aligned with basic idea of MV-HEVC Draft 3. ] |  |
| ViewId[ layerId ] = layerId >> 1 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V ViewId[ layerId ] should be derived as specified in MV-HEVC Draft 3. ] |  |
| view_id[ layerId ] [ Ed. (GT): This syntax element is different from ViewId[ layerId ]. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| dlt_depth_value[ layerId ][ 0 ] | ue(v) |
| if (num_depth_values_in_dlt[ layered ] > 1) { |  |
| dlt_bit_map_flag [ layerId ] | u(1) |
| if ( !dlt_bit_map_flag ) { |  |
| max_diff [ layerId ] | ue(v) |
| min_diff [ layerId ] | u(v) |
| if ( max_diff [ layerId ] != min_diff [ layerId ] ) |  |
| for( j = 1; j < num_detph_values_in_dlt [ layerId ]; j++ ) |  |
| dlt_depth_value_diff_minus_min[ layerId ] [ j ] | u(v) |
| } |  |
| else { |  |
| diff_max_min_dlt_value [ layerId ] | ue(v) |
| dlt_bit_map [ layerId ] [ 0 ] = 1 |  |
| for ( j = 1; j < diff_max_min_dlt_value [ layerId ]; j ++ ) |  |
| dlt_bit_map [ layerId ] [ j ] | u(1) |
| dlt_bit_map [ layerId ] [ diff_max_min_dlt_value [ layerId ] ] = 1 |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

Semantics:

dlt_depth_value[ layerId ][ 0 ] specifies the value of the 0-th entry in the depth lookup table for depth view components with layer_id equal to layerId. The value of dlt_depth_value[layerId][0] is the smallest depth value in the depth lookup table for depth view components with layer_id equal to layerId.

dlt_bit_map_flag [ layerId ] equal to 1 specifies that the bit map is used to code the entries in depth lookup table for depth view components with layer_id equal to layerId.

max_diff [ layerId ] specifies the maximum value in the values of dlt_depth_value[ layerId ][ j ] – dlt_depth_value[ layerId ][ j – 1] (for j being from 1 to num_depth_values_in_dlt[ layerId ] – 1) for depth view components with layer_id equal to layerId. The value of max_diff [ layerId ] shall not be negative.

min_diff [ layerId ] specifies the minimum value in the values of dlt_depth_value[ layerId ][ j ] – dlt_depth_value[ layerId ][ j – 1 ] (for j being from 1 to num_depth_values_in_dlt[ layerId ] – 1) for depth view components with layer_id equal to layerId. The value of min_diff [ layerId ] shall not be negative. The number of bits used to represent min_diff [ layerId ] is equal to Ceil( Log2(max_diff [ layerId ] + 1) ).

dlt_depth_value_diff_minus_min [ layerId ] [ j ] plus min_diff [layerId] specifies the value of dlt_depth_value[ layerId ][ j ] – dlt_depth_value[ layerId ][ j – 1 ] (for j being from 1 to num_depth_values_in_dlt[ layerId ] – 1) for depth view components with layer_id equal to layerId. The value of dlt_depth_value_diff_minus_min [ layerId ][ j ] shall be in the range of 0 to (max_diff [layerId] – min_diff [layerId]), inclusive. The number of bits used to represent dlt_depth_value_diff_minus_min [ layerId ] [ j ] is Ceil( Log2(max_diff [layerId] –min_diff [layerId] + 1) ). When not present, the value of dlt_depth_value_diff_minus_min[ layerId ] [ j ] is derived to be equal to 0.

The value of dlt_depth_value [ layerId ][ j ] is derived as follow:

dlt_depth_value [ layerId ][ j ] = dlt_depth_value_diff_minus_min [ layerId ] [ j ] + min_diff [ layerId ] + dlt_depth_value [ layerId ][ j – 1 ],

for j being from 1 to num_depth_values_in_dlt[ layerId ] – 1.

diff_max_min_dlt_value [ layerId ] specifies the difference value between the maximum and minimum values in the depth lookup table for the depth view components with layer_id equal to layerId.

dlt_bit_map [ layerId ] [ j ] is used in the derivation of the value of the j-th entry in the depth lookup table for the depth components with layer_id equal to layerId as follow.

k = 1;

for ( j = 1; j <= diff_max_min_dlt_value [ layerId ]; j++ ) {

if ( dlt_bit_map [ layerId ][ j ] ) {

dlt_depth_value [ layerId ][k] = dlt_depth_value [ layerId ][ 0 ] + j;

k ++;

}

}

# Experimental results

The proposed method was implemented on HTM8.0. The comparisons of DLT coding bits between the HTM8.0 anchor and the proposed method are listed in Table 3, where only the four sequences, i.e. Balloons, Kendo, Newspaper and PoznanHall2, are involved. As observed, using the proposed method, the coding bits for DLT are reduced by about 85% (D1/D0) (taking an average of 44% in the VPS coding bits (D1/V1)), while the bits for VPS are saved by 71% (V1/V0). Table 4 shows that the proposed method can achieve a slight BD-rate saving about 0.02% under the common test condition [2].

Table 3.  Comparisons between HTM8.0 anchors and proposed on DLT and SPS coding bits

| Sequence | View | HTM8.0 Anchor |  |  | HTM8.0 + Proposed |  |  | Comparisons |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DLT bits (D0) | VPS bits (V0) | D0/V0 (%) | DLT bits (D1) | VPS bits (V1) | D1/V1 (%) | D1/D0 (%) | V1/V0 (%) |
| Balloon | 3 | 556 | 2135 | 85.06 | 69 | 716 | 55.45 | 12.41 | 33.54 |
|  | 1 | 642 |  |  | 166 |  |  | 25.86 |  |
|  | 5 | 618 |  |  | 162 |  |  | 26.21 |  |
|  | Total | 1816 |  |  | 397 |  |  | 21.86 |  |
| Kendo | 3 | 486 | 2093 | 84.76 | 65 | 532 | 40.04 | 13.37 | 25.42 |
|  | 1 | 638 |  |  | 75 |  |  | 11.76 |  |
|  | 5 | 650 |  |  | 73 |  |  | 11.23 |  |
|  | Total | 1774 |  |  | 213 |  |  | 12.01 |  |
| Newspaper | 4 | 607 | 2245 | 85.79 | 70 | 536 | 40.49 | 11.53 | 23.88 |
|  | 2 | 622 |  |  | 71 |  |  | 11.41 |  |
|  | 6 | 697 |  |  | 76 |  |  | 10.90 |  |
|  | Total | 1926 |  |  | 217 |  |  | 11.27 |  |
| Poznan_hall2 | 6 | 421 | 1503 | 78.78 | 59 | 493 | 35.29 | 14.01 | 32.80 |
|  | 7 | 381 |  |  | 59 |  |  | 15.49 |  |
|  | 5 | 382 |  |  | 56 |  |  | 14.66 |  |
|  | Total | 1184 |  |  | 174 |  |  | 14.70 |  |
| Average per Sequence |  | 1675.00 | 1994.00 | 84.00 | 250.25 | 569.25 | 43.96 | 14.94 | 28.55 |

Table 4.  Performance comparisons between HTM8.0 CTC anchor and proposed

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.03% | -0.03% | 82.1% | 89.5% | 97.5% |
| Kendo | -0.1% | 0.0% | 0.0% | 0.0% | -0.03% | -0.03% | 82.2% | 95.1% | 96.0% |
| Newspaper_CC | -0.1% | 0.0% | 0.0% | 0.0% | -0.03% | -0.03% | 76.3% | 98.0% | 90.3% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 97.9% | 96.5% | 89.3% |
| Poznan_Hall2 | -0.1% | 0.0% | 0.0% | 0.0% | -0.04% | -0.04% | 117.9% | 97.3% | 93.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 86.1% | 96.9% | 90.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 103.8% | 94.4% | 91.6% |
| 1024x768 | -0.1% | 0.0% | 0.0% | 0.0% | -0.03% | -0.03% | 80.2% | 94.2% | 94.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 101.4% | 96.3% | 91.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | -0.02% | 92.3% | 95.4% | 92.6% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 103.0% | 97.4% | 91.8% |

# Conclusions

In this document, a single-view differential coding method for DLT is proposed, which reduces the DLT coding bits by an average of 85%, and VPS coding bits by 71%. Additionally, the CTC test results show that the proposed method provides almost the same coding performance as that of HTM8.0 anchor. Therefore, it is recommended to adopt this method into 3D-HEVC.

# References

- Fabian Jager. 3D-CE6.h: Simplified depth coding with an optional depth lookup table, JCT-3V document JCT3V-B0036, October, 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, Aug, 2013.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).