| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0140 |
| --- | --- |

| Title: | 3D-CE1 related: Problem fix on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current BVSP process, either 8x4 or 4x8 prediction block is used for the second disparity compensation prediction. However, there can potentially be 4x4 prediction block at the second disparity compensation prediction when a 16x16 coding unit is coded with the asymmetric motion partition mode. In order to solve this problem, it is proposed to set the size of the second disparity compensation to the size of current prediction unit when either width or height of the current prediction unit is not multiple of 8. The proposed method reportedly provides no coding loss.

# Introduction

In the current backward view synthesis prediction (BVSP) process [1], either 8x4 or 4x8 block partition size is used for the second disparity compensation prediction (DCP) and the decision for these block partition sizes is performed for each 8x8 block in each prediction unit (PU).

Figure 1. Example of the decision for block partition 
(CU size: 16x16, PU partitioning mode: PART_nLx2N)

However, the current decision scheme has a potential problem when the current coding unit size is 16x16 block and is coded with the asymmetric motion partition (AMP). For instance, if the partitioning mode is PART_nLx2N, the second PU can have 4x4 prediction block for second DCP as shown in Figure 1. For the first PU, the size of second DCP should be set to the size of the current PU because the width of the current PU is less than 8 according to the specification of 3D-HEVC. For the second PU, the width is extended from 12 to 16 at the mode decision time to perform 8x8-block basis decision and thus the decision for the left 8x8 blocks is performed over the boundary of PU. Here, if the decided partition size of the left 8x8 block is 8x4, then two 4x4 DCP processes should be performed. In order to have design consistency with HEVC version 1, it is desirable to avoid 4x4 prediction block. Therefore, it is needed to solve this problem in a proper way.

# Proposed Method

In this contribution, while keeping the current 8x8-block basis decision scheme, it is proposed to simply set the size of the second disparity compensation prediction to the size of the current prediction unit when either width or height of the current prediction unit is not multiple of 8. And for all the other cases, the 8x8 block basis decision is performed as the current BVSP process. The proposed method can be illustrated as shown in Figure 2. With the proposed method, the 4x4 disparity compensation prediction can be easily avoided.

Figure 2. Comparison between the current method and the proposed method

# Experimental Results

The simulation was carried out on the common test conditions [2] and was based on HTM 8.0 reference software. The performance of the proposed method is shown in Table 1. The proposed method provides no coding loss.

Table 1. Comparison between the CTC anchor and the proposed method (3-view test case)

# Conclusion

This contribution provides the problem fix for BVSP which can avoid a 4x4 disparity compensation prediction without any coding loss. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Draft Text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” JCT3V-E1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-E1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.