| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0140_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Draft Text for JCT3V-F0140 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides a proposed text of 3D-HEVC Draft Text. The changes are highlighted in green.

H.8.5.3.3.8	View synthesis prediction process

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- a location ( xBl, yBl ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- the prediction list indicator X

- the reference index refIdxLX

Outputs of this process are:

–	an array predSamplesL of luma prediction samples,

–	an array predSamplesCb of chroma prediction samples for the component Cb

–	an array predSamplesCr of chroma prediction samples for the component Cr

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the current picture is derived by:

  - xP = xCb + xBl		(Error: Reference source not found-1)

  - yP = yCb + yBl		(Error: Reference source not found-2)

The reference picture consisting of an ordered two-dimensional array refPicL of luma samples and two ordered two-dimensional arrays refPicCb and refPicCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

The variable refViewIdx is set equal to the ViewIdx( RefPicListX[ refIdxLX ] ) and the variable depthViewIdx is set equal to RefViewIdx[ xCb + xBl ][ yCb + yBl ]. The variable mvDisp is set equal to MvDisp[ xCb + xBl ][ yCb + yBl ]

The derivation process for a disparity sample array as specified in section Error: Reference source not found is invoked with the luma location ( xP, yP ), the disparity vector mvDisp, the variable refViewIdx, the variable depthViewIdx, the variable nPSW equal to nPbW, the variable nPSH equal to nPbH, and the variable splitFlag equal to ( !( nPbW  & 7)  && !( nPbH  & 7 ) ) as the inputs, and the output is the array disparitySamples of size (nPSW)x(nPSH).

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units.

For each luma sample location ( xL = 0..nPbW − 1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSamplesL, the corresponding prediction luma sample value predSamplesL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

  - xIntL = xP + xL+ disparitySamples[ xL ][ yL ]		(Error: Reference source not found-3)

  - yIntL = yP + yL		(Error: Reference source not found-4)

  - xFracL = disparitySamples[ xL ][ yL ] & 3		(Error: Reference source not found-5)

  - yFracL = 0		(Error: Reference source not found-6)

- The prediction luma sample value predSamplesL[ xL][ yL ] is derived by invoking the process specified in subclause 8.5.3.3.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicL given as input.

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-eighth sample units.

For each chroma sample location ( xC = 0..nPbW / 2−1, yC = 0..nPbH /2 −1 ) inside the prediction chroma sample arrays predSamplesCb and predSamplesCr, the corresponding prediction chroma sample values predSampleLXCb[ xC ][ yC ] and predSamplesCr[ xC ][ yC ] are derived as follows:

- The variables xIntC, yIntC, xFracC, and yFracC are derived by

  - xIntC = ( xP / 2 ) + xC + disparitySamples[ xC  <<  1 ][ yC  <<  1 ]	(Error: Reference source not found-7)

  - yIntC = ( yP / 2 ) + yC		(Error: Reference source not found-8)

  - xFracC = disparitySamples[ xC  <<  1][ yC   <<  1 ] & 7		(Error: Reference source not found-9)

  - yFracC = 0		(Error: Reference source not found-10)

- The prediction sample value predSamplesCb[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.3.3.3.3 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCb given as input.

The prediction sample value predSamplesCr[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.3.3.3.3 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCr given as input.