| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0142_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Draft Text for JCT3V-F0142 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides a proposed text of 3D-HEVC Draft Text. The changes are highlighted in green.

H.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for merging candidates from neighbouring prediction unit partitions in subclause 8.5.3.2.2 is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the partition index partIdx as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, and mvLXB2, with X being 0 or 1.

  - The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col.The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

      - availableFlagCol = availableFlagL0Col		(Error: Reference source not found-1)

      - predFlagL0Col = availableFlagL0Col		(Error: Reference source not found-2)

      - predFlagL1Col = 0		(Error: Reference source not found-3)

  - When slice_type is equal to B, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

      - availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(Error: Reference source not found-4)

      - predFlagL1Col = availableFlagL1Col		(Error: Reference source not found-5)

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, as the inputs and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively)..

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], the following applies.

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ) as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on DepthFlag, the following applies.

    - If DepthFlag is equal to 0, the variable availableFlagT is set equal to 0.

    - Otherwise ( DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as the inputs and the outputs are the flag availableFlagT, the prediction utilization flags predFlagL0T and predFlagL1T, the reference indices refIdxL0T and refIdxL1T, and the motion vectors mvL0T and mvL1T.

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - The variable vspCandInListFlag is set to equal to 0.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies:

      - When the following condition is true,

        - availableFlagT = = 0  &&  availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1  &&  predFlagLXN  !=  predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  mvLXN  !=  mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  refIdxLXN  !=  refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, if vspCandInListFlag is equal to 0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH − 1 ], otherwise, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the following applies:

      - When the following condition is true ,

        - availableFlagT  = =  0  &&  availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

        - availableFlagN  = =  1  &&  predFlagLXN  !=  predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  mvLXN  !=  mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  refIdxLXN  !=  refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, if vspCandInListFlag is equal to 0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW − 1 ][ yPb − 1 ], otherwise, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0,   and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0, if vspCandInListFlag is equal to 0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW ][ yPb − 1 ] , otherwise, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

      - availableFlagA1  = =  0,

      - predFlagLXA1  !=  predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXA1  !=  mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXA1  !=  refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

      - availableFlagB1  = =  0,

      - predFlagLXB1  !=  predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXB1  !=  mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXB1  !=  refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0, if vspCandInListFlag is equal to 0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH ] , otherwise, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to B2, if vspCandInListFlag is equal to 0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb − 1 ] , otherwise, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, and the variable numMergeCand is increased by 1.

  - When availableFlagIvMCShift is equal to 1 and numMergeCand is less than 6, and one or more of the following conditions are true,

      - availableFlagIvMC  = =  0,

      - predFlagLXMC  !=  predFlagLXMCShift  (with X being replaced by 0 and 1),

      - mvLXMC  !=  mvLXIvMCShift (with X being replaced by 0 and 1),

      - refIdxLXMC  !=  refIdxLXMCShift (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvMCShift, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - A variable availableFlagIvDCShift is set to 0 and when all of the following conditions are true

      - DepthFlag is equal to 0, [Ed (CY): this condition is already assumed thus could be removed.]

      - availableFlagIvMCShift is equal to 0,

      - numMergeCand is less than 6,

the derivation process for the shifted disparity merging candidate as specified in subclause Error: Reference source not found is invoked with the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in mergeCandList, mergeCandList, mergeCandIsVspFlag, and numMergeCand as the inputs and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift. When availableFlagIvDCShift is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvDCShift, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagVSP is equal to 1 and vspCandInListFlag is equal to 0, the entry mergeCandList[ numMergeCand ] is set equal to VSP, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal 1 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - The variable numOrigMergeCand is set equal to numMergeCand.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, mergeCandIsVspFlag, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand, and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk, and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + k ] is set equal to 0.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.3.2.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + numCombMergeCand + m ] is set equal to 0.

  - The following assignments are made with N being the candidate at position merge_idx[ xOrigP ][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(Error: Reference source not found-6)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(Error: Reference source not found-7)

      - refIdxLX = refIdxLXN		(Error: Reference source not found-8)

      - predFlagLX = predFlagLXN		(Error: Reference source not found-9)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies

      - refIdxL1 = −1		(Error: Reference source not found-10)

      - predFlagL1 = 0		(Error: Reference source not found-11)

  - The variable vspModeFlag is set equal to mergeCandIsVspFlag[ merge_idx[ xPb][ yPb ] ]

  - When vspModeFlag is equal to 1, the following applies:

    - The variables xVsp and yVsp are derived depending on the value of mergeCandList[ merge_idx[ xPb][ yPb ] ] as specified in Table Error: Reference source not found-1.

    - The variable vspRefViewIdx is set equal RefViewIdx[ xVsp ][ yVsp ] and the variable vspMvDisp is set equal to MvVSPDisp[ xVsp ][ yVsp ].

    - For x in the range of xPb to ( xPb + nPbW − 1 ), inclusive, the following applies:

      - For y in the range of yPb to ( yPb + nPbH − 1 ), inclusive, the following applies:

        - RefViewIdx[ x ][ y ] = vspRefViewIdx		(Error: Reference source not found-12)

        - MvDisp[ x ][ y ] = vspMvDisp		(Error: Reference source not found-13)

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If one of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - predFlagLXIvMC  = =  1  &&  mergeCandList[ merge_idx[ xPb][ yPb ] ]  =  =  IvMC

      - predFlagLXIvMCShift  = =  1  &&  and mergeCandList[ merge_idx[ xPb][ yPb ] ]  = =  IvMCShift

[Ed. (GT): PredMode[ xCb ][ yCb ]  = =  MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

Table Error: Reference source not found-1 – Specification of the xVsp and yVsp depending on N = mergeCandList[ merge_idx[ xPb][ yPb ] ]

| N | A1 | B1 | B0 | VSP | A0 | B2 |
| --- | --- | --- | --- | --- | --- | --- |
| xVsp | xPb − 1 | xPb + nPbW − 1 | xPb + nPbW | xPb | xPb − 1 | xPb − 1 |
| yVsp | yPb + nPbH − 1 | yPb − 1 | yPb − 1 | yPb | yPb + nPbH | yPb − 1 |

H.8.5.3.2.13	Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

- The variable availableFlagVSP is set equal to 1, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, the variables refIdxL0VSP and refIdxL1VSP are set equal to −1 and the variable refViewAvailableFlag is set equal to 0.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX – 1, inclusive, the following applies:

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xCb ][ yCb ], the following applies:

        - refViewAvailableFlag = 1		(Error: Reference source not found-14)

        - predFlagLXVSP = 1		(Error: Reference source not found-15)

        - mvLXVSP = MvVSPDisp[ xCb ][ yCb ]		(Error: Reference source not found-16)

        - refIdxLXVSP = i		(Error: Reference source not found-17)

        - Y = 1 – X		(Error: Reference source not found-18)

When the current slice is a B slice and refViewAvailableFlag is equal to 1, refViewAvailableFlag is set equal to 0 and the following applies:

  - For i in the range of 0 to NumRefPicsLY – 1, inclusive, the following applies.

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListY[ i ] ) is not equal to RefViewIdx[ xCb ][ yCb ] and ViewIdx( RefPicListY[ i ] ) is not equal to ViewIdx, the following applies:

      - The variables refViewAvailableFlag, predFlagLYVSP, mvLYVSP, and refIdxLYVSP are derived as specified in the following:

        - refViewAvailableFlag = 1		(Error: Reference source not found-19)

        - predFlagLYVSP = 1		(Error: Reference source not found-20)

        - mvLYVSP =  MvVSPDisp[ xCb ][ yCb ]		(Error: Reference source not found-21)

        - refIdxLYVSP = i		(Error: Reference source not found-22)

      - When iv_mv_scaling_flag is equal to 1, mvLYVSP is modified as specified in the following:

        - td = Clip3( −128, 127, ViewId – view_id_val[ RefViewIdx[ xCb ][ yCb ] ) ]	(Error: Reference source not found-23)

        - tb = Clip3( −128, 127, ViewId – ViewId( RefPicListY[ i ] ) )	(Error: Reference source not found-24)

        - tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(Error: Reference source not found-25)

        - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-26)

- mvLYVSP[ 0 ] = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLYVSP[ 0 ] ) 
	* ( (Abs( distScaleFactor * mvLYVSP[ 0 ] ) + 127 )  >>  8 ) )	(Error: Reference source not found-27)

        - mvLYVSP[ 1 ] = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLYVSP[ 1 ] ) 
	* ( (Abs( distScaleFactor * mvLYVSP[ 1 ] ) + 127 )  >>  8 ) )	(Error: Reference source not found-28)

H.8.5.3.3.8	View synthesis prediction process

…

The variable refViewIdx is set equal to the ViewIdx( RefPicListX[ refIdxLX ] ) and the variable depthViewIdx is set equal to RefViewIdx[ xCb  ][ yCb  ]. The variable mvDisp is set equal to MvVSPDisp[ xCb  ][ yCb  ]

…

H.8.5.5	Derivation process for disparity vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - refViewIdx is set equal to 0, and mvRefinedDisp is set equal to ( DepthToDisparityB[ refViewIdx ][ BitDepthY – 1], 0 ).

  - The derivation process for disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvRefinedDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, and the variable splitFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySample[ 0 ][ 0 ].

  - mvVSPDisp is set to equal to mvRefinedDisp.

  - The derivation process for disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvRefinedDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, and the variable splitFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySample[ 0 ][ 0 ].

When depth_refinement_flag[ nuh_layer_id ] is equal to 0, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 and iv_res_pred_weight_idx is greater than zero, the following applies:

- The variable checkParallelMergeFlag is derived as follows:

  - If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

    - PredMode[ xCb ][ yCb ] is equal to MODE_SKIP.

    - PredMode[ xCb ][ yCb ] is equal to MODE_INTER and merge_flag[ xCb ][ yCb ] is equal to 1.

  - Otherwise, checkParallelMergeFlag is set equal to 0.

- The derivation process for a disparity vector from temporal neighbour block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), and the variable nCbS as inputs, and the outputs are the flag availableDV, the disparity vector mvDisp and the reference view order index refViewIdxTemp.

- When availableDV is equal to 0, for each N being A1, B1 and ( xN, yN ) being ( xCb − 1,  yCb + nCbS − 1 ), ( xCb + nCbS − 1,  yCb − 1 ), respectively, the following ordered steps apply.

  - When yCb − 1 is less than ( ( yCb  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ), the following applies.

        - xB1 = ( ( xB1  >>  3 )  <<  3 ) + ( ( xB1  >>  3 ) & 1) * 7	(Error: Reference source not found-29)

  - The derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to the ( xCb, yCb ) and the luma location ( xN, yN ) as the input and the output assigned to availableN.

  - When availableN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_INTRA, availableN is set equal to 0. [Ed. (GT): 2+3 correspond to 6.4.2 for CU when ( xN, yN ) outside CU. Cross-check appreciated. ]

  - When all of the following conditions are true, availableN is set equal to 0.

    - checkParallelMergeFlag is equal to 1

    - ( xCb  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to
( xN  >>  ( log2_parallel_merge_level_minus2 + 2) )

    - ( yCb  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to ( yN  >>  ( log2_parallel_merge_level_minus2 + 2) ).

  - The flag availableIvpMvSearchFlagN is set equal to availableN.

  - When one of the following conditions is true, N is equal to B1 and ( ( yN  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ) is less than ( ( yCb >> Log2CtbSizeY )  <<  Log2CtbSizeY), availableIvpMvSearchFlagN is set equal to 0.

  - The flag availableFlagIvpMvN is set equal to 0.

  - For each X from 0 to 1, the following applies:

    - When availableDV is equal to 0, availableN is equal to 1, RefIdxLX[ xN ][ yN ] is greater than or equal to 0, and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture of the current picture, the following applies:

          - refViewIdxTemp = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(Error: Reference source not found-30)

          - mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-31)

          - availableDV = 1		(Error: Reference source not found-32)

      - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

        - When availableIvpMvSearchFlagN is equal to 1, availableFlagIvpMvN is equal to 0, and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

            - ivpMvDispN = MvRefinedDisp[ xN ][ yN ]	(Error: Reference source not found-33)

            - refViewIdxN = RefViewIdx[ xN ][ yN ]	(Error: Reference source not found-34)

            - availableFlagIvpMvN = 1		(Error: Reference source not found-35)

- When availableDV is equal to 0 for each N being A1 and B1, the following applies.

  - When availableDV is equal to 0 and availableFlagIvpMvN is equal to 1, the following applies:

      - mvDisp = ivpMvDispN		(Error: Reference source not found-36)

      - refViewIdxTemp = refViewIdxN		(Error: Reference source not found-37)

      - availableDV = 1		(Error: Reference source not found-38)

- When availableDV is equal to 0, refViewIdxTemp is set equal to 0, and mvDisp is set equal to ( 0, 0 ).

- When depth_refinement_flag[ nuh_layer_id ] is equal to 0, the variable mvRefinedDisp is set equal to mvDisp and refViewIdx is set to equal to refViewIdxTemp.

[Ed. (GT): It is currently an issue under non-CTC that a view with refViewIdx equal to 0 might not be available in the reference picture list. In software a preliminary fix has been incorporated, to enable simulations in IBP configuration. ]

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

  - MvDisp[ x ][ y ] = mvDisp		(Error: Reference source not found-39)

  - MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(Error: Reference source not found-40)

  - MvVSPDisp[ x ][ y ] = mvVSPDisp 		(Error: Reference source not found-41)

  - RefViewIdx[ x ][ y ] = refViewIdx		(Error: Reference source not found-42)

  - DefaultDispFlag[ x ][ y ] = !availableDV		(Error: Reference source not found-43)