| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0142 |
| --- | --- |

| Title: | 3D-CE2: Results on 2-Step Disparity Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a depth-based 2-step disparity vector derivation (2-step DV derivation) which was proposed in JCT3V-E0142. The 2-step DV derivation consists of two steps. The first disparity vector is derived from the corresponding depth block pointed by the initial disparity vector corresponding to the depth value 128 (1 << (bit depth – 1)) and then the final disparity vector is derived from the corresponding depth block pointed by the first disparity vector. Additionally, if the current prediction unit is coded with the residual prediction, then the NBDV derivation process is additionally performed. The proposed method reportedly provides an average of 0.2% bit-saving for coded and synthesized views. And also it is additionally proposed to use the first disparity vector for the view synthesis prediction and to allow only one view synthesis prediction candidate in the merge candidate list in order to reduce the number of depth block fetch. With the proposed simplified method, the number of depth block to be fetched can be kept as the current 3D-HEVC in the worst case. The proposed simplified method reportedly also provides an average of 0.2% bit-saving for coded and synthesized views.

# Introduction

The depth-based 2-step disparity vector derivation (2-step DV derivation) which was proposed in JCT3V-E0142 [2] enables to have better coding performance than the current disparity vector derivation. For the memory complexity, the number of depth block fetch is three in the worst case for the 2-step DV derivation while the current DV derivation is two. Therefore, the simplified method also is presented in this contribution.

# Proposed Method

## 2-Step DV Derivation

The 2-step DV derivation consists of two steps as shown in Figure 1. The first disparity vector is derived from the corresponding depth block pointed by the initial disparity vector corresponding to the depth value 128 (1 << (bit depth – 1)) and then the final disparity vector is derived from the corresponding depth block pointed by the first disparity vector. Additionally, if the current prediction unit is coded with the residual prediction, then NBDV derivation process is additionally performed.

Figure 1. Proposed 2-Step DV Derivation

## Simplified 2-Step DV Derivation

The 2-step derivation needs to fetch three depth blocks in the worst case. Two depth blocks are basically needed to fetch for the 2-step DV derivation, and when the current coding unit is coded with VSP candidate, then one more depth block is needed to fetch. On the other hand, the disparity vector derivation in the current 3D-HEVC needs to fetch two depth blocks in the worst case. One depth block is needed to fetch for the DoNBDV derivation, and when the current coding unit is coded with inherited VSP, then one more depth block is needed to fetch.

Therefore, in order to reduce the number of depth block fetches, it is proposed to use the first disparity vector (1st DV Figure 1) in from 2-step DV derivation for the view synthesis prediction and to allow only one view synthesis prediction candidate in the merge candidate list.

# Experimental Results

The simulation was carried out on the common test conditions [3] and was based on HTM 8.0 reference software. The performance of the proposed method is shown in Table 1. The proposed method provides an average 0.3% and 0.7% bit-savings for video 1 and video 2, and 0.2% bit-savings for coded and synthesized views.

Table 1. Comparison between the CTC anchor and the proposed method (3-view test case)

The simplified method provides an average 0.1% and 0.5% bit-savings for video 1 and video 2, and 0.2% bit-savings for coded and synthesized views.

Table 2. Comparison between the CTC anchor and the simplified method (3-view test case)

# Conclusion

In this contribution, the 2-step DV derivation is proposed. Additionally, in order to reduce the number of depth block fetches, simplified 2-step DV derivation is also proposed. The proposed method provides an average of 0.2% bit-saving for coded and synthesized views without additional complexity. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Draft Text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” JCT3V-E1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[2] M. W. Park, J. Y. Lee, B. Choi, Y. Cho, C. Kim, “3D-CE2.h related: Simplified NBDV and improved disparity vector derivation,” JCT3V-E0142, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-E1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.