| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0144 |
| --- | --- |

| Title: | 3D-CE3 related: Additional Depth-based DV Candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, there are two shifted DV candidates to be able to add to the merge candidate list: one is the shifted first available DV and the other is the shifted DoNBDV. These candidates are generated by adding 4 to the horizontal component of the existing disparity vectors. However, these candidates reportedly provide no coding gain. Therefore, in order to improve the coding performance of the DV candidate, it is proposed to replace the current shifted DV candidates with a proposed depth-based DV candidate. While the current depth-based DV is derived by choosing the maximum depth value from the corresponding depth block, the proposed depth-based DV is derived by choosing the minimum depth value from the corresponding depth block. The proposed method reportedly provides 0.1% bit-saving for synthesized view. Compared with disabled the shifted DV candidates, the proposed method reportedly provides 0.1% bit-saving for both coded and synthesized views.

# Introduction

Table 1. Comparison between the CTC-anchor with disabled the shifted DVs and the CTC-anchor

In the current 3D-HEVC [1], there are two shifted DV candidates to be able to add to the merge candidate list: one is the shifted first available DV and the other is the shifted DoNBDV. These candidates are generated by adding 4 to the horizontal component of the existing disparity vectors. However, as can be seen in Table 1, these candidates provide no coding gain. Therefore, this contribution introduces a method to improve the coding performance of the DV candidate.

# Proposed Method

In order to improve the coding performance of the DV candidate, it is proposed to replace the current shifted DV candidates with a proposed depth-based DV candidate. While the current depth-based DV (DoNBDV) is derived by choosing the maximum depth value among 4 corner pixels in the corresponding depth block, the proposed depth-based DV (DoNBDVmin) is derived by choosing the minimum depth value among 4 corner pixels from the corresponding depth block.

Figure 1. Depth-based DV derivation

Since the DoNBDV is derived in coding unit level, sometimes DoNBDV can be different from the real disparity of some prediction unit if the coding unit has more than 2 prediction units. For instance, when the prediction mode is PART_Nx2N as shown in Figure 2, and the first prediction unit (PU 0) has a background and the second prediction unit (PU 1) has an object, DoNBDV is most likely to become a disparity of the second prediction unit and cannot reflect the disparity of the first prediction unit. However, if the proposed disparity vector (DoNBDVmin) is used, DoNBDVmin might be able to reflect the disparity of the first prediction unit.

Figure 2. Example of the coding unit having 2 prediction units

# Experimental Results

The simulation was carried out on the common test conditions [2] and was based on HTM 8.0 reference software. The performance of the proposed method is shown in Table 2. The proposed method provides 0.1% bit-saving for synthesized view.

Table 2. Comparison between the CTC-anchor and the proposed method

Compared with disabled the shifted DV candidates, the proposed method provides 0.1% bit-saving for both coded and synthesized views.

Table 3. Comparison between the CTC-anchor with disabled the shifted DVs and the proposed method

# Conclusion

In this contribution, it is proposed to replace the current shifted DV candidates with a proposed depth-based DV candidate. The proposed method provides 0.1% bit-saving for synthesized view. Therefore, it is recommended to adopt proposed method into next 3D-HEVC Draft Text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” JCT3V-E1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-E1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.