| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0144_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Draft Text for JCT3V-F0144 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides proposed text of 3D-HEVC Draft Text. The changes are highlighted in green.

H.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for merging candidates from neighbouring prediction unit partitions in subclause 8.5.3.2.2 is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the partition index partIdx as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, and mvLXB2, with X being 0 or 1.

  - The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col.The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

      - availableFlagCol = availableFlagL0Col		(Error: Reference source not found-1)

      - predFlagL0Col = availableFlagL0Col		(Error: Reference source not found-2)

      - predFlagL1Col = 0		(Error: Reference source not found-3)

  - When slice_type is equal to B, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

      - availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(Error: Reference source not found-4)

      - predFlagL1Col = availableFlagL1Col		(Error: Reference source not found-5)

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, as the inputs and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift, availableFlagIvDC and availableIvDCmin the reference indices refIdxLXIvMC, refIdxLXIvMCShift, refIdxLXIvDC and refIdxLXIvDC the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift, predFlagLXIvDC and predFlagLXIvDCmin, and the motion vectors mvLXIvMC, mvLXIvMCShift, mvLXIvDC and mvLXIvDCmin (with X being 0 or 1, respectively)..

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], the following applies.

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ) as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on DepthFlag, the following applies.

    - If DepthFlag is equal to 0, the variable availableFlagT is set equal to 0.

    - Otherwise ( DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as the inputs and the outputs are the flag availableFlagT, the prediction utilization flags predFlagL0T and predFlagL1T, the reference indices refIdxL0T and refIdxL1T, and the motion vectors mvL0T and mvL1T.

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies:

      - When the following condition is true,

        - availableFlagT = = 0  &&  availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1  &&  predFlagLXN  !=  predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  mvLXN  !=  mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  refIdxLXN  !=  refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the following applies:

      - When the following condition is true ,

        - availableFlagT  = =  0  &&  availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

        - availableFlagN  = =  1  &&  predFlagLXN  !=  predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  mvLXN  !=  mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  refIdxLXN  !=  refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW − 1 ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

      - availableFlagA1  = =  0,

      - predFlagLXA1  !=  predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXA1  !=  mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXA1  !=  refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

      - availableFlagB1  = =  0,

      - predFlagLXB1  !=  predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXB1  !=  mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXB1  !=  refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagVSP is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to VSP, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal 1 and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH ] and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagIvMCShift is equal to 1 and numMergeCand is less than 6, and one or more of the following conditions are true,

      - availableFlagIvMC  = =  0,

      - predFlagLXMC  !=  predFlagLXMCShift  (with X being replaced by 0 and 1),

      - mvLXMC  !=  mvLXIvMCShift (with X being replaced by 0 and 1),

      - refIdxLXMC  !=  refIdxLXMCShift (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvMCShift, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When all of the following conditions are true

      - DepthFlag is equal to 0, [Ed (CY): this condition is already assumed thus could be removed.]

      - availableFlagIvDCmin is equal to 1,

      - availableFlagIvMCShift is equal to 0,

      - numMergeCand is less than 6,

and one or more of the following conditions is true,

      - predFlagLXIvDC != predFlagLXIvDCmin (with X being replaced by 0 and 1),

      - mvLXIvDC != mvLXIvDCmin (with X being replaced by 0 and 1),

      - refIdxLXIvDC != refIdxLXIvDCmin (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDCmin, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - The variable numOrigMergeCand is set equal to numMergeCand.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, mergeCandIsVspFlag, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand, and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk, and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + k ] is set equal to 0.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.3.2.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + numCombMergeCand + m ] is set equal to 0.

  - The following assignments are made with N being the candidate at position merge_idx[ xOrigP ][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(Error: Reference source not found-6)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(Error: Reference source not found-7)

      - refIdxLX = refIdxLXN		(Error: Reference source not found-8)

      - predFlagLX = predFlagLXN		(Error: Reference source not found-9)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies

      - refIdxL1 = −1		(Error: Reference source not found-10)

      - predFlagL1 = 0		(Error: Reference source not found-11)

  - The variable vspModeFlag is set equal to mergeCandIsVspFlag[ merge_idx[ xPb][ yPb ] ]

  - When vspModeFlag is equal to 1, the following applies:

    - The variables xVsp and yVsp are derived depending on the value of mergeCandList[ merge_idx[ xPb][ yPb ] ] as specified in Table Error: Reference source not found-1.

    - The variable vspRefViewIdx is set equal RefViewIdx[ xVsp ][ yVsp ] and the variable vspMvDisp is set equal to MvDisp[ xVsp ][ yVsp ].

    - For x in the range of xPb to ( xPb + nPbW − 1 ), inclusive, the following applies:

      - For y in the range of yPb to ( yPb + nPbH − 1 ), inclusive, the following applies:

        - RefViewIdx[ x ][ y ] = vspRefViewIdx		(Error: Reference source not found-12)

        - MvDisp[ x ][ y ] = vspMvDisp		(Error: Reference source not found-13)

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If one of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - predFlagLXIvMC  = =  1  &&  mergeCandList[ merge_idx[ xPb][ yPb ] ]  =  =  IvMC

      - predFlagLXIvMCShift  = =  1  &&  and mergeCandList[ merge_idx[ xPb][ yPb ] ]  = =  IvMCShift

[Ed. (GT): PredMode[ xCb ][ yCb ]  = =  MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

Table Error: Reference source not found-1 – Specification of the xVsp and yVsp depending on N = mergeCandList[ merge_idx[ xPb][ yPb ] ]

| N | A1 | B1 | B0 | VSP | A0 | B2 |
| --- | --- | --- | --- | --- | --- | --- |
| xVsp | xPb − 1 | xPb + nPbW − 1 | xPb + nPbW | xPb | xPb − 1 | xPb − 1 |
| yVsp | yPb + nPbH − 1 | yPb − 1 | yPb − 1 | yPb | yPb + nPbH | yPb − 1 |

H.8.5.3.2.10	Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC, predFlagLXIvMCShift and predFlagLXIvDC,

- the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC,

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies;

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ] and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(Error: Reference source not found-14)

      - predFlagL0IvMC = availableFlagL0IvMC		(Error: Reference source not found-15)

      - predFlagL1IvMC = availableFlagL1IvMC		(Error: Reference source not found-16)

The shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X , the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ), and the reference index refIdxLX being equal to −1, and as the inputs and the outputs are the flag availableFlagLXIvMCShift, the motion vector mvLXIvMCShift and the reference index refIdxLXIvMCShift.

  - The availability flag availableFlagIvMCShift, and the prediction utilization flags predFlagL0IvMCShift and predFlagL1IvMCShift are derived by

      - availableFlagIvMCShift = availableFlagL0IvMCShift | | availableFlagL1IvMCShift	(Error: Reference source not found-17)

      - predFlagL0IvMCShift = availableFlagL0IvMCShift	(Error: Reference source not found-18)

      - predFlagL1IvMCShift = availableFlagL1IvMCShift	(Error: Reference source not found-19)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a disparity inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ], and the prediction list indication X, as the inputs and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(Error: Reference source not found-20)

      - predFlagL0IvDC = availableFlagL0IvDC		(Error: Reference source not found-21)

      - predFlagL1IvDC = availableFlagL1IvDC		(Error: Reference source not found-22)

The minimum disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a disparity inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvMinRefinedDisp[ xPb ][ yPb ], and the prediction list indication X, as the inputs and the outputs are the flag availableFlagLXIvDCmin, the motion vector mvLXIvDCmin, and the reference index refIdxLXIvDCmin.

  - The availability flag availableFlagIvDCmin, and the prediction utilization flags predFlagL0IvDCmin and predFlagL1IvDCmin are derived by

      - availableFlagIvDCmin = availableFlagL0IvDCmin | | availableFlagL1IvDCmin 	(Error: Reference source not found-23)

      - predFlagL0IvDCmin = availableFlagL0IvDCmin	(Error: Reference source not found-24)

      - predFlagL1IvDCmin = availableFlagL1IvDCmin	(Error: Reference source not found-25)

- -

H.8.5.3.3.8	View synthesis prediction process

…

The derivation process for a disparity sample array as specified in section Error: Reference source not found is invoked with the luma location ( xP, yP ), the disparity vector mvDisp, the variable refViewIdx, the variable depthViewIdx, the variable nPSW equal to ( nPbW  = =  12 ? 16 : nPbW ), the variable nPSH equal to ( nPbH  = =  12 ? 16 : nPbH ), the variable splitFlag equal to ( nPbW > 4  &&  nPbH > 4 ), and the variable minFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nPSW)x(nPSH).

…

H.8.5.5	Derivation process for disparity vectors

…

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, the variable splitFlag equal to 0, and the variable minFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

  - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS,  the variable splitFlag equal to 0, and the varialble minFlag equal to 1 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvMinRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

  - MvDisp[ x ][ y ] = mvDisp		(Error: Reference source not found-26)

  - MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(Error: Reference source not found-27)

  - MvMinRefinedDisp[ x ][ y ] = mvMinRefinedDisp		(Error: Reference source not found-28)

  - RefViewIdx[ x ][ y ] = refViewIdx		(Error: Reference source not found-29)

  - DefaultDispFlag[ x ][ y ] = !availableDV		(Error: Reference source not found-30)

H.8.5.5.2	Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag

- a variable minFlag.

…

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) – 1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW) – 1 ), inclusive, the following applies:

    - The variables xB, yB, xP0, yP0, xP1, yP1, are derived as specified in the following:

xB = sBx * nSubBlkW
			yB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB )
			yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB )
			xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB + nSubBlkW – 1 )
			yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB + nSubBlkH – 1 )

    - The variable nSubSubBlkW is set equal to nSubBlkW and the variable nSubSubBlkH is set equal to nSubBlkH.

    - When splitFlag is equal to 1, nSubSubBlkW and nSubSubBlkW are modified as follows:

      - The variable horSplitFlag is derived as specified in the following:

        - horSplitFlag = ( refDepPels[ xP0 ][ yP0 ] > refDepPels[ xP1 ][ yP1 ] )
	  = =  ( refDepPels[ xP1 ][ yP0 ] > refDepPels[ xP0 ][ yP1] ) )	(Error: Reference source not found-31)

        - nSubSubBlkW = horSplitFlag ? nSubSubBlkW : ( nSubSubBlkW  >>  1 )		(Error: Reference source not found-32)

        - nSubSubBlkH = horSplitFlag ? ( nSubSubBlkH  >>  1 ) : nSubSubBlkW		(Error: Reference source not found-33)

    - The derivation process for a disparity sample block as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), variables nSubBlkW and nSubBlkH, the array of reconstructed depth samples refDepPels, the luma location ( xTL, yTL ), the variables nSubSubBlkW and nSubSubBlkW, the view order index refViewIdx, the array disparitySamples, and the variable minFlag as the inputs, and the output is the modified array disparitySamples.

H.8.5.5.2.1	Derivation process for a disparity sample sub block

- Inputs to this process are:

- a luma location ( xB, yB ) of the top-left luma sample of the current sub block relative to the top-left luma sample of the current prediction unit,

- variables nSubBlkW and nSubBlkH specifying the width and the height, respectively, of the current sub block.

- an array of reconstructed depth samples refDepPels

- a luma location ( xTL, yTL ) of the top-left luma sample of the current reference block relative to the top-left luma sample of the array of reconstructed depth samples,

- variable nSubSubBlkW and nSubSubBlkW specifying the conversion precision of the corresponding depth samples,

- a view order index refViewIdx specifying a reference view,

- an array disparitySamples of disparities values

- a variable minFlag

Outputs of this process are:

- an modified array disparitySamples of disparities values.

The array disparitySamples is modified as specified in the following:

- For sBy in the range of 0 to ( ( nSubBlkH / nSubSubBlkH) –1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nSubBlkW / nSubSubBlkW) –1 ), inclusive, the following applies:

    - The variable repDep is set equal to –1 and modified as specified in the following.

xSubB = xB + sBx * nSubSubBlkW
		ySubB = yB + sBy * nSubSubBlkH
		xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xSubB )
		yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + ySubB ) 
		xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xSubB + nSubSubBlkW – 1 )
		yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + ySubB + nSubSubBlkH – 1 )
       if( minFlag == 0) {
			repDep = Max(repDep, refDepPels[ xP0 ][ yP0 ] )
			repDep = Max(repDep, refDepPels[ xP0 ][ yP1] )
			repDep = Max(repDep, refDepPels[ xP1][ yP0 ] )
			repDep = Max(repDep, refDepPels[ xP1][ yP1 ] )
       }
       else {
             		repDep = Min( repDep, refDepPels[ xP0 ][ yP0 ]
                 repDep = Min( repDep, refDepPels[ xP0 ][ yP1 ]
                 repDep = Min( repDep, refDepPels[ xP1 ][ yP0 ]
                 repDep = Min( repDep, refDepPels[ xP1 ][ yP1 ]
       }

    - The values of the array depthSamples are modified as specified in the following:

for ( yOff = 0; yOff < nSubSubBlkH; yOff++ )
			for( xOff = 0; xOff < nSubSubBlkW; xOff++ ) {
				x = xSubB + xOff 
				y = ySubB + yOff 
				disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]
			}