| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0145 |
| --- | --- |

| Title: | 3D-CE4: Results on Simplification of Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the residual prediction is applied to 2Nx2N merge and AMVP modes. In order to reduce the complexity at both encoder and decoder, it is proposed to apply the residual prediction only to the 2Nx2N merge modes. The proposed method reportedly provides no coding loss with 97.7% encoding time. With the proposed method, the number of mode decision can be reduced and also the DV derivation can be only performed when the current coding unit is coded with merge mode. Additionally, regarding the signalling ic_flag and iv_res_pred_weight_idx, the disabling the illumination compensation when iv_res_pred_weight_idx is greater than 0 has the same meaning to the disabling the residual prediction when ic_flag is equal to 1. Therefore, this contribution examines both cases. Firstly, when disabling iv_res_pred_weight_idx this method with applying the residual prediction only to 2Nx2N merge modes reportedly provides 0.03% and 0.04% bit-savings for coded and synthesized views with about 96.5% encoding time. Secondly, when disabling ic_flag this method with applying the residual prediction only to 2Nx2N merge modes reportedly provides 0.01% and 0.02% bit-savings for coded and synthesized views with about 96.4% encoding time.

# Introduction

In the current 3D-HEVC [1], the residual prediction is used to predict the residual date of the current coding unit from the one of the corresponding block in the reference view. However, since the residual prediction has two weighting factor 0.5 and 1, two more decisions at the encoder are needed for every 2Nx2N coding unit. Therefore, the mode decision complexity needs to be reduced while maintaining the coding performance. Additionally, regarding the signalling ic_flag and iv_res_pred_weight_idx, the disabling the illumination compensation when iv_res_pred_weight_idx is greater than 0 has the same meaning to the disabling the residual prediction when ic_flag is equal to 1. Therefore, this contribution examines both cases.

# Proposed Method

## Applying RP only to 2Nx2N merge modes

The residual prediction in the current 3D-HEVC [1] is applied to 2Nx2N merge modes and 2Nx2N AMVP modes, thus the number of mode decision for the residual prediction in 2Nx2N partition size is six. Therefore, in order to reduce the mode decision complexity, it is proposed to apply the residual prediction only to 2Nx2N merge mode. With the proposed method, the number of the mode decision can be reduced from six to three and also the DV derivation can be only performed when the current coding unit is coded with merge mode.

## Signaling ic_flag and iv_res_pred_weight_idx

At the last meeting, the method for disabling the residual prediction when ic_flag is equal to 1 was proposed in JCT3V-E0144 [2]. However, the disabling the illumination compensation when iv_res_pred_weight_idx is greater than 0 has the same meaning to the disabling the residual prediction when ic_flag is equal to 1 as shown in Figure 1. Therefore, this contribution examines both cases with the method to apply RP only to 2Nx2N merge modes.

Figure 1. Relationship between ic_flag and iv_res_pred_weight_idx

# Experimental Results

The simulation was carried out on the common test conditions [3] and was based on HTM 8.0 reference software.

## Applying RP only to 2Nx2N merge modes

The proposed method provides no coding loss with 97.7% encoding time as shown in Table 1. The simulation results show that the residual prediction is only suitable for the merge modes and the proposed method can reduce the mode decision complexity.

Table 1. Comparison between the CTC-anchor and the proposed method 
(Applying RP only to 2Nx2N merge mode)

## Signaling ic_flag and iv_res_pred_weight_idx

The method for disabling the residual prediction when ic_flag is equal to 1 provides -0.03% and -0.04% bit-saving for coded and synthesized views with 96.5% encoding time. And the method for disabling the illumination compensation when iv_res_pred_weight_idx is greater than 0 provides -0.01% and -0.02% bit-saving for coded and synthesized views with 96.4% encoding time. From the results, the disabling the residual prediction has even slightly better performance than the disabling the illumination compensation.

Table 2. Comparison between the CTC-anchor and 
the method for disabling RP when ic_flag = 1

Table 3. Comparison between the CTC-anchor and 
the method for disabling IC when iv_res_pred_weight_idx > 0

# Conclusion

In this contribution, it is proposed to allow the residual prediction in only 2Nx2N merge modes and to disable the residual prediction when ic_flag is equal to 1. The proposed methods provide 96.5% encoding time saving with even slightly better coding performance (0.03% and 0.04% bit-saving for coded and synthesized views). Therefore, it is recommended to adopt proposed methods into next 3D-HEVC Draft Text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” JCT3V-E1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[2] M. W. Park, J. Y. Lee, J. Yoon, C. Kim, “Simplification and improvement of ARP for 3D-HEVC,” JCT3V-E0144, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-E1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.