| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0145_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Draft Text for JCT3V-F0145 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Draft Text. The changes are highlighted in green.

H.7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if ( iv_res_pred_flag[ nuh_layer_id ] && ic_flag && TempRefPicInListsFlag ) |  |
| iv_res_pred_weight_idx | ae(v) |
| } |  |
| else { |  |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if ( iv_res_pred_flag[ nuh_layer_id ] && ic_flag && merge_flag[ x0 ][ y0 ] && TempRefPicInListsFlag ) |  |
| iv_res_pred_weight_idx | ae(v) |
| } … |  |
| } |  |
| … |  |
| } |  |

H.7.4.9.5	Coding unit semantics

…

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

  - icEnableFlag = ( merge_idx[ x0 ][ y0 ] ! = 0 ) | | !slice_ic_disable_merge_zero_idx_flag 	(Error: Reference source not found-1)

- Otherwise ( merge_flag[ x0 ][ y0 ] is equal to 0 ), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to ViewIdx( RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ] ).

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L0 && refViewIdxL1  !=  ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L1 && refViewIdxL0  !=  ViewIdx)	(Error: Reference source not found-2)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.