| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0146 |
| --- | --- |

| Title: | Illumination Compensation for View Synthesis Prediction Candidate for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Ho-Cheon Wey Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com hc.wey@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, even though ic_flag is always signalled for every view synthesis prediction (VSP) candidate, the VSP candidate does not apply the illumination compensation, i.e., ic_flag is always set to 0 for the VSP candidates. This is because the current 3D-HEVC does not have a proper method to support the illumination compensation for VSP process. Therefore, in order to support the illumination compensation for the VSP candidate, it is proposed to perform the disparity compensation prediction (DCP) with the illumination compensation when the VSP candidate has ic_flag equal to 1. The proposed method reportedly provides 0.1% bit-saving for overall coded and synthesized views.

# Introduction

In 3D-HEVC [1], ic_flag, which indicates whether the illumination compensation is performed or not, is always signalled for every view synthesis prediction (VSP) candidate, but the illumination compensation is currently never used, this is, ic_flag is always set to 0 for the VSP candidate. This is because the current 3D-HEVC does not have a proper method to support the illumination compensation for VSP process. However, if there is an appropriate method to make up for these cases, it is expected that the more coding gains can be obtained.

# Proposed Method

In this contribution, it is proposed to simply perform the disparity compensation prediction (DCP) with the illumination compensation when the VSP candidate has ic_flag equal to 1.

Figure 1. Comparison between the current and the proposed inter-view prediction process

The current and the proposed inter-view prediction process can be illustrated as Figure 1. In the current inter-view prediction process, if vsp_flag, which indicates whether the current prediction unit is the VSP candidate or not, is equal to 0, the DCP is performed and then the illumination compensation is additionally performed according to the value of ic_flag. Otherwise, if vsp_flag is equal to 1, which means the current prediction unit is the VSP candidate, the VSP is performed. In order to support the illumination compensation for the VSP candidate, the propose method simply adds one checking condition, which ic_flag is equal to 1, to the first condition as shown in Figure 1.

# Experimental Results

The simulation was carried out on the common test conditions [2] and was based on HTM 8.0 reference software. The performance of the proposed method is shown in Table 1. The proposed method provides an average of 0.1% bit-saving for overall coded and synthesized views.

Table 1. Comparison between the CTC anchor and the proposed method (3-view test case)

# Conclusion

In this contribution, to support the illumination compensation for the VSP candidate, it is proposed to simply perform the disparity compensation prediction (DCP) with the illumination compensation when the VSP candidate has ic_flag equal to 1. The changes in text and software are minor, but the proposed method provides 0.1% bit-saving for coded and synthesized views. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Draft Text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” JCT3V-E1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-E1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2013, Vienna, Austria.