| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0147 |
| --- | --- |

| Title: | 3D-CE5 related: DMM simplification and signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Ho-Cheon Wey, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

It is reported that some depth modelling modes (DMM) have the minor or no coding gain in the previous meetings. As a follow-up proposal of D0123 and E0146, this contribution evaluates the performance of DMMs again on the top of 3D-HTM 8.0. Currently, four DMM modes, which consist of DMM1, DMM3, DMM4, and RBC, were employed. However, it is observed that DMM3 and RBC have the negative impact on the coding performance. Hence, the proposed removes these two modes in the depth intra mode sets. Results demonstrate that the proposed method improves the coding gain by -0.1% in terms of the coding PSNR and reduces the encoding time by about 95%, respectively. Furthermore, the specification texts related to DMM3 and RBC are significantly cleaned up.

# DMM Simplification and Signalling

Four DMMs have been added to the conventional intra modes for the efficient edge coding as follows:

- DMM1: Explicit wedgelet signalling;

- DMM3: Inter-component prediction of wedgelet partitions;

- DMM4: Inter-component-predicted contour partitioning;

- RBC  : Region boundary signalling with chain codes.

Since DMMs have been developed with similar concept, they are very redundant. Especially, DMM3 and RBC modes have the negative impact on the coding performance. Hence, the proposed removes these two modes in the depth intra mode sets.

As the first experiment, the proposed method removes DMM3, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 0 | - | 0 |
| INTRA_DEP_NONE | 10 | 0 | 1 |
| INTRA_DEP_SDC_DMM_WFULL | 110 | - | - |
| INTRA_DEP_DMM_WFULL | 1110 | 10 | - |
| INTRA_DEP_DMM_CPREDTEX | 11110 | - | - |
| INTRA_DEP_CHAIN | 11111 | 11 | - |

As the second experiment, the proposed method removes RBC, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 0 | - | 0 |
| INTRA_DEP_NONE | 10 | 0 | 1 |
| INTRA_DEP_SDC_DMM_WFULL | 110 | - | - |
| INTRA_DEP_DMM_WFULL | 1110 | 10 | - |
| INTRA_DEP_DMM_CPREDTEX | 11110 | - | - |
| INTRA_DEP_DMM_WPREDTEX | 11111 | 11 | - |

As the third test, the proposed method removes DMM3, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 0 | - | 0 |
| INTRA_DEP_NONE | 10 | 0 | 1 |
| INTRA_DEP_SDC_DMM_WFULL | 110 | - | - |
| INTRA_DEP_DMM_WFULL | 1110 | 1 | - |
| INTRA_DEP_DMM_CPREDTEX | 1111 | - | - |

# Test Results

Tables 1 and 2 show the performance in CTC and all intra case, respectively, when DMM3 is removed.

Table 1. Removal of DMM3 (CTC)

Table 2. Removal of DMM3 (All intra case)

Tables 3 and 4 show the performance in CTC and all intra case, respectively, when RBC is removed.

Table 3. Removal of RBC (CTC)

Table 4. Removal of RBC (All intra case)

Tables 5 and 6 show the performance in CTC and all intra case, respectively, when DMM3 and RBC are removed.

Table 5. Removal of DMM3 and RBC (CTC)

Table 6. Removal of DMM3 and RBC (All intra case)

Through the exhaustive experiments above, it is observed that DMM3 and RBC give some coding loss. Furthermore, this DMM simplification and signaling makes the specification text much simpler than the current one, as shown in the proposed specification text.

# Conclusion

By integrating the proposed methods on 3D-HTM 8.0, it significantly reduces the encoding times with the very small coding loss at the same time simplifies the current design and specification. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).