| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0148 |
| --- | --- |

| Title: | 3D-CE5: Simplified depth coding based on most probable mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Byeongdoo Choi, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Simplified depth coding (SDC) is employed for efficient depth intra coding in 3D-HEVC. Currently, SDC mode consists of Planar and DMM1 modes, and its partition size is always 2Nx2N. Instead of transform and quantization processes for coding of residual samples, SDC signals one or two constant values, which are the average difference between the original depth values and the predicted depth values. The proposed method uses the most probable modes as SDC modes to improve the coding performance, instead of Planar and DMM1 modes. The results demonstrate that the proposed method achieves the gain by -0.2% in both CTC and all intra condition. It is also observed that the proposed method reduces bit rate of -0.2% in the additional sequence Shark.

# SDC Based on Most Proposed Mode

The proposed SDC mode employs the most probable modes to improve the coding performance, instead of Planar and DMM1 modes. When block size is less than 64x64, DMM1 mode is used for the third most probable mode. In general, based on the mode information of the neighboring blocks, the most probable modes are decided. If the neighboring blocks are coded with the depth modeling modes such as DMM and Chain modes, Planar mode is instead used. In addition, the proposed method uses the four corner pixels for DMM1 mode and every fourth pixel for the angular modes in the residual DC calculation part, respectively.

Table 1. SDC mode construction

| Block size | SDC modes |
| --- | --- |
| 64x64 | The first, second, and third most probable modes |
| Less than 64x64 | The first and second most probable modes and DMM1 |

# Test Results

Tables 1 and 2 illustrate the coding performance of the proposed method based on the most probable mode in CTC and all intra case, respectively. The proposed method obtains about -0.2% gain in terms of the coded and synthesized PSNR over the total bit rate in CTC and all intra condition.

Table 1. Performance of the proposed method based most probable mode in CTC

Table 2. Performance of the proposed method based most probable mode in all intra case

# Conclusion

By integrating the proposed methods on 3D-HTM 8.0, it improves the coding efficiency of about -0.2%. Therefore, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).