| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0149 |
| --- | --- |

| Title: | 3D-CE5 related: Simplified depth inter mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Simplified depth inter mode coding (E0156) is adopted at the last meeting and implemented in 3D-HEVC. It encodes one representative value (RV) for each PU instead of the residual for all pixels within a CU. RV is calculated by average difference (DC) between the original and predicted pixels. However, this method is not very efficient in the partitioned PUs such as 2NxN, Nx2N, NxN, and so on. Since the correlation between the residual values in the partitioned PU is very low, DC cannot represent all the residual values. Hence, the proposed method applies the simplified depth inter mode coding to only 2Nx2N. In addition, to find more accurate RV for all the residual, the proposed method tests three candidates, which include DC, DC-1, and DC+1. The best RV is decided and coded through RDO. The results demonstrate that the proposed method achieves the gain by -0.3% in the synthesized view. It is also observed that the proposed method reduces bit rate of -0.5% in the additional sequence Shark. In addition, this contribution defines a flag to enable or disable the simplified depth inter mode coding in VPS extension

# Proposed Simplified Depth Intra Mode Coding Method

Currently, the simplified depth inter mode coding encodes one RV for each PU instead of the residual for all pixels within a CU. RV is calculated by DC between the original and predicted pixels (RV=DC). As shown in Fig. 1, the current method is used in all the PUs such as 2Nx2N, 2NxN, Nx2N, NxN, 2NxnU, 2NxnD, nLx2N, and nRx2N.

# Fig. 1. PUs for the simplified depth intra mode coding

Unlike 2Nx2N, the correlation between the residual values in the partitioned PUs such as 2NxN, Nx2N, NxN, 2NxnU, 2NxnD, nLx2N, and nRx2N is very low. Therefore, DC cannot represent all the residual values. Based on this observation, the proposed method applies the simplified depth inter mode coding to only 2Nx2N. In addition, to find more accurate RV for all the residual, the proposed method tests three candidates, which include DC, DC-1, and DC+1, and decides the best RV through RDO (RV=k, k={DC-1, DC, DC+1}). Fig. 2 shows three different RV candidates. When CU is coded by the simplified depth inter mode, the magnitude of RV is coded and then its sign is signaled.

# Fig. 2. Three RV candidates in 2Nx2N

# Simplified Depth Intra Mode Coding Flag

In order to enable or disable the simplified depth inter mode coding, a flag is defined in VPS extension. In the current SW, this flag is implemented, but missed in WD.

# Test Results

Test results of the proposed method are summarized in Table 1. The coding result of the proposed method obtains about -0.3% gain in terms of the synthesized PSNR over the total bit rate. It is also observed that the proposed method reduces bit rate of -0.5% in the additional sequence Shark.

Table 1. Performance of the proposed method based most probable mode in CTC

# Conclusion

By integrating the proposed method on 3D-HTM 8.0, it improves the coding efficiency of -0.3%. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).