| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0150 |
| --- | --- |

| Title: | 3D-CE3 related: MPI candidate in depth merge mode list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Motion information inheritance (MPI) candidate is used in depth merge mode. The MPI candidate reuses the motion information in the collocated texture block and is added at the first position in the merge mode list. It is always used in the depth merge mode, when the depth image is coded. In this contribution, a flag is defined to enable or disable MPI in VPS, so the MPI candidate is only added when the flag is equal to 1. In addition, the current SW employs 5 candidates in the depth merge mode, but WD describes 6. The max number of the texture merge candidates depends on an inter-view motion vector flag in VPS extension (5 +iv_mv_pred_flag). In the same manner as the texture merge mode list construction, the first option in the proposed method employs 5 or 6 merge candidates, based on the MPI flag in VPS. As the second option, the proposed method always uses 5 candidates regardless of the MPI flag.

# MPI Flag Signalling

The MPI candidate is always added in the depth merge mode list, when the depth image is coded. In order to enable or disable MPI, an MPI flag mpi_flag is defined in VPS extension. Therefore, when mpi_flag is equal to 1, the MPI candidate is only added to the depth merge mode list.

# Depth Merge Mode List Construction

In the texture merge mode list construction, the max number of the candidates (MaxNumMergeCand) depends on an inter-view motion vector flag in VPS extension

MaxNumMergeCand = 5 + iv_mv_pred_flag[ nuh_layer_id ]

Currently, the max number of the depth merge candidates in 3D-HTM 8.0 is 5, but WD describes 6 ones.

To fix the problem above, two methods are proposed.

(Option 1) The proposed method employs 5 or 6 merge candidates, based on the MPI flag mpi_flag in VPS, in the same manner as the texture merge mode list construction.

MaxNumMergeCand = 5 + iv_mv_pred_flag[ nuh_layer_id ] + mpi_flag[ nuh_layer_id ]

(Option 2) The proposed method always uses 5 candidates regardless of the MPI flag in the depth merge mode.

MaxNumMergeCand = 5 + iv_mv_pred_flag[ nuh_layer_id ]

# Test Results

Test results of the proposed method are summarized in Tables 1 and 2. Table 1 is the result of the option 1, when the max number of the candidates is decided by the MPI flag (MaxNumMergeCand=6). Table 2 is the result of the option 2, when the max number of the candidates is 5 regardless of the MPI flag. Since the number of the merge candidates in the option 2 is same as that in the current SW, the result in Table 2 is exactly same as the anchor.

Table 1. Performance for the option 1

Table 2. Performance for the option 2

# Conclusion

Since the proposed method fixes the several problems in the depth merge mode list construction, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).