| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0151 |
| --- | --- |

| Title: | Removal of IC in depth coding and IC flag signalling in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In current 3D-HEVC, Illumination compensation (IC) is used in both texture and depth image coding. It compensates the illumination difference between dependent views. However, it is observed that the depth IC gives the negative impact on the overall coding performance and the encoding complexity. Therefore, the proposed method removes the depth IC in 3D-HEVC. In addition, IC flag is defined in VPS extension. It enables or disables the IC tool and control slice_ic_flag in slice header.

# Removal of IC in Depth Coding

In order to compensate the illumination difference between different views, IC is used in both texture and depth image component coding. However, it is observed that IC in the depth components decreases the overall coding performance. Therefore, the proposed method removes the depth IC in 3D-HEVC.

- H.7.3.6 Slice segment header syntax

- H.7.3.6.1 General slice segment header syntax

if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) )

pred_weight_table( )

else if(!DepthFlag layer_id && !MvHevcCompatibilityFlag ) {

slice_ic_enable_flag

if( slice_ic_enable_flag )

slice_ic_disable_merge_zero_idx_flag

}

five_minus_max_num_merge_cand

}

# IC Flag Signalling

In order to enable or disable the IC tool, an IC flag ic_flag is defined in VPS extension. Also, this flag can controls slice_ic_flag in slice header. When ic_flag is equal to 0 i.e. IC is disabled, slice_ic_flag is not signalled and inferred to be equal to 0

- H.7.3.2.1.2 Video parameter set extension 2 syntax

ic_flag[ layerId ] indicates whether illumination compensation is used in the decoding process of the layer with nuh_layer_id equal to layerId. ic_flag[ layerId ] equal to 0 specifies that illumination compensation is not used for the layer with nuh_layer_id equal to layerId. ic_flag[ layerId ] equal to 1 specifies that illumination compensation may be used for the layer with nuh_layer_id equal to layerId. When not present, the value of ic_flag[ layerId ] shall be inferred to be equal to 0. vps_extension2( ) {

while( !byte_aligned( ) )

vps_extension_byte_alignment_reserved_one_bit

for( i = 0; i <= vps_max_layers_minus1; i++ ) {

layerId = layer_id_in_nuh[ i ]

if ( layerId ! = 0 ) {

cp_present_flag[ layerId ]

if ( !VpsDepthFlag[ layerId ] ) {

iv_mv_pred_flag[ layerId ]

iv_res_pred_flag[ layerId ]

depth_refinement_flag[ layerId ]

view_synthesis_pred_flag[ layerId ]

ic_flag[ layerId ]

} else {

vps_depth_modes_flag[ layerId ]

lim_qt_pred_flag[ layerId ]

if( vps_depth_modes_flag[ layerId ] )

dlt_flag[ layerId ]

if( dlt_flag[ layerId ] ) {

num_depth_values_in_dlt[ layerId ]

for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) {

dlt_depth_value[ layerId ][ j ]

}

}

}

}

}

- H.7.3.6 Slice segment header syntax

- H.7.3.6.1 General slice segment header syntax

if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) )

pred_weight_table( )

else if(ic_flag[ layerId ] && !DepthFlag layer_id && !MvHevcCompatibilityFlag ) {

slice_ic_enable_flag

if( slice_ic_enable_flag )

slice_ic_disable_merge_zero_idx_flag

}

five_minus_max_num_merge_cand

}

# Test Results

Test results of the proposed method are summarized in Tables 1 and 2. The proposed method achieves the gain of -0.1% in terms of the coding PSNR, as shown in Table 1. In addition, the encoding complexity is reduced by 93.7% on average. Table 2 shows the result when IC flag is defined in VPS extension.

Table 1. Removal of depth IC

Table 2. Removal of depth IC and IC flag signaling

# Conclusion

By integrating the proposed methods on 3D-HTM 8.0, it achieves the minor coding gain and the encoding time reduction. Since the modifications are significantly simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).