| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 Jul. –2 Aug. 2013 | Document: JCT3V-E1001-v3 |
| --- | --- |

| Title: | 3D-HEVC Draft Text 1 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | 3D-HEVC working draft |  |  |
| Author(s) or Contact(s): | Gerhard Tech Fraunhofer HHI Krzysztof Wegner Poznan University of Technology Ying Chen Qualcomm Incorporated Sehoon Yea LG Electronics | Email: Email: Email: Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl cheny@qti.qualcomm.com sehoon.yea@lge.com |
| Source: | Editor |  |  |

_____________________________

Abstract

Draft 1 of 3D-HEVC

Ed. Notes (Draft 1) (changes compare to JCT3V-D1005)

- ----------- Release v3 -----------

- Accepted change marks.

- ----------- Release v2 -----------

- (3DC-04) Fix disparity derivation

- (3D-GT6) Review, cleanups.

- (3DC-03) Update definitions.

- (3DC-02/JCT3V-E0159) Removal further unused parts of overlap between DMM1 and DMM3.

- (3DC-01 Fix refIdx for VSP )

- (3DN-22/JCT3V-E0172) Added missing dv-scaling of item 3.

- (3DC-GT5) Revised editors comments. Cleanups. Fixed ticket #41, #42, #43

- (3DE-02) Update to HEVC version 1

- ----------- Release v1 -----------

- (3DN-GT3) Cleanups

- (3DE-CY1): Review and editorial improvements.

- (3DN-23/JCT3V-E0163) Camera parameter presence indication.

- (3DN-22/JCT3V-E0172/Items 3+4) CE2: VSP Fix

- (3DC-GT2) Fix tickets #35, #30, #32, #33, #34, #37

- (3DN-21/JCT3V-E0126) CE3: Merge candidates derivation from vector shifting.

- (3DN-20/JCT3V-E0142,JCT3V-E0190) CE2: Simplified NBDV and improved disparity vector derivation

- (3DN-19/JCT3V-E0207) + JCT3V-E0208 CE1: Adaptive block partitioning for VSP and clipping.

- (3DN-18/JCT3V-E0141) CE2: Clipping in depth-based disparity vector derivation

- (3DN-17/JCT3V-E0156) CE6: Simplified Inter Mode Coding of Depth Decision

- (3DE-01) Added Decoding process for the residual signal of coding units coded in inter predmode from base spec

- (3DN-16/JCT3V-E0034) HLS: Revision of the Alternative Depth Info SEI message

- (3DN-15/JCT3V-E0160) HLS: Make 3D-HEVC Compatible with MV-HEVC Adopt (solution 2)

- (3DN-14/JCT3V-E0134) HLS: Signalling of camera parameters.

- (3DN-13/JCT3V-E0057) HLS: On parameter sets. Adopt View Id aspect

- (3DN-12/JCT3V-E0104) HLS: Only portion that swaps multiview and depth flag in scalability dimension

- (3DN-11/JCT3V-E0182) CE3: A bug-fix for the texture merging candidate

- (3DC-GT1) Review and editorial improvements

- (3DN-10/JCT3V-E0172/Item 5) CE2: Disparity inter-view motion vector derivation

- (3DN-10/JCT3V-E0172/Item 7) CE2: DVMCP Fix

- (3DN-09/JCT3V-E0170) CE3: Motion data buffer reduction for 3D-HEVC Decision: Adopt (first scheme)

- (3DN-08/JCT3V-E0117) CE6: Simplified DC calculation for SDC

- (3DN-07/JCT3V-E0242) CE5: On DMM simplification

- (3DN-06/JCT3V-E0204) CE5: Simplified Binarization for depth_intra_mode

- (3DN-05/JCT3V-E0159) CE5: Removal of Overlap between DMM1 and DMM3

- (3DN-04/JCT3V-E0158) CE6: Removal of DC from SDC Mode

- (3DN-03/JCT3V-E0146) CE5: DMM simplification and signalling. Remove DMM2.

- (3DN-02/JCT3V-E0168) CE4: Complexity reduction of bi-prediction for illumination compensation

- (3DN-01/JCT3V-E0046) CE4: Resampling in IC parameter derivation and 4x4 Chroma removal

Decision: Adopt: JCT3V-E0046

Ed. Notes (TM4) (changes compare to JCT3V-C1005):

- ----------- Release v4 -----------

- Accepted all change marks.

- ----------- Release v3 -----------

- (3DC-GT4) Review and editorial improvements

- ----------- Release v2 -----------

- (3DN-22/JCT3V-D0166) On reference view selection in NBDV and VSP

- (3DC-01 availableFlagDV) Fixed availability flag for disparity vector.

- (3DC-GT3) Review and editorial improvements

- (3DC-CY1) Review and editorial improvements

- ----------- Release v1 -----------

- (3DC-GT2) Clean-up of variables not used any more ( related to disparity derivation)

- (3DN-21/D0220/ViewId) ViewId not reflecting coding order any more.

- (3DN-20/JCT3V-D0272) Signaling Global View and Depth

- (3DN-19/JCT3V-D0103) Signaling Warp Maps as an Alternative 3D Format

- (3DN-18/JCT3V-D0032/JCT3V-D0141/JCT3V-D0034) SDC Residual CABAC contexts.

- (3DN-17/JCT3V-D0035) DLT for DMM deltaDC coding

- (3DN-16/JCT3V-D0195) Unification of new intra modes in 3D-HEVC

- (3DN-15/JCT3V-D0193) Clean-up for 64x64 SDC

- (3DN-14/JCT3V-D0183) Simplified DC predictor for depth intra modes

- (3DN-13/JCT3V-D0110) Sample-based simplified depth coding.

- (3DN-12/JCT3V-D0060) Removal of parsing dependency for illumination compensation

- (3DN-11/JCT3V-D0122) AMVP candidate list construction

- (3DN-10/JCT3V-D0091) Inter-view SAO process in 3DV coding

- (3DN-09/JCT3V-D0177) Advanced residual prediction for multiview coding

- (3DN-08/JCT3V-D0138) Simplified DV derivation for DoNBDV and BVSP

- (3DN-07/JCT3V-D0112) Default disparity vector derivation

- (3DN-06/JCT3V-D0105) BVSP NBDV

- (3DN-05/JCT3V-D0191) Clean-ups for BVSP in 3D-HEVC.

- (3DN-04/JCT3V-D0092) CE1.h related: BVSP mode inheritance

- (Incorporated 8.5.2.1.3  from base spec ) Derivation process for combined bi-predictive merging candidates.

- (3DN-03/JCT3V-D0181) CE2.h related: CU-based Disparity Vector Derivation

- (Incorporated 8.5 from base spec)

- (3DN-02/JCT3V-D0135) CE5: Unification of disparity vector rounding

- (3DN-01/JCT3V-D0156): HLS for stereo compatibility.(Also covers disabling of VSP for depth as proposed in D0105 and D0139).

- (3DC-GT1) Editorial improvements, small corrections.

Ed. Notes (TM3) (changes compare to JCT3V-B1005):

- (3Dc-04) Revised text related to edge intra.

- (3DC-GT2) Editorial improvements, small corrections.(among others tickets #21 #22)

- ----------- Release d0 -----------

- Converted to .doc- File

- Split of Test Model text and specification text

- (3DE-05) Alignment with MV-HEVC draft 3.

- (3DE-01) Reordered sub-clauses related to disparity estimation and additional motion candidates.

- (3DN-20) Alignment of JCT3V-C0152 + JCT3V-C0137.

- (3DN-07/JCT3V-C0137) Texture motion vector candidate for depth.

- (3DN-07/JCT3V-C0137) Removal of MPI.

- (3DN-19) Camera parameters

- (3Dn−03) Wedgelet pattern generation process.

- (3Dn-01) Incorporated missing intra-predicted wedgelet partition mode

- (3DN-08/JCT3V-C0138) Removal of parsing dependency for inter-view residual prediction.

- (3DN-18/JCT3V-C0160) QTL disabled for RAP.

- (3DN-17/JCT3V-C0154) Reference sub-sampling for SDC and DMM.

- (3Dc-03) Fix SDC

- (3DN-16/JCT3V-C0096) Removal of DMM 2 from SDC.

- (3DN-15/JCT3V-C0034) Delta DC processing for DMMs.

- (3DN-14/JCT3V-C0044) Signalling of wedgeIdx for DMM3.

- (3DN-02/JCT3V-C0152) View synthesis prediction (without disparity derivation part).

- (3DN-03/JCT3V-C0112) Restricted search of max disparity.

- (3DN-01,02/JCT3V-C0131,JCT3V-C0152) Disparity derivation from depth maps.

- (3Dc-02) Incorporated missing conditions of long/short-term pictures in AMVP (related to JCT3V-B0046).

- (3DN-13/JCT3V-C0116) Inter-view vector scaling for AMVP.

- (3DE-03) Incorporated derivation process for AMVP from base spec.

- (3DN-12/JCT3V-C0115) Signalling of inter-view motion vector scaling.

- (3DE-02) Incorporated TMVP text from base spec.

- (3DN-09/JCT3V-C0047) Alternative reference index for TMVP.

- (3DN-10/JCT3V-C0051) Unification of inter-view candidate derivation.

- (3DE-01) Revised text related to residual prediction.

- (3DN-06/JCT3V-C0129) Vertical component in residual prediction.

- (3DN-05/JCT3V-C0097/JCT3V-C0141) Temporal blocks first in DV derivation.

- (3DC-GT1) Editorial improvements, small corrections.

- (3DN-04/JCT3V-C0135) Restriction on the temporal blocks for memory bandwidth reduction in DV derivation.

- (3Dn-02) Full sample MV accuracy for depth.

- (3DN-11/JCT3V-C0046) Extension of illumination compensation to depth.

- (3Dc-01) Fix Illumination compensation (including ic_flag for skip).

Ed. Notes (TM2) (changes compared to JCT3V-A1005)

- Accepted changes and marked delta to base spec

- (3DC-GT2) Editorial improvements, small corrections

- (3DC-CY) Editorial improvements, small corrections

- (MVS-02/JCT3V-B0046) Treatment of inter-view pictures as long term- reference pictures

- (3DE-11) Revised text related to 3Dn-01

- (3Dn-01/m23639) Results on motion parameter prediction

- (3DE-12) Revised text related residual prediction

- (3DE-10) Revised text Related to Illumination compensation.

- (3DN-01/JCT3V-B0045) Illumination compensation for inter-view prediction.

- (3Dn-02/m24766) Restricted Inter-View Residual Prediction

- (3DE-09) Revised text related to depth intra: Edge Intra

- (3DE-09) Revised text related to depth intra: SDC

- (3DE-09) Revised text related to depth intra: DMMs

- (3DO-01/JCT3V-B0131) Depth distortion metric with a weighted depth fidelity term

- (3DN-12/JCT3V-B0036) Simplified Depth Coding with an optional Depth LUT

- (3DN-13/JCT3V-B0039) Simplified Wedgelet search for DMM modes 1 and 3

- (3DN-03/JCT3V-B0083) Unconstrained motion parameter inheritance

- (3DE-08) Incorporated context tables for SDC

- (3DE-07) Improved MPI text.

- (3DN-02/JCT3V-B0068) Incorporated Depth Quadtree Prediction.

- (3DE-06) Incorporated parsing process, including tables for DMMs.

- (3DE-05) Added missing initialization of invalid motion/disparity parameters

- (3DC-03) Added missing pruning of collocated merge candidate due to number of total candidates.

- (3DE-04) Moved pruning of spatial merge candidate B2 due to number of total candidates.

- (3DE-03) Moved derivation of disparity one level higher in process hierarchy.

- (3DE-02) Inserted "Derivation process for motion vector components and reference indices" from base spec

- (3DC-02) Fixed storage of IvpMvFlagLX and IvpMvDisp.

- (3DN-09-10-11/JCT3V-B0048,B0069,B0086) Modification inter-view merge candidates

- (3DC-01) Fixed derivation of inter-view merge candidates.

- (3DE-01) Revised derivation of disparity from temporal candidates

- (3DN-04/JCT3V-B0047) Improvements for disparity vector derivation)

- (3DN-08/JCT3V-B0136) Support of parallel merge in disparity vector derivation

- (3DN-05/JCT3V-B0135) Modified disparity vector derivation process for memory reduction

- (3DN-04/JCT3V-B0111) Decoupling inter-view candidate for AMVP

- (3DN-07/JCT3V-B0096) Removal of dependency between multiple PUs in a CU for DV-derivation

- (3DC-GT) Small corrections, editorial improvements

Ed. Notes (TM1) ( changes compare to N12744)

- (3D08/JCT3V-A0126) (T,N) Simplified disparity derivation

- (3D16) Moved 3D-tool related flags from SPS to VPS, removal camera parameters

- (3D09/JCT3V-A0049) (N) Inter-view motion prediction modification

- (3D13/JCT3V-A0119) (T) VSO depth fidelity

- (3D07/JCT3V-A0070) (T,N) Region boundary chain coding for depth maps

- (3D06/JCT3V-A0087) (T) RDO selection between Non-Zero Residual and All-Zero Residual Intra

- (3D12) (T) Depth Quadtree Prediction

- (3D15) (N) Fix references

- (3D11) (T,N) Improvement of text of already adopted tools

- (3D10/JCT3V-A0097) (T;N) Disparity vector generation

- (3D02) (N) Removed MV-Part and update to Annex F

- (3D03) (T) Labelling of tools not in CTC/Software. Removal?

- (3D05/JCT3V-A0093) (T) VSO early skip

- (3D04/JCT3V-A0033) (T) VSO model based estimation

- (3D14) (N) Update of low level specification to match HEVC text specification 8(d7)

- (3D01) (N): Removed HEVC text specification

Contents

Abstract	2

Contents	6

List of Figures	7

List of Tables	7

Annex H 	3D High Efficiency Video Coding	9

H.1	Scope	9

H.2	Normative references	9

H.3	Definitions	9

H.4	Abbreviations	9

H.5	Conventions	10

H.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	10

H.7	Syntax and semantics	10

H.7.1	Method of specifying syntax in tabular form	10

H.7.2	Specification of syntax functions, categories, and descriptors	10

H.7.3	Syntax in tabular form	10

H.7.3.1	NAL unit syntax	10

H.7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	10

H.7.3.3	Profile, tier and level syntax	12

H.7.3.4	Scaling list data syntax	12

H.7.3.5	Supplemental enhancement information message syntax	12

H.7.3.6	Slice segment header syntax	13

H.7.3.7	Short-term reference picture set syntax	16

H.7.3.8	Slice segment data syntax	16

H.7.4	Semantics	22

H.7.4.1	General	22

H.7.4.2	NAL unit semantics	22

H.7.4.3	Raw byte sequence payloads, trailing bits, and byte alignment semantics	22

H.7.4.4	Profile, tier and level semantics	25

H.7.4.5	Scaling list	25

H.7.4.6	Supplemental enhancement information message semantics	25

H.7.4.7	Slice segment header semantics	25

H.7.4.8	Short-term reference picture set semantics	26

H.7.4.9	Slice data semantics	26

H.8	Decoding process	31

H.8.1	General decoding process	31

H.8.1.1	Decoding process for a coded picture with nuh_layer_id greater than 0	31

H.8.2	NAL unit decoding process	31

H.8.3	Slice decoding process	31

H.8.3.1	Decoding process for picture order count	31

H.8.3.2	Decoding process for reference picture set	31

H.8.3.3	Decoding process for generating unavailable reference pictures	31

H.8.3.4	Decoding process for reference picture lists construction	31

H.8.3.5	Derivation process for the candidate picture list for disparity vector derivation	32

H.8.3.6	Decoding process for a depth lookup table	32

H.8.3.7	Derivation process for the alternative target reference index for TMVP in merge mode	33

H.8.4	Decoding process for coding units coded in intra prediction mode	33

H.8.4.1	General decoding process for coding units coded in intra prediction mode	33

H.8.4.2	Derivation process for luma intra prediction mode	33

H.8.4.3	Derivation process for chroma intra prediction mode	35

H.8.4.4	Decoding process for intra blocks	35

H.8.5	Decoding process for coding units coded in inter prediction mode	45

H.8.5.1	General decoding process for coding units coded in inter prediction mode	45

H.8.5.2	Inter prediction process	45

H.8.5.3	Decoding process for prediction units in inter prediction mode	45

H.8.5.4	Decoding process for the residual signal of coding units coded in inter prediction mode	71

H.8.5.5	Derivation process for disparity vectors	72

H.8.6	Scaling, transformation and array construction process prior to deblocking filter process	76

H.8.7	In-loop filter process	76

H.9	Parsing process	77

H.9.1	General	77

H.9.2	Parsing process for 0-th order Exp-Golomb codes	77

H.9.2.1	General	77

H.9.2.2	Mapping process for signed Exp-Golomb codes	77

H.9.3	CABAC parsing process for slice segment data	77

H.9.3.1	General	77

H.9.3.2	Initialization process	77

H.9.3.3	Binarization process	82

H.9.3.4	Decoding process flow	84

H.9.3.5	Arithmetic encoding process (informative)	85

H.10	Sub-bitstream extraction process	85

H.11	Profiles and levels	85

H.12	Byte stream format	85

H.13	Hypothetical reference decoder	85

H.14	Supplemental enhancement information	85

H.14.1	General	85

H.14.2	SEI payload syntax	85

H.14.2.1	Alternative depth information SEI message syntax	86

H.14.3	SEI payload semantics	86

H.14.3.1	Alternative depth information SEI message semantics	86

H.15	Video usability information	89

List of Figures

Figure H-1 Fractional sample position dependent variables in bi-linear interpolation  and surrounding integer position samples A, B, C, and D	69

Figure H-2 Relation between camera ID and GVD texture/depth and packing of texture/depth views to the base and residual texture/depth layer	89

List of Tables

Table H-1  – Mapping of ScalabiltyId to scalability dimensions	22

Table H-2 – Specification of DepthIntraMode and associated name depending on depthIntraModeSet and depth_intra_mode and specification of and depthIntraModeMaxLen depending on depthIntraModeSet	28

Table H-3 – Interpretation of edge_code[ x0 ][ y0 ][ k ]	29

Table H-4 – Specification of intra prediction mode and associated names	33

Table H−5 – Specification of resShift	40

Table H−6 – Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE	41

Table H−7 – Specification of xS, yS, xE, yE	42

Table H−8 Specification of xOff, yOff	43

Table H-9 – Specification of the xVsp and yVsp depending on N = mergeCandList[ merge_idx[ xPb][ yPb ] ]	51

Table H-10 – Specification of divCoeff depending on sDenomDiv	67

Table H-11 – Specification of the variables xOff, yOff, and interSdcResiIdx[ i ]	72

Table H-12 – Association of ctxIdx and syntax elements for each initializationType in the initialization process	78

Table H-13 – Values of initValue for wedge_full_tab_idx ctxIdx	79

Table H-14– Values of initValue for wedge_predtex_tab_idx ctxIdx	79

Table H-15 – Values of initValue for dmm_dc_abs ctxIdx	79

Table H-16 – Values of initValue for edge_code ctxIdx	79

Table H-17 – Values of initValue for iv_res_pred_weight_idx ctxIdx	79

Table H-18 – Values of initValue for ic_flag ctxIdx	79

Table H-19 – Values of initValue for sdc_residual_flag ctxIdx	79

Table H-20 – Values of initValue for sdc_residual_abs_minus1 ctxIdx	79

Table H-21 – Values of initValue for depth_intra_mode ctxIdx	80

Table H-22 – Values of initValue for dmm_dc_flag ctxIdx	81

Table H-23 – Values of initValue for edge_dc_flag ctxIdx	81

Table H-24 – Values of initValue for edge_dc_abs ctxIdx	81

Table H-25 – Values of initValue for inter_sdc_flag ctxIdx	81

Table H-26 – Values of initValue for inter_sdc_resi_abs_minus1 ctxIdx	81

Table H-27 – Values of initValue for inter_sdc_resi_sign_flag ctxIdx	81

Table H-28 – Syntax elements and associated binarizations	82

Table H-30 –Values of wedgePredTexTabIdxBits[ log2PUSize ]	83

Table H-31 –Assignment of ctxInc to syntax elements with context coded bins	84

Table H-32 – Specification of ctxInc using left and above syntax elements	85

Table H-33 – Interpretation of depth_type	87

Table H-34– Top-left corner co-ordinates of GVD sub-residual views packed in a residual layer of width = W and height=H.	89

3D High Efficiency Video Coding Draft Specification

The specifications made in this section are based on HEVC version 1 and MV-HEVC draft 5 (JCT3V-E1004).

In text blocks copied from HEVC version 1 or MV-HEVC draft 5 changes are highlighted.

Modifications of HEVC specification:

In Foreword replace paragraph that start with "In this Recommendation | International Standard Annexes":

In this Recommendation | International Standard Annexes A through H contain normative requirements and are an integral part of this Recommendation | International Standard.

In 0.7, add the following paragraph after the paragraph that starts with "Annex F":

Annex H specifies multiview and depth video coding, referred to as 3D High Efficiency Video Coding (3D-HEVC). The reader is referred to Annex H for the entire decoding process for 3D-HEVC, which is specified there with references being made to clauses 1-9 and Annexes A-G.

- Annex H 	3D High Efficiency Video Coding

This annex specifies 3D high efficiency video coding, referred to as 3D-HEVC.

- Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses 2-9 and Annexes A-G.

[Ed. (GT): Some references to Annex F might be replaced to references to Annex G and vice versa. This should be fixed when MV-HEVC structure is finalized. When a referenced subclause does not exist in Annex G, the corresponding subclause in Annex F is valid or vice versa.]

- Normative references

The specifications in clause 2 apply.

- Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause F.3 and G.3. These definitions are either not present in clause F.3 and G.3 or replace definitions in clause F.3 and G.3.

  - depth view: A sequence of pictures associated with the same value of ViewOrderIdx and DepthFlag equal to 1.

  - depth view component: A coded representation of a the depth view.

  - texture view: A sequence of pictures associated with the same value of ViewOrderIdx and DepthFlag equal to 0.

  - texture view component: A coded representation of a the texture view.

  - view component: A coded representation of a view that may contain a depth view component and a texture view component.

- Abbreviations

The specification in clause 4 apply.

- Conventions

The specification in clause 5 apply.

- Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specification in clause 6 apply.

- Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

  - Method of specifying syntax in tabular form

The specifications in subclause 7.1 apply.

  - Specification of syntax functions, categories, and descriptors

The specifications in subclause 7.2 apply.

  - Syntax in tabular form

    - NAL unit syntax

The specifications in subclause G.7.3.1 and all its subclauses apply.

    - Raw byte sequence payloads and RBSP trailing bits syntax

      - Video parameter set RBSP

The specifications in subclause G.7.3.2.1 apply.

[ Ed. (GT): Inclusion of potential VPS extension 3 needs to be specified. ]

        - Video parameter set extension syntax

The specifications in subclause G.7.3.2.1.1 apply.

        - Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| cp_present_flag[ layerId ] | u(1) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| } else { |  |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| if( vps_depth_modes_flag[ layerId ] ) |  |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| cp_precision | ue(v) |
| cp_in_slice_segment_header_flag | u(1) |
| if( !cp_in_slice_segment_header_flag ) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if( cp_present_flag[ layerId ] ) { |  |
| viewIdx = ViewOrderIdx[ layerId ] |  |
| for( j = 0; j < viewIdx; j++ ) { |  |
| vps_cp_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_off[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ viewIdx ][ j ] | se(v) |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

      - Sequence parameter set RBSP syntax

The specifications in subclause G.7.3.2.2 apply.

[ Ed. (GT): Inclusion of a potential further SPS extension needs to be specified. ]

        - Sequence parameter set extension syntax

The specifications in subclause G.7.3.2.2.1 apply.

      - Picture parameter set RBSP syntax

The specifications in subclause G.7.3.2.3 apply.

      - Supplemental enhancement information RBSP syntax

The specifications in subclause G.7.3.2.4 apply.

      - Access unit delimiter RBSP syntax

The specifications in subclause G.7.3.2.5 apply.

      - End of sequence RBSP syntax

The specifications in subclause G.7.3.2.6 apply.

      - End of bitstream RBSP syntax

The specifications in subclause G.7.3.2.7 apply.

      - Filler data RBSP syntax

The specifications in subclause G.7.3.2.8 apply.

      - Slice layer RBSP syntax

The specifications in subclause G.7.3.2.9 apply.

      - RBSP slice trailing bits syntax

The specifications in subclause G.7.3.2.10 apply.

      - RBSP trailing bits syntax

The specifications in subclause G.7.3.2.11 apply.

      - Byte alignment syntax

The specifications in subclause G.7.3.2.12 apply.

    - Profile, tier and level syntax

The specifications in subclause G.7.3.3 apply.

    - Scaling list data syntax

The specifications in subclause G.7.3.4 apply.

    - Supplemental enhancement information message syntax

The specifications in subclause G.7.3.5 apply.

    - Slice segment header syntax

      - General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_reset_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| for( i = 1; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nuh_layer_id > 0 ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if( layer_id && !MvHevcCompatibilityFlag ) { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| slice_segment_header_extension( ) | u(1) |
| slice_segment_header_extension2_flag |  |
| if( slice_segment_header_extension2_flag ) { | u(1) |
| slice_segment_header_extension2_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| [ Ed. (GT): Current extension design needs to be aligned with other extensions. ] |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

        - Slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

      - Reference picture list modification syntax

The specifications in subclause 7.3.6.2 apply.

      - Weighted prediction parameters syntax

The specifications in subclause 7.3.6.3 apply.

    - Short-term reference picture set syntax

The specifications in subclause 7.3.5.2 apply.

    - Slice segment data syntax

The specifications in subclause 7.3.8 apply.

      - General slice segment data syntax

The specifications in subclause 7.3.8.1 apply.

      - Coding tree unit syntax

The specifications in subclause 7.3.8.2 apply.

      - Sample adaptive offset syntax

The specifications in subclause 7.3.8.3 apply.

      - Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && log2CbSize > MinCbLog2SizeY && !predSplitCuFlag ) |  |
| split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, cqtDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, cqtDepth + 1 ) |  |
| } else |  |
| coding_unit( x0, y0, log2CbSize, cqtDepth ) |  |
| } |  |

      - Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if ( iv_res_pred_flag[ nuh_layer_id ] && TempRefPicInListsFlag ) |  |
| iv_res_pred_weight_idx | ae(v) |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ] ) |  |
| depth_mode_parameters( x0 + i , y0+ j , log2CbSize ) |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if ( iv_res_pred_flag[ nuh_layer_id ] && TempRefPicInListsFlag ) |  |
| iv_res_pred_weight_idx | ae(v) |
| } else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if( DepthFlag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |
| inter_sdc_flag | ae(v) |
| if( inter_sdc_flag ) { |  |
| puNum = ( PartMode = = PART_2Nx2N ) ? 1 : ( PartMode = = PART_NxN ? 4 : 2 ) |  |
| for( i = 0; i < puNum; i++ ) { |  |
| inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf && !inter_sdc_flag ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

        - Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if( ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( log2CbSize > 3 && log2CbSize < 6 ) ) |  |
| depth_intra_mode_set_indication_flag | ae(v) |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL) |  |
| wedge_partition_type[ x0 ][ y0 ] | ae(v) |
| if (wedge_partition_type[ x0 ][ y0 ] == ' WEDGE_NON_RECTANGLE ' ) |  |
| { |  |
| wedge_scalar[ x0 ][ y0 ] | ae(v) |
| wedge_ori[ x0 ][ y0 ] | ae(v) |
| } |  |
| wedge_abs_offset[ x0 ][ y0 ] | ae(v) |
| if (wedge_abs_offset[ x0 ][ y0 ] != 0 ) |  |
| { |  |
| wedge_offset_sign[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| if( DmmFlag[ x0 ][ y0 ] ) { |  |
| dmm_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2; i ++ ) { |  |
| dmm_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmm_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| else if( SdcFlag[ x0 ][ y0 ] ) { |  |
| sdcNumSegments = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) ? 2 : 1 |  |
| for( i = 0; i < sdcNumSegments; i++ ) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( sdc_residual_flag[ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | - ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | - ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

      - Prediction unit syntax

The specifications in subclause 7.3.8.6 apply.

      - PCM sample syntax

The specifications in subclause 7.3.8.7 apply.

      - Transform tree syntax

The specifications in subclause 7.3.8.8 apply.

      - Motion vector difference coding syntax

The specifications in subclause 7.3.8.9 apply.

      - Transform unit syntax

The specifications in subclause 7.3.8.10 apply.

      - Residual coding syntax

The specifications in subclause 7.3.8.11 apply.

  - Semantics

    - General

    - NAL unit semantics

      - General NAL unit semantics

The specifications in subclause G.7.4.2.1  apply.

      - NAL unit header semantics

The specification in subclause G.7.4.2.2 apply with the following modifications and additions.

The variable RapPicFlag is derived as specified in the following:

RapPicFlag = ( nal_unit_type  >=  BLA_W_LP  &&  nal_unit_type  <=  RSV_IRAP_VCL23 )	(H-1)

      - Encapsulation of an SODB within an RBSP (informative)

The specifications in subclause G.7.4.2.3 apply.

      - Order of NAL units and association to coded pictures, access units, and video sequences

The specifications in subclause G.7.4.2.4 apply.

    - Raw byte sequence payloads, trailing bits, and byte alignment semantics

      - Video parameter set RBSP semantics

The specifications in subclause G.7.4.3.1 apply, with the following modifications and additions:

vps_extension2_flag equal to 0 specifies that no vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. The variable MvHevcCompatibilityFlag is set equal to !vps_extension2_flag. [ Ed.(GT): At some stage this might be changed to profile Idc. Moreover, vps_extensions for different HEVC extensions need to be harmonized. ]

vps_extension3_flag equal to 0 specifies that no vps_extension3_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension2_flag shall be equal to 1 in bitstreams conforming to Annex H of this Recommendation | International Standard. The value of 1 for vps_extension3_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension3_flag in a VPS NAL unit.

        - Video parameter set extension semantics

The specifications in subclause G.7.4.3.1.1 apply, with the following modifications and additions:

- Table F-1 is replaced by Table H-1.

Table H-1  – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Depth | Depth Flag |
| 1 | Multiview | View Order Index |
| 2-15 | Reserved |  |

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, the variable ViewOrderIdx[ layer_id_in_nuh[ i ] ] specifying the view order index of the i-th layer, the variable VpsDepthFlag[ layer_id_in_nuh[ i ] ] specifying the depth flag of the i-th layer, and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

NumViews = 1

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j = 0; smIdx < 16; smIdx++ )

if( scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

VpsDepthFlag[ lId ] = ScalabilityId[ i ][ 0 ]

ViewOrderIdx[ lId ] = ScalabilityId[ i ][ 1 ]

if( i > 0  &&  ( ViewOrderIdx[ lId ]  !=  ScalabilityId[ i – 1][ 1 ] ) )

NumViews++

ViewScalExtLayerFlag[ lId ] = ( ViewOrderIdx[ lId ] > 0 )

}

The function ViewIdx( picX ) is specified as follows:

  - ViewIdx( picX ) = ViewOrderIdx[ nuh_layer_id of the picture picX ]	(H-2)

The function DepthFlag( picX ) is specified as follows:

  - DepthFlag( picX ) = VpsDepthFlag[ nuh_layer_id of the picture picX ]	(H-3)

The function ViewId( picX ) is specified as follows:

  - ViewId( picX ) = ViewId[ nuh_layer_id of the picture picX ]	(H-4)

The function DiffViewId( picA, picB ) is specified as follows:

  - DiffViewId( picA, picB ) = ViewId( picA ) − ViewId( picB )	(H-5)

        - Video parameter set extension 2 semantics

cp_present_flag[ layerId ] equal to 1 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], may be present for the layer with nuh_layer_id equal to layerId. cp_present_flag[ layerId ] equal to 0 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], are not present. When VpsDepthFlag[ layerId ] is equal to 1 and depth_refinement_flag[ layerId ] or view_synthesis_pred_flag[ layerId ] is equal to 1, the value of cp_present_flag[ layerId ] shall be equal to 1. When not present, the value of cp_present_flag[ layerId ] is inferred to be equal to 0.

[Ed. (CY): it is unclear whether texture layer and depth layer of the same view share the same set of camera parameters or not and if they are not the same, whether duplications are allowed. However, the current design enables the full functionality and optimizations can be considered later.]

[Ed. (GT): In current design camera parameters of depth layers overwrite camera parameters of texture layers, when they have the same ViewIdx. This makes no sense. Moreover camera parameters are not used for depth layers. Parameter presence should be signalled for each ViewIdx. ]

iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of the layer with nuh_layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction is not used for the layer with nuh_layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with nuh_layer_id equal to layerId. When not present, the value of iv_mv_pred_flag[ layerId ] shall be inferred to be equal to 0.

iv_res_pred_flag[ layerId ] indicates whether inter-view residual prediction is used in the decoding process of the layer with nuh_layer_id equal to layerId. iv_res_pred_flag[ layerId ] equal to 0 specifies that inter-view residual prediction is not used for the layer with nuh_layer_id equal to layerId. iv_res_pred_flag[ layerId ] equal to 1 specifies that inter-view residual prediction may be used for the layer with nuh_layer_id equal to layerId. When not present, the value of iv_res_pred_flag[ layerId ] shall be inferred to be equal to 0.

view_synthesis_pred_flag[ layerId ] equal to 0 specifies that view synthesis prediction merge candidates are not used for the layer with nuh_layer_id equal to layerId. view_synthesis_pred_flag[ layerId ] equal to 1 specifies that view synthesis prediction merge candidates might be used for the layer with nuh_layer_id equal to layerId. When not present, the value of view_synthesis_pred_flag[ layerId ] shall be inferred to be equal to 0.

depth_refinement_flag[ layerId ] equal to 0 specifies that depth view components are not used in the derivation process for a disparity vector for the layer with nuh_layer_id equal to layerId. depth_refinement_flag[ layerId ] equal to 1 specifies that depth components are used in the derivation process for a disparity vector for the layer with nuh_layer_id equal to layerId. When not present, the value of depth_refinement_flag[ layerId ] shall be inferred to be equal to 0.

vps_depth_modes_flag[ layerId ] equal to 1 specifies that depth map modelling modes, the chain coding mode and simplified depth coding modes may be used in the decoding process of the layer with layer_id equal to layerId. vps_depth_modes_flag[ layerId ] equal to 0 specifies that depth map modelling modes, the chain coding mode and simplified depth coding modes are not used in the decoding process of the layer with layer_id equal to layerId. When not present, vps_depth_modes_flag[ layerId ] shall be inferred to be equal to 0.

lim_qt_pred_flag[ layerId ] equal to 1 specifies that prediction of a limited quadtree is used for the layer with nuh_layer_id equal to layerId.. lim_qt_pred_flag[ layerId ] equal to 0 specifies that prediction of a limited quadtree is not used for the layer with nuh_layer_id equal to layerId. When not present, the value of lim_qt_pred_flag[ layerId ] shall be inferred to be equal to 0.

dlt_flag[ layerId ] equal to 1 specifies that a depth lookup table is used and that residual DC offset values for PUs with DmmFlag equal to 1 or SdcFlag equal to 1 represent indices of the depth lookup table for layers with nuh_layer_id equal to layerId. dlt_flag[ layerId ] equal to 0 specifies that a depth lookup table is not used and that residual DC offset values for PUs with DmmFlag equal to 1 or SdcFlag equal to 1 represent indices of the depth lookup table for layers with nuh_layer_id equal to layerId. When not present, the value of dlt_flat[ layerId ] shall be inferred to be equal to 0.

num_depth_values_in_dlt[ layerId ] specifies the number of different depth values and the number of elements in the depth lookup table for the layer with nuh_layer_id equal to layerId.

dlt_depth_value[ layerId ][ j ] specifies the j-th entry in the depth lookup table for layers with nuh_layer_id equal to layerId.

cp_precision specifies the precision of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] in the VPS and cp_scale[ i], cp_off[ i], cp_inv_scale_plus_scale[ i], and cp_inv_off_plus_off[ i ] in the slice segment header. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_segment_header_flag equal to 1 specifies that the syntax elements of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] are not present in the video parameter set. A value of camera_parameter_in_slice_header_flag equal to 0 specifies that the vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] syntax elements are present in the slice segment header.

vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] specify conversion parameters for converting a depth value to a disparity value.

iv_mv_scaling_flag equal to 1 specifies that motion vectors used for inter-view prediction in a layer with nuh_layer_id equal to layerId may be scaled based on ViewId[ layerId ] values. iv_mv_scaling_flag equal to 0 specifies that motion vectors used for inter-view prediction in a layer with nuh_layer_id equal to layerId are not scaled based on ViewId[ layerId ] values.

      - Sequence parameter set RBSP semantics

The specifications in subclause G.7.4.3.2 and its subclauses apply, with the following modifications and additions:

sps_extension2_flag equal to 0 specifies that no sps_extension2( ) syntax structure is present in the SPS RBSP syntax structure. sps_extension2_flag equal to 1 specifies that the sps_extension2( ) syntax structure is present in the SPS RBSP syntax structure.

sps_extension3_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension3_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension3_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all sps_extension_data_flag syntax elements that follow the value 1 for sps_extension3_flag in an SPS NAL unit.

      - Picture parameter set RBSP semantics

The specifications in subclause G.7.4.3.3 apply.

      - Supplemental enhancement information RBSP semantics

The specifications in subclause G.7.4.3.4 apply.

      - Access unit delimiter RBSP semantics

The specifications in subclause G.7.4.3.5 apply.

      - End of sequence RBSP semantics

The specifications in subclause G.7.4.3.6 apply.

      - End of bitstream RBSP semantics

The specifications in subclause G.7.4.3.7 apply.

      - Filler data RBSP semantics

The specifications in subclause G.7.4.3.8 apply.

      - Slice layer RBSP semantics

The specifications in subclause G.7.4.3.9 apply.

      - RBSP slice trailing bits semantics

The specifications in subclause G.7.4.3.10 apply.

      - RBSP trailing bits semantics

The specifications in subclause G.7.4.3.11 apply.

      - Byte alignment semantics

The specifications in subclause G.7.4.3.12 apply.

    - Profile, tier and level semantics

The specifications in subclause G.7.4.4 apply.

    - Scaling list

The specifications in subclause G.7.4.5 apply.

    - Supplemental enhancement information message semantics

The specifications in subclause 7.4.6 apply.

    - Slice segment header semantics

      - General slice segment header semantics

The specification in subclause G.7.4.7.1 apply with the following modifications and additions.

The variable DepthFlag is set equal to VpsDepthFlag[ nuh_layer_id ] and the variable ViewIdx is set equal to ViewOrderIndex[ nuh_layer_id ].

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

  - MaxNumMergeCand = 5 − five_minus_max_num_merge_cand + iv_mv_pred_flag[ nuh_layer_id ]	(H-6)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to (5 + iv_mv_pred_flag[ nuh_layer_id ]), inclusive.

[Ed (GT): Check if there is a condition related to the texture merge candidate. ]

slice_ic_enable_flag equal to 1 specifies illumination compensation is enabled for the current slice. slice_ic_enable_flag equal to 0 specifies that illumination compensation is disabled for the current slice, When not present, slice_ic_enable_flag is inferred to be equal to 0.

slice_ic_disable_merge_zero_idx_flag equal to 1 specifies that ic_flag is not present in the coding units with partitioning mode equal to PART_2Nx2N of the current slice when merge_flag is equal to 1 and merge_idx of the prediction unit in the coding unit is equal to 0. slice_ic_disable_merge_zero_idx_flag equal to 0 specifies that ic_flag might be present in the coding units with partitioning mode equal to PART_2Nx2N of the current slice when merge_flag is equal to 1 and merge_idx of the prediction unit in the coding unit is equal to 0. When not present, slice_ic_disable_merge_zero_idx_flag is inferred to be equal to 0.

slice_segment_header_extension2_flag equal to 0 specifies that no slice segment header extension 2 syntax elements are present slice_segment_header_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for slice_segment_header_extension2_present_flag is reserved for future use by ITU-T | ISO/IEC.

slice_segment_header_extension2_length specifies the length of the slice segment segment header extension data in bytes, not including the bits used for signalling slice_segment_header_extension_length itself. The value of slice_segment_header_extension_length shall be in the range of 0 to 256, inclusive.

        - Slice segment header extension semantics

cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a disparity value. When not present, the values of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ], are inferred to be equal to vps_cp_scale[ ViewIdx ][ i ], vps_cp_off[ ViewIdx ][ i ], vps_cp_inv_scale_plus_scale[ ViewIdx ][ i ], and vps_cp_inv_off_plus_off[ ViewIdx ][ i ], respectively, with ViewIdx equal to ViewOrderIndex[ nuh_layer_id ].

[Ed. (GT): Consider adding range limitations for values of above syntax elements. ]

The array DepthToDisparityB[ i ][ d ] specifying the disparity between the current view and the view with ViewIdx equal i corresponding to the depth value d in the view with ViewIdx equal to i and the array DepthToDisparityF[ i ][ d ] specifying the disparity between the view with ViewIdx equal i and the current view corresponding to the depth value d in the current view is derived as specified in the following:

- The variable log2Div is set equal to BitDepthY − 1 + cp_precision.

- For d in range of 0 to ( ( 1  <<   BitDepthY ) − 1), inclusive, the following applies:

  - For i in the range of 0 to ViewIdx − 1, the following applies:

    - offset = ( cp_off[ i ]  <<  BitDepthY ) + ( ( 1  <<  log2Div )  >>  1 )	(H-7)

    - scale = cp_scale[ i ]		(H-8)

    - DepthToDisparityB[ i ][ d ] = ( scale * d + offset )  >>  log2Div	(H-9)

    - invOffset = ( ( cp_inv_off_plus_off[ i ] − cp_off[ i ] )  <<  BitDepthY ) + ( (  1 <<  log2Div )  >>  1 )	(H-10)

    - invScale = ( cp_inv_scale_plus_scale[ i ] − cp_scale[ i ] )	(H-11)

    - DepthToDisparityF[ i ][ d ] = ( invScale * d + invOffset )  >>  log2Div 	(H-12)

      - Reference picture list modification semantics

The specifications in subclause G.7.4.5.2 apply.

    - Short-term reference picture set semantics

The specifications in subclause G.7.4.8 apply.

    - Slice data semantics

      - Slice data semantics

The specifications in subclause 7.4.9.1 apply.

      - Coding tree unit semantics

The specifications in subclause 7.4.9.2 apply, with the following modifications and additions.

Let DepthPic be the picture in the current access unit with ViewIdx( DepthPic ) equal to ViewIdx and DepthFlag( DepthPic ) equal to 1.

Let TexturePic be the picture in the current access unit with ViewIdx( TexturePic ) equal to ViewIdx and DepthFlag( TexturePic ) equal to 0.

The arrays TextureCtDepth TexturePartMod, TexturePredMode, and TextureIntraPredModeY are set equal to the arrays CtDepth, CtPartMode, PredMode, and IntraPredModeY of Texture Pic, respectively...

      - Sample adaptive offset semantics

The specification in subclause 7.4.9.3 apply.

      - Coding quadtree semantics

The specifications in subclause 7.4.9.4 apply.

The variable predSplitCuFlag specifying whether the split_cu_flag is predicted by inter-component prediction is derived as specified in the following.

  - If slice_type is not equal to I, RapPicFlag is equal to 0, and lim_qt_pred_flag[ nuh_layer_id ] is equal to 1, predSplitCuFlag is set equal to ( TextureCtDepth[ x0 ][ y0 ]  <=  ctDepth )

  - Otherwise (slice_type is equal to I or RapPicFlag is equal to 1 or lim_qt_pred_flag[ nuh_layer_id ] is equal to 0), predSplitCuFlag is set equal to 0.

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than MinCbLog2SizeY and predSplitCuFlag is equal to 0 , the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	Otherwise (log2CbSize is equal to MinCbLog2SizeY or predSplitCuFlag is equal to 1), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

      - Coding unit semantics

The specification in subclause 7.4.9.5 apply with the following modifications and additions:

- If slice_type is not equal to I, RapPicFlag is equal to 0 and lim_qt_pred_flag[ nuh_layer_id ] is equal to 1, the variable predPartModeFlag specifying whether part_mode is predicted by inter-component prediction is derived as follows.

predPartModeFlag = ( TextureCtDepth[ x0 ][ y0 ]  = =  ctDepth )  &&  ( TexturePartMode[ x0 ][ y0 ]  ! =  PART_NxN )	(H-13)

- Otherwise (slice_type is equal to I or RapPicFlag is equal to 1 or lim_qt_pred_flag[ nuh_layer_id ] is equal to 0), predPartModeFlag is set equal to 0.

iv_res_pred_weight_idx specifies the index of the weighting factor used for residual prediction. iv_res_pred_weight_idx equal to 0 specifies that residual prediction is not used for the current coding unit. iv_res_pred_weight_idx not equal to 0 specifies that residual prediction is used for the current coding unit. When not present, the value of iv_res_pred_weight_idx is inferred to be equal to 0.

When DefaultDispFlag[ x0 ][ y0 ] is equal to 1, iv_res_pred_weight_idx shall be equal to 0.

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

  - icEnableFlag = ( merge_idx[ x0 ][ y0 ] ! = 0 ) | | !slice_ic_disable_merge_zero_idx_flag 	(H-14)

- Otherwise ( merge_flag[ x0 ][ y0 ] is equal to 0 ), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to ViewIdx( RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ] ).

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L0 && refViewIdxL1  !=  ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L1 && refViewIdxL0  !=  ViewIdx)	(H-15)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

inter_sdc_flag equal to 1 specifies simplified depth coding of residual blocks is used for the current coding unit. inter_sdc_flag equal to 0 specifies simplified depth coding of residual blocks is not used for the current coding unit. When not present, inter_sdc_flag is inferred to be equal to 0.

inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ], inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] are used to derive InterSdcResi[ x0 ][ y0 ][ i ] as follows:

  - InterSdcResi[ x0 ][ y0 ][ i ] = ( 1 − 2 * inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] ) * 
		( inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] + 1 )	(H-16)

rqt_root_cbf equal to 1 specifies that the transform_tree( ) syntax structure is present for the current coding unit. rqt_root_cbf equal to 0 specifies that the transform_tree( ) syntax structure is not present for the current coding unit.. When not present, the value of rqt_root_cbf is inferred to be equal to !SdcFlag[ x0 ][ y0 ].

When DepthFlag is equal to 0, for use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = x0..x0 + (1  <<  log2cbSize) − 1, y = y0..y0 + (1  <<  log2cbSize) − 1:

  - CtDepth[ x ][ y ] = ctDepth		(H-17)

  - CtPartMode[ x ][ y ] = PartMode		(H-18)

  - CbSize[ x ][ y ] = ( 1  <<  log2cbSize )		(H-19)

  - CbPosX[ x ][ y ] = x0		(H-20)

  - CbPosY[ x ][ y ] = y0		(H-21)

        - Depth mode parameter semantics

The variable Log2MaxDmmCbSize is set equal to 5.

depth_intra_mode_set_indication_flag indicates the set of possible depth intra modes.

The variables depthIntraModeSet is derived as specified in the following:

- If log2CbSize is equal to 6, depthIntraModeSet is set equal to 0.

- Otherwise, if log2CbSize is equal to 3 and PartMode[ xC ][ yC ] is equal to PART_NxN, depthIntraModeSet is set equal to 1.

- Otherwise ( (log2CbSize  =  = 3 && PartMode[ xC ][ yC ]  = =  PART_2Nx2N) || (log2CbSize > 3 && log2CbSize < 6 ), the following applies:

  - If depth_intra_mode_set_indication_flag is equal to 1, depthIntraModeSet is set equal to 2.

  - Otherwise ( depth_intra_mode_set_indication_flag is equal to 0 ), depthIntraModeSet is set equal to 3.

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. Table H-2 specifies the value of the variable depthIntraModeMaxLen depending on depthIntraModeSet and the value of the variable DepthIntraMode and the associated name depending on the on depth_intra_mode and depthIntraModeSet.

It is a requirement of bitstream conformance, that when TexturePredMode[ x0 ][ y0 ] is not equal to MODE_INTRA or TextureIntraPredModeY[ x0 ][ y0 ] not in the range of 2 to 34, inclusive, DepthIntraMode is not equal to INTRA_DEP_DMM_WPREDTEX.

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

  - SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(H-22)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WFULL )  | |  	(H-23)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_CPREDTEX )  | |  
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WPREDTEX )

Table H-2 – Specification of DepthIntraMode and associated name depending on depthIntraModeSet and depth_intra_mode and specification of and depthIntraModeMaxLen depending on depthIntraModeSet

|  | depthIntraModeSet | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- |
|  | depthIntraModeMaxLen | 1 | 3 | 3 | 2 |
| DepthIntraMode | Associated name | depth_intra_mode |  |  |  |
| 0 | INTRA_DEP_SDC_PLANAR | 0 | - | 0 | - |
| 1 | INTRA_DEP_NONE | 1 | 0 | - | 0 |
| 2 | INTRA_DEP_SDC_DMM_WFULL | - | - | - | 1 |
| 3 | INTRA_DEP_DMM_WFULL | - | 1 | 3 | - |
| 4 | INTRA_DEP_DMM_CPREDTEX | - | - | - | 2 |
| 5 | INTRA_DEP_DMM_WPREDTEX | - | 2 | 2 | - |
| 6 | INTRA_DEP_CHAIN | - | 3 | 1 | - |

wedge_partition_type[ x0 ][ y0 ] specifies the partition type of of the wedgelet pattern according to Table H-x1.

Table H-x – Specification of wedge_partition_type[ x0 ][ y0 ] and associated names

| wedge_partition_type[ x0 ][ y0 ] | Associated name |
| --- | --- |
| 0 | WEDGE_NON_RECTANGLE |
| 1 | WEDGE_HORIZONTAL |
| 2 | WEDGE_VERTICAL |

wedge_scalar[ x0 ][ y0 ] specifies the slope of the wedgelet pattern. If wedge_scalar[ x0 ][ y0 ] is not present, wedge_scalar[ x0 ][ y0 ] is set to 0.

wedge_ori[ x0 ][ y0 ] specifies the orientation of the wedgelet pattern.

wedge_abs_offset[ x0 ][ y0 ] and  wedge_offset_sign[ x0 ][ y0 ] are used to derive WedgeOffset[ x0 ][ y0 ] as follows:

- If  wedge_scalar[ x0 ][ y0 ] is equal to 2 or 3, the following applies:

  - Set WedgeThres as 1 << (  log2CbSize - 2 )

  - WedgeOffset[ x0 ][ y0 ] = ( 1 - 2 * wedge_offset_sign[ x0 ][ y0 ] ) * 
( ( wedge_abs_offset[ x0 ][ y0 ] - WedgeThres ) > 0 ? (WedgeThres + 2 * ( wedge_abs_offset[ x0 ][ y0 ] - WedgeThres  ) : wedge_abs_offset[ x0 ][ y0 ] )

- Otherwise if wedge_scalar[ x0 ][ y0 ] is equal to 4 or 5, the following applies:

  - Set WedgeThres as ( 1 << (  log2CbSize - 2 ) + 1 << (  log2CbSize - 3 ) )

  - WedgeOffset[ x0 ][ y0 ] = ( 1 - 2 * wedge_offset_sign[ x0 ][ y0 ] ) * 
( ( wedge_abs_offset[ x0 ][ y0 ] - WedgeThres ) > 0 ? (WedgeThres + 2 * ( wedge_abs_offset[ x0 ][ y0 ] - WedgeThres  ) : wedge_abs_offset[ x0 ][ y0 ] )

- Otherwise, WedgeOffset[ x0 ][ y0 ] = ( 1 - 2 * wedge_offset_sign[ x0 ][ y0 ] ) * wedge_abs_offset[ x0 ][ y0 ]

edge_start_left_flag[ x0 ][ y0 ] equal to 0 specifies that the start point of region boundary chain coding is located on the top row boundary of the current block. edge_start_left_flag[ x0 ][ y0 ] equal to 1 specifies that the start point of region boundary chain coding is located on the left column boundary of the current block.

edge_start_position[ x0 ][ y0 ] specifies the column position of the start point of region boundary chain coding when edge_start_left_flag[ x0 ][ y0 ] is equal to 0 and specifies the row position of the start point of region boundary chain coding when edge_start_left_flag[ x0 ][ y0 ] is equal to 1.

num_edge_codes_minus1[ x0 ][ y0 ] +1 specifies the number of edges within the current block.

edge_code[ x0 ][ y0 ][ k ] shall be one of the values shown in Table H-3. edge_code[ x0 ][ y0 ][ k ] is used to derive the edge direction of the k-th edge when edge_intra_flag[ x0 ][ y0 ] is equal to 1.

Table H-3 – Interpretation of edge_code[ x0 ][ y0 ][ k ]

| edge_code[ x0 ][ y0 ][ k ] | edge direction |
| --- | --- |
| 0 | 0° |
| 1 | 45° |
| 2 | −45° |
| 3 | 90° |
| 4 | −90° |
| 5 | 135° |
| 6 | −135° |

dmm_dc_flag[ x0 ][ y0 ] equal to 1 specifies that dmm_dc_abs[ x0 ][ y0 ][ i ] and dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are present, dmm_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  dmm_dc_abs[ x0 ][ y0 ][ i ] and  dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

dmm_dc_abs[ x0][ y0 ][ i ], dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *dmm_dc_sign_flag[ x0 ][ y0 ][ i ] ) * dmm_dc_abs[ x0 ][ y0 ][ i ]	(H-24)

edge_dc_flag[ x0 ][ y0 ] equal to 1 specifies that edge_dc_abs[ x0 ][ y0 ][ i ] and edge_dc_sign_flag[ x0 ][ y0 ][ i ] are present, edge_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  edge_dc_abs[ x0 ][ y0 ][ i ] and  edge_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

edge_dc_abs[ x0][ y0 ][ i ], edge_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *edge_dc_sign_flag[ x0 ][ y0 ][ i ] ) * edge_dc_abs[ x0 ][ y0 ][ i ]	(H-25)

sdc_residual_flag[ x0 ][ y0 ][ i ] equal to 0 specifies that the residual is zero for segment i. sdc_residual_flag equal to 1 specifies that the residual is non-zero and the sdc_residual_sign_flag and sdc_residual_abs[ i ] syntax elements are present for segment i.

sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] and sdc_residual_sign_flag[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

  - SdcResidual[ x0 ][ y0 ][ i ] = 
( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( sdc_residual_mag_minus1[ x0 ][ y0 ][ i ] + 1) 	(H-26)

      - Prediction unit semantics

The specifications in subclause 7.4.9.6 apply.

      - PCM sample semantics

The specifications in subclause 7.4.9.7 apply.

      - Transform tree semantics

The specifications in subclause 7.4.9.8 apply.

      - Motion vector difference coding semantics

The specifications in subclause 7.4.9.9 apply.

      - Transform unit semantics

The specifications in subclause 7.4.9.10 apply.

      - Residual coding semantics

The specifications in subclause 7.4.9.11 apply.

- Decoding process

  - General decoding process

The specifications in subclause F.8.1 apply with the following modifications:.

- "ViewScalExtLayerFlag[ nuh_layer_id ] is equal to 1" is replaced by "ViewScalExtLayerFlag[ nuh_layer_id ] is equal to 1 or VpsDepthFlag[ nuh_layer_id ] is equal to 1"

- All invocations of the process specified in subclause G.8.1 are replaced with invocations of the process specified in subclause H.8.1.1.

    - Decoding process for a coded picture with nuh_layer_id greater than 0

- The decoding process operates as follows for the current picture CurrPic:

  - The decoding of NAL units is specified in subclause G.8.2.

  - The processes in subclause G.8.1.2 and G.8.3.4 specify the following decoding processes using syntax elements in the slice segment layer and above:

    - Prior to decoding the first slice of the current picture, subclause G.8.1.2 is invoked.

    - At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause G.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

    - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1 or iv_res_pred_flag[ nuh_layer_id ] is equal to 1, the decoding process for candidate picture list for disparity vector derivation in subclause H.8.3.7 is invoked at the beginning of the decoding process for each P or B slice. [Ed. (GT): VSP should be added here as condition.]

    - At the beginning of the decoding process for each P or B slice, the derivation process for the alternative target reference index for TMVP in merge mode as specified in subclause H.8.3.5 is invoked.

    - When dlt_flag[ nuh_layer_id ] is equal to 1, the decoding process for the depth lookup table in subclause H.8.3.6 is invoked at the beginning of the decoding process of first slice.

  - The processes in subclauses H.8.4, H.8.5, H.8.6, and H.8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

  - After all slices of the current picture have been decoded, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause G.8.1.3 is invoked.

  - NAL unit decoding process

The specification in subclause G.8.2 apply.

  - Slice decoding process

    - Decoding process for picture order count

The specifications in subclause G.8.3.1 apply.

    - Decoding process for reference picture set

The specifications in subclause G.8.3.2 apply.

    - Decoding process for generating unavailable reference pictures

The specifications in subclause G.8.3.3 apply.

    - Decoding process for reference picture lists construction

The specifications in subclause G.8.3.4 apply.

    - Derivation process for the candidate picture list for disparity vector derivation

[Ed. (GT): This algorithm is different from the algorithm in software. However, it seems to be equivalent. ]

The variable NumDdvCandPics is set equal to 0 and the candidate picture list DdvCandPicList with a number of NumDdvCandPics elements is constructed as follows.

When slice_temporal_mvp_enabled_flag is equal to 1 the following ordered steps apply:

- DdvCandPicList[ 0 ] is set equal to RefPicListX[ collocated_ref_idx ], with X equal to ( 1 – collocated_from_l0_flag ), and NumDdvCandPics  is set equal to 1.

- The variable lowestTemporalIdRefs is set equal to 7.

- NumDdvCandPics, DdvCandPicList[ 1 ] and lowestTemporalIdRefs are derived as specified in the following:

for ( dir = 0; dir < 2 ; dir++) {
	X = dir ? collocated_from_l0_flag : (1 – collocated_from_l0_flag)
	for( i = 0; i  <=  num_ref_idx_lX_default_active_minus1; i++ ) {
		if(  ViewIdx  = =  ViewIdx( RefPicListX[ i ] ) 
			&&  ( X  = =  collocated_from_l0_flag  | |  i  !=  collocated_ref_idx ) 
			&&  ( NumDdvCandPics  ! =  2 ) ) {
			if( RefPicListX[ i ] is a random access view component ) {
				DdvCandPicsList[ 1 ] = RefPicListX[ i ]
				NumDdvCandPics = 2
			}
			else if( lowestTemporalIdRefs > TemporalId of RefPicListX[ i ] )
				lowestTemporalIdRefs = TemporalId of RefPicListX[ i ]
		}
	}
}

- When NumDdvCandPics is equal to 1, the following applies:

pocDistance =255
for( dir = 0; dir < 2 ; dir++) {
	X = dir ? collocated_from_l0_flag : ( 1 – collocated_from_l0_flag )
	for( i =0; i <= num_ref_idx_lX_default_active_minus1; i++ ) {
		if( ViewIdx  = =  ViewIdx( RefPicListX[ i ] )
			&&  ( X  = =  collocated_from_l0_flag  | |  i != collocated_ref_idx )
			&&  TemporalId of RefPicListX[ i ] = = lowestTemporalIdRefs 
			&&  ( Abs( PicOrderCntVal – PicOrderCnt( RefPicListX[ i ] ) ) < pocDistance ) ) {
			pocDistance = Abs( PicOrderCntVal – PicOrderCnt( RefPicListX[ i ] )
			Z = X
			idx = i
		}
 	}
}
if( pocDistance < 255 ) {
	DdvCandPicsList[ 1 ] = RefPicListZ[ idx ]
	NumDdvCandPics =2
}

    - Decoding process for a depth lookup table

This process is only invoked when dlt_flag[ nuh_layer_id ] is equal to 1.

The list elements Idx2DepthValue[ i ] specifying the depth value of the i-th index in the lookup table with i ranging from 0 to  num_depth_values_in_dlt[ nuh_layer_id ] – 1, inclusive is derived as follows.

- For i in the range of 0 to num_depth_values_in_dlt[ nuh_layer_id ] – 1, inclusive,  the elements in Idx2DepthValue are derived as follows:

  - Idx2DepthValue[ i ] is set equal to dlt_depth_value[ nuh_layer_id ][ i ]

The list elements DepthValue2Idx[ d ] specifying the index of depth values d in the lookup table with d ranging from 0 to BitDepthY – 1, inclusive are derived as specified in the following:

for( d = 0; d < BitDepthY; d++ ) {
		idxLower = 0 
		for( iL = 1, foundFlag = 0; iL < num_depth_values_in_dlt[ nuh_layer_id ]  &&  !foundFlag; iL++ ) 
			if( Idx2DepthValue[ iL ] > d ) {
				idxLower = iL – 1
				foundFlag = 1
			}
		idxUpper = num_depth_values_in_dlt[ nuh_layer_id ] – 1
		for( iU = num_depth_values_in_dlt[ nuh_layer_id ] – 2, foundFlag = 0; iU >= 0 && !foundFlag; iU++ ) 
			if ( Idx2DepthValue[ iU ] < d ) {
				idxUpper = iU + 1
				foundFlag = 1
			}
		if( Abs( d – Idx2DepthValue[ idxLower ] ) < Abs ( d – Idx2DepthValue[ idxUpper ] ) ) 
			DepthValue2Idx[ d ] = idxLower
		else
			DepthValue2Idx[ d ] = idxUpper

    - Derivation process for the alternative target reference index for TMVP in merge mode

This process is invoked when the current slice is a P or B slice.

The variables AltRefIdxL0 and AltRefIdxL1 are set equal to –1, TempRefPicInListsFlag is set equal to 0 and the following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

zeroIdxLtFlag = RefPicListX[ 0 ] is a short-term reference picture ? 0 : 1
for( i = 1; i  <=  num_ref_idx_lX_active_minus1  &&  AltRefIdxLX  = =  –1; i++)
	if( ( zeroIdxLtFlag && RefPicListX[ i ] is a short-term reference picture)  | |
		( !zeroIdxLtFlag  &&  RefPicListX[ i ] is a long-term reference picture ) ) 
		AltRefIdxLX = i 
TempRefPicInListsFlag = TempRefPicInListsFlag  | |  !zeroIdxLtFlag  | |  ( AltRefIdxLX  ! =  –1 )

  - Decoding process for coding units coded in intra prediction mode

    - General decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.4.2 are replaced with invocations of the process specified in subclause H.8.4.2.

–	All invocations of the process specified in subclause 8.4.4.1 are replaced with invocations of the process specified in subclause H.8.4.4.1.

    - Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Table H-4 specifies the value for the intra prediction mode and the associated names.

Table H-4 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |
| 35 | INTRA_DMM_WFULL |
| 36 | INTRA_DMM_WPREDTEX |
| 37 | INTRA_DMM_CPREDTEX |
| 38 | INTRA_CHAIN |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8 1.

- If DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_SDC_PLANAR, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_PLANAR.

- Otherwise, if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_SDC_DMM_WFULL, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WFULL.

- Otherwise, if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_WFULL, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WFULL.

- Otherwise, if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_WPREDTEX, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WPREDTEX.

- Otherwise if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_CPREDTEX, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_CPREDTEX.

- Otherwise if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_CHAIN, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_CHAIN.

- Otherwise ( DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_NONE ), IntraPredModeY[ xPb ][ yPb ] is derived as the following ordered steps:

  - The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

  - For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

    - The availability derivation process for a block in z-scan order as specified in subclause 6.4.2 is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

    - The candidate intra prediction mode candIntraPredModeX is derived as follows:

      - If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

      - Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

      - Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

      - Otherwise, if candIntraPredModeX is larger than 34, candIntraPredModeX is set equal to INTRA_DC.

      - Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

  - The candModeList[ x ] with x = 0..2 is derived as follows:

    - If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

      - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

          - candModeList[ 0 ] = INTRA_PLANAR	(H-27)

          - candModeList[ 1 ] = INTRA_DC		(H-28)

          - candModeList[ 2 ] = INTRA_ANGULAR26	(H-29)

      - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

          - candModeList[ 0 ] = candIntraPredModeA	(H-30)

          - candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(H-31)

          - candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(H-32)

    - Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

      - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

          - candModeList[ 0 ] = candIntraPredModeA	(H-33)

          - candModeList[ 1 ] = candIntraPredModeB	(H-34)

      - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

      - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

      - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

  - IntraPredModeY[ xPb ][ yPb ] is derived by applying the following procedure:

    - If prev_intra_luma_pred_flag[ xPb ][ yPb ] is equal to 1, the IntraPredModeY[ xPb ][ yPb ] is set equal to  candModeList[ mpm_idx ].

    - Otherwise, IntraPredModeY[ xPb ][ yPb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..2 is modified as the following ordered steps:

  - When candModeList[ 0 ] is greater than candModeList[ 1 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 1 ] ) = Swap( candModeList[ 0 ], candModeList[ 1 ] )	(H-35)

  - When candModeList[ 0 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 2 ] ) = Swap( candModeList[ 0 ], candModeList[ 2 ] )	(H-36)

  - When candModeList[ 1 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 1 ], candModeList[ 2 ] ) = Swap( candModeList[ 1 ], candModeList[ 2 ] )	(H-37)

- IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

  - IntraPredModeY[ xPb ][ yPb ] is set equal to rem_intra_luma_pred_mode[ xPb ][ yPb ].

  - For i equal to 0 to 2, inclusive, when IntraPredModeY[ xPb ][ yPb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xPb ][ yPb ] is incremented by one.

    - Derivation process for chroma intra prediction mode

The specifications in subclause 8.4.3 apply.

    - Decoding process for intra blocks

      - General decoding process for intra blocks

- Inputs to this process are:

- a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable log2TrafoSize specifying the size of the current transform block,

- a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

- a variable predModeIntra specifying the intra prediction mode,

- a variable cIdx specifying the colour component of the current block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

  - ( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0  <<  1, yTb0  <<  1 )	(H-38)

The variable splitFlag is derived as follows:

- If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

- Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

  - cIdx is greater than 0

  - split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

  - log2TrafoSize is greater than 2

- Otherwise, splitFlag is set equal to 0.

Depending on the value of splitFlag, the following applies:

–	If splitFlag is equal to 1, the following ordered steps apply:

  - The variables xTb1 and yTb1 are derived as follows:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (splitFlag is equal to 0), the following ordered steps apply:

  - The variable nTbS is set equal to 1  <<  log2TrafoSize.

  - The general intra sample prediction process as specified in subclause 8.4.4.2.1 is invoked with the transform block location ( xTb0, yTb0 ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

  - Depending on SdcFlag[  xTb0 ][ xTb0 ], the following applies:

    - If SdcFlag[ xTb0 ][ yTb0 ] is equal to 0, following applies:

      - The scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xTbY, yTbY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

      - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

    - Otherwise ( SdcFlag[ xTb0 ][ yTb0 ] is equal to 1) the following ordered steps apply:

      - The depth value reconstruction process as specified in subclause H.8.4.4.3 is invoked with the location ( xTb0, yTb0 ), the transform size trafoSize set equal to nTbS, the (nTbS)x(nTbS) array predSamples, and the intra prediction mode predModeIntra, as the inputs and the output is a (nTbS)x(nTbS) array resSamples.

  - For x in the range of 0 to nTbS − 1 and y in the range of 0 to nTbS − 1, the following applies:

    - When cIdx is equal to 0, ResSamplesL[ xTb0 + x ][ yTb0 + y ] is set equal to 0.

    - When cIdx is equal to 1, ResSamplesCb[ xTb0 + x ][ yTb0 + y ] is set equal to 0.

    - When cIdx is equal to 2, ResSamplesCr[ xTb0 + x ][ yTb0 + y ] is set equal to 0.

      - Intra sample prediction

        - General intra sample prediction

The specification in subclause 8.4.4.2.1 with the following paragraphs added to the end of subclause apply:

- Otherwise, if predModeIntra is equal to INTRA_DMM_WFULL, the corresponding intra prediction mode specified in subclause H.8.4.4.2.7 is invoked with the location ( xTbY, yTbY ), the sample array p and the transform block size nTbS as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DMM_WPREDTEX, the corresponding intra prediction mode specified in subclause H.8.4.4.2.8 is invoked with the location ( xTbY, yTbY ), the sample array p and the transform block size nTbS as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX, the corresponding intra prediction mode specified in subclause H.8.4.4.2.9 is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_CHAIN, the corresponding intra prediction mode specified in subclause H.8.4.4.2.10 is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS  as the inputs and the output are the predicted sample array predSamples.

        - Reference sample substitution process for intra sample prediction

The specifications in subclause 8.4.4.2.2 apply.

        - Filtering process of neighbouring samples

The specifications in subclause 8.4.4.2.3 apply.

        - Specification of intra prediction mode INTRA_PLANAR

The specifications in subclause 8.4.4.2.4 apply.

        - Specification of intra prediction mode INTRA_DC

The specifications in subclause 8.4.4.2.5 apply.

        - Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

The specifications in subclause 8.4.4.2.6 apply.

        - Specification of intra prediction mode INTRA_DMM_WFULL

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

      - wedgePattern =

      - WedgePatternTable[ Log2( nTbS ) ][ wedge_partition_type [ xTb ][ yTb ] ] 
     [ wedge_scalar[ xTb ][ yTb ]][ WedgeOffset[ xTb ][ yTb ] ]	       (H-39)

  - The depth partition value derivation and assignment process as specified in subclause H.8.4.4.2.11 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ xTb ][ yTb ], the transform size nTbS, the dcOffsetAvailFlag set equal to dmm_dc_flag[ xTb ][ yTb ], intraChainFlag set equal to 0, and the DC Offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- -

      - -

      - -

      - -

        - Specification of intra prediction mode INTRA_DMM_CPREDTEX

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable recTextPic is set equal to the array of the reconstructed luma picture samples of TexturePic..

  - The variable textThresh specifying a threshold for the segmentation of recTextPic is derived as specified in the following.

    - The variable sumTextPicVals is set equal to 0.

    - For x = 0..nTbS − 1 the following applies

      - For y = 0..nTbS − 1 the following applies

          - sumTextPicVals  +=  recTextPic[ xTb + x ][ yTb + y ]	(H-43)

    - The variable textThresh is set equal to ( sumTextPicVals  >>  ( 2 * log2( nTbS ) ) )

  - The variable wedgeletPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following.

    - For x = 0..nTbS − 1 the following applies

      - For y = 0..nTbS − 1 the following applies

          - wedgeletPattern[ x ][ y ] = ( recTextPic[ xTb + x ][ yTb + y ] > textThresh )	(H-44)

  - The depth partition value derivation and assignment process as specified in subclause H.8.4.4.2.11 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgeletPattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to dmm_dc_flag[ xTb ][ yTb ], intraChainFlag set equal to 0, and the DC Offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

        - Specification of intra prediction mode INTRA_CHAIN

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified in the following ordered steps.

  - [Ed. (GT) The process of edge reconstruction (items 1 - 3) need to be specified more in detail. In current state the specification is not precise enough. (#3)]

  - The start point of the chain in the current prediction unit is derived from edge_start_left_flag[ xTb ][ yTb ] and edge_start_position[ xTb ][ yTb ].

- If edge_start_left_flag[ xTb ][ yTb ] is equal to 1, the start point is set as s[ 0 ][ yS ] where yS is equal to edge_start_position[ xTb ][ yTb ].

- Otherwise, the start point is set as s[ xS ][ 0 ] where xS is equal to edge_start_position[ xTb ][ yTb ].

  - The number of edges and the direction of each edge are derived from num_edge_codes_minus1[ xTb ][ yTb ] and edge_code[ xTb ][ yTb ][ k ], for k = 0.. num_edge_codes_minus1[ xTb ][ yTb ]. The direction of the edge is derived from edge_code as shown in Table H-3.

  - The region boundary generated by connecting each edge separates the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 into two regions: the region rA that covers the top-left pixel (x0, y0) and the region rB that covers the remaining region.

For x, y = 0..nTbS − 1 the binary pattern edgePattern[ x ][ y ] is derived as follows:

- If (x, y) is covered by rA, edgePattern[ x ][ y ] =  0.

- Otherwise, edgePattern[ x ][ y ] = 1.

  - The depth partition value derivation and assignment process as specified in subclause H.8.4.4.2.11 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern edgePattern[ x ][ y ], the transform size nTbS, the dcOffsetAvailFlag set equal to edge_dc_flag[ xTb ][ yTb ], intraChainFlag set equal to 1, and the DC offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

        - Depth partition value derivation and assignment process

Inputs to this process are:

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a binary array partitionPattern[ x ][ y ], with x, y =0..nTbS − 1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

- a variable nTbS specifying the transform block size,

- a flag dcOffsetAvailFlag, specifying whether DC Offset values are available,

- a flag intraChainFlag, specifying whether the current  intra prediction mode is equal to INTRA_CHAIN

- the variables dcOffsetP0 and dcOffsetP1, specifying the DC offsets for the block partitions

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variables vertEdgeFlag and horEdgeFlag are derived as specified in the following:

  - vertEdgeFlag = ( partitionPattern[ 0 ][ 0 ]  !=  partitionPattern[ nTbS − 1 ][ 0 ] ) ? 1 : 0	(H-45)

  - horEdgeFlag = ( partitionPattern[ 0 ][ 0 ]  !=  partitionPattern[ 0 ][ nTbS − 1 ] ) ? 1 : 0	(H-46)

The variables dcVal0 and dcVal1 are derived as specified in the following:

- If vertEdgeFlag is equal to horEdgeFlag, the following applies:

    - dcValBR = horEdgeFlag ? 
	( ( p[ −1 ][ nTbS − 1 ] + p[ nTbS − 1 ][ −1 ] )  >>  1 ) : ( 1  <<  ( BitDepthY − 1 ) )	(H-47)

    - dcValLT = ( p[ −1 ][ 0 ] + p[ 0 ][ −1 ] )  >>  1		(H-48)

- Otherwise ( horEdgeFlag is not equal to vertEdgeFlag), the following applies:

    - dcValBR = horEdgeFlag ? p[ −1 ][ nTbS − 1 ] : p[ nTbS − 1 ][ −1 ]	(H-49)

    - dcValLT = horEdgeFlag ? p[ ( nTbS – 1 )  >>  1 ][ −1 ] : p[ −1 ][ ( nTbS – 1 )  >>  1 ]	(H-50)

- The variable dcOffsetScale is derived as follows:

  - dcOffsetScale = intraChainFlag ? Clip3( 1, ( 1 << BitDepthY ) − 1, Round( 2 (QP’Y /10)- 2 ) ) : 1 	(H-51)

- The predicted sample values predSamples[ x ][ y ] are derived as specified in the following:

- For x in the range of 0 to ( nTbS − 1 ), inclusive the following applies:

  - For y in the range of 0 to ( nTbS − 1 ), inclusive the following applies:

    - The variables predDcVal and dcOffset are derived as specified in the following:

        - predDcVal = ( partitionPattern[ x ][ y ] = = partitionPattern[ 0 ][ 0 ] ) ? dcValLT : dcValBR	(H-52)

        - dcOffset = dcOffsetAvailFlag ? ( partitionPattern[ x ][ y ]  = =  0 ? dcOffsetP0 : dcOffsetP1 ) : 0	(H-53)

        - dcOffset = dcOffset * dcOffsetScale		(H-54)

    - If intraChainFlag is equal to 1 or dlt_flag[ nuh_layer_id ] is equal to 0, the following applies:

        - predSamples[ x ][ y ] = predDcVal + dcOffset	(H-55)

    - Otherwise ( intraChainFlag is equal to 0 and dlt_flag[ nuh_layer_id ] is equal to 1 ), the following applies:

        - predSamples[ x ][ y ] = Idx2DepthValue[ DepthValue2Idx[ predDcVal ] + dcOffset ]	(H-56)

        - Specification of tables WedgePatternTable, TextModePredWedgeIndTable.

NOTE 1  − Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this subclause can be carried out once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed lookup tables, such that the derivation process described in this section does not need to be implemented in the decoder at all.

The list WedgePatternTable[ log2BlkSize ] of binary partition patterns of size (1  <<  log2BlkSize )x(1  <<  log2BlkSize), the variable NumWedgePattern[ log2BlkSize ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ] are derived as specified in the following:

t

  - The variable wBlkSize is set equal to ( 1  <<  log2BlkSize )

E

  - For m in the range of 0 to wBlkSize − 1, inclusive, the following applies:

      - For n in the range of 0 to wBlkSize − 1, inclusive,  the following applies:

        - The Wedgelet pattern generation process as specified in subclause H.8.4.4.2.12.1 is invoked with the variable log2BlkSize , the  variable wedge_partition_type[ x0 ][ y0 ], the variable wedge_scalar[ x0 ][ y0 ], the variable wedge_ori[ x0 ][ y0 ] and the variable WedgeOffset[ x0 ][ y0 ]  as inputs and the output is the binary array curWedgePattern.

          - Wedgelet pattern generation process

Inputs to this process are:

- a variable log2BlkSize  specifying the binary partition pattern size,

- wedge_partition_type[ x0 ][ y0 ] specifying the partition type of of the wedgelet pattern

- a variable wedge_scalar[ x0 ][ y0 ] specifying the slope of the wedgelet pattern,

- a variable wedge_ori[ x0 ][ y0 ] specifying the orientation of the wedgelet pattern,

- a variable WedgeOffset[ x0 ][ y0 ] specifying the position of the wedgelet pattern partition line.

Output of this process is:

- binary array WedgePatternTable[log2BlkSize ][ wedge_scalar[ x0 ][ y0 ] ][ wedge_ori[ x0 ][ y0 ] ][ WedgeOffset[ x0 ][ y0 ] ] of size ( 1 <<  log2BlkSize   ) x ( 1 <<  log2BlkSize   )

  - -

E

- The values of variable curPattern[ x ][ y ], are derived as specified by the following ordered steps.

  - Set the variable curSize equal to 1 <<  log2BlkSize

  - For x, y = 0..curSize − 1, curPattern[ x ][ y ] is set equal to 0.

  - If wedge_partition_type[ x0 ][ y0 ] is equal to "WedgeHorizontal", the following applies:

- for ( iX = 0; iX < curSize; iX++ )

- for ( iY = 0; iY < curSize; iY++ )

- curPattern[ iX ][ iY ] = ( iY > (curSize >> 1 ) - WedgeOffset[ x0 ][ y0 ] ) ? 0 : 1;

  - If wedge_partition_type[ x0 ][ y0 ] is equal to "WedgeVertical", the following applies:

- for ( iX = 0; iX < curSize; iX++ )

- for ( iY = 0; iY < curSize; iY++ )

- curPattern[ iX ][ iY ] = ( iX > (curSize >> 1 ) - WedgeOffset[ x0 ][ y0 ] ) ? 0 : 1;

  - Otherwise, the following applies:

      - The array basePointOffset is specified as  basePointOffset[ k ] = {0, -4, 8, -6, 24} with k = 0..4.

      - The array scalarFactor is specified as  basePointOffset[ k ] = {1, 2, 1/2, 4, 1/4} with k = 0..4.

      - If wedge_ori[ x0 ][ y0 ] is equal to 0, set the variable terminalPos as curSize - 1 + ( ( curSize * basePointOffset[wedge_scalar[ x0 ][ y0 ]  ] ) >> 4 ), the following applies:

- for ( iX = 0; iX < curSize; iX++ )

- for ( iY = 0; iY < curSize; iY++ )

- curPattern[ iX ][ iY ] = (terminalPos – curSize * WedgeOffset[ x0 ][ y0 ] - iX > iY / wedge_scalar[ x0 ][ y0 ] ) ? 0 : 1;

      - If wedge_ori[ x0 ][ y0 ] is equal to 1, set the variable terminalPos as -( ( curSize * basePointOffset[wedge_scalar[ x0 ][ y0 ]  ] ) >> 4 ) , the following applies:

- for ( iX = 0; iX < curSize; iX++ )

- for ( iY = 0; iY < curSize; iY++ )

- curPattern[ iX ][ iY ] = ( iX - terminalPos – curSize * WedgeOffset[ x0 ][ y0 ] < iY / wedge_scalar[ x0 ][ y0 ]  ) ? 0 : 1;

f

        - -

    - The pattern WedgePatternTable[log2BlkSize ][ wedge_scalar[ x0 ][ y0 ] ][ wedge_ori[ x0 ][ y0 ] ][ WedgeOffset[ x0 ][ y0 ] ] is set equal to wedgePattern

    - The value of NumWedgePattern is increased by one

      - Depth value reconstruction process

Inputs to this process are:

- a luma location ( xTb, yTb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nTbS specifying the transform block size,

- predicted samples predSamples[ x ][ y ], with x, y =0..nTbS − 1

- the intra prediction mode predModeIntra,

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = 0.. nTbS − 1.

Depending on predModeIntra the array wedgePattern[ x ][ y ] with x, y =0..nTbS − 1 specifying the binary segmentation pattern is derived as follows.

- If predModeIntra is equal to INTRA_DMM_WFULL, the following applies.

    - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]

- Otherwise ( predModeIntra is not equal to INTRA_DMM_WFULL ), the following applies.

  - For x, y = 0..nTbS − 1 wedgePattern[ x ][ y ] is set equal to 0.

Depending on dlt_flag[ nuh_layer_id ] the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

- If dlt_flag[ nuh_layer_id ] is equal to 0, the following applies:

  - For x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - resSamples[ x ][ y ] = predSamples[ x ][ y ] + 	SdcResidual[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(H-59)

- Otherwise ( dlt_flag[ nuh_layer_id ] is equal to 1 ), the following applies:

  - The variables dcPred[ 0 ] and dcPred[ 1 ] are derived as specified in the following:

    - If predModeIntra is equal to INTRA_DC, the following applies:

        - dcPred[ 0 ] = predSamples[ nTbS − 1 ][ nTbS − 1 ]		(H-60)

    - Otherwise, if predModeIntra is equal to INTRA_PLANAR, the following applies:

        - dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nTbS − 1 ] + predSamples[ nTbS − 1 ][ 0 ]
		+ predSamples[ nTbS − 1 ][ nTbS − 1 ] + 2 )  >>  2	(H-61)

    - Otherwise, ( predModeIntra is equal to INTRA_DMM_WFULL ), the following applies.

        - dcPred[ wedgePattern[ 0 ][ 0 ] ] = predSamples[ 0 ][ 0 ]	(H-62)

        - dcPred[ wedgePattern[ nTbS − 1 ][ 0 ] ] = predSamples[ nTbS − 1 ][ 0 ]	(H-63)

        - dcPred[ wedgePattern[ 0 ][ nTbS − 1 ] ] = predSamples[ 0 ][ nTbS − 1 ]	(H-64)

        - dcPred[ wedgePattern[ nTbS − 1 ][ nTbS − 1 ] ] = predSamples[ nTbS − 1 ][ nTbS − 1 ]	(H-65)

  - For x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(H-66)

      - dltIdxResi = SdcResidual[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(H-67)

      - resSamples[ x ][ y ] =	predSamples[ x ][ y ]  + 	Idx2DepthValue[ dltIdxPred + dltIdxResi ]  − 
	dcPred[ wedgePattern[ x ][ y ] ]	(H-68)

  - Decoding process for coding units coded in inter prediction mode

    - General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to 1  <<  ( log2CbSize − 1 ).

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for disparity vectors as specified in subclause H.8.5.4.2 is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as the inputs.

  - The inter prediction process as specified in subclause H.8.5.3 is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays predSamplesL, predSamplesCb, and predSamplesCr.

  - The decoding process for the residual signal of coding units coded in inter prediction mode specified in subclause H.8.5.4 is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays resSamplesL, resSamplesCb, and resSamplesCr.

  - The reconstructed samples of the current coding unit are derived as follows:

    - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the luma coding block location ( xCb, yCb ), the variable nCurrS set equal to nCbSL, the variable cIdx set equal to 0, the (nCbSL)x(nCbSL) array predSamples set equal to predSamplesL, and the (nCbSL)x(nCbSL) array resSamples set equal to resSamplesL as inputs.

    - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the chroma coding block location ( xCb / 2, yCb / 2 ), the variable nCurrS set equal to nCbSC, the variable cIdx set equal to 1, the (nCbSC)x(nCbSC) array predSamples set equal to predSamplesCb, and the (nCbSC)x(nCbSC) array resSamples set equal to resSamplesCb as inputs.

    - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the chroma coding block location ( xCb / 2, yCb / 2 ), the variable nCurrS set equal to nCbSC, the variable cIdx set equal to 2, the (nCbSC)x(nCbSC) array predSamples set equal to predSamplesCr, and the (nCbSC)x(nCbSC) array resSamples set equal to resSamplesCr as inputs.

    - Inter prediction process

The specifications in subclause 8.5.2 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.3 are replaced with invocations of the process specified in subclause H.8.5.3.

    - Decoding process for prediction units in inter prediction mode

      - General

The specifications in subclause 8.5.3.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.3.2 are replaced with invocations of the process specified in subclause H.8.5.3.2.

–	All invocations of the process specified in subclause 8.5.3.3 are replaced with invocations of the process specified in subclause H.8.5.3.3.

      - Derivation process for motion vector components and reference indices

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xBl, yBl ) of the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCbS specifying the size of the current luma coding block,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

- the luma motion vectors mvL0 and mvL1,

- the chroma motion vectors mvCL0 and mvCL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xPb, yPb ) specify the top-left sample location of the current luma prediction block relative to the top-left luma sample of the current picture where xPb = xCb + xBl and yPb = yCb + yBl.

Let the variable currPic and ListX be the current picture and RefPicListX, with X being 0 or 1, of the current picture, respectively.

The function LongTermRefPic( aPic, aPb, refIdx, LX ), with X being 0 or 1, is defined as follows:

- If the picture with index refIdx from reference picture list LX of the slice containing prediction block aPb in the picture aPic was marked as "used for long term reference" at the time when aPic was the current picture, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 1.

- Otherwise, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 0.

The variables vspModeFlag, ivpMvFlagL0 and ivpMvFlagL1 are set equal to 0.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1, as well as predFlagL0 and predFlagL1, the following applies:

- If merge_flag[ xPb ][ yPb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause H.8.5.3.2.1 is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1,, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following applies:

    - The variables refIdxLX and predFlagLX are derived as follows:

        - If inter_pred_idc[ xPb ][ yPb ] is equal to PRED_LX or PRED_BI,

            - refIdxLX = ref_idx_lX[ xPb ][ yPb ]	(H-69)

            - predFlagLX = 1		(H-70)

        - Otherwise, the variables refIdxLX and predFlagLX are specified by:

            - refIdxLX = −1		(H-71)

            - predFlagLX = 0		(H-72)

    - The variable mvdLX is derived as follows:

            - mvdLX[ 0 ] = MvdLX[ xPb ][ yPb ][ 0 ]	(H-73)

            - mvdLX[ 1 ] = MvdLX[ xPb ][ yPb ][ 1 ]	(H-74)

    - When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause 8.5.3.2.5 is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the variables nPbW, nPbH, refIdxLX, and the partition index partIdx as inputs, and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as follows:

            - uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 216 ) % 216	(H-75)

            - mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(H-76)

            - uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 216 ) % 216	(H-77)

            - mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(H-78)

NOTE – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1, inclusive.

When ChromaArrayType is not equal to 0 and predFlagLX, with X being 0 or 1, is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.3.2.9 is invoked with mvLX as input, and the output being mvCLX.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xPb.. ( xPb + nPbW − 1 ), y = yPb..( yPb + nPbH− 1 ) (with X being either 0 or 1):

  - IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(H-79)
VspModeFlag[ x ][ y ] = vspModeFlag		(H-80)

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xPb ][ yPb ] is equal to 1, where ( xPb, yPb ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

- the flag vspModeFlag, specifying, whether the current PU is coded using view synthesis prediction,

The location ( xOrigP, yOrigP ) and the variables nOrigPbW and nOrigPbH are derived to store the values of ( xPb, yPb ), nPbW, and nPbH as follows:

      - ( xOrigP, yOrigP ) is set equal to ( xPb, yPb )		(H-81)

      - nOrigPbW = nPbW		(H-82)

      - nOrigPbH = nPbH		(H-83)

When Log2ParMrgLevel is greater than 2 and nCbS is equal to 8, ( xPb, yPb ), nPbW, nPbH, and partIdx are modified as follows:

      - ( xPb, yPb ) = ( xCb, yCb )		(H-84)

      - nPbW = nCbS		(H-85)

      - nPbH = nCbS		(H-86)

      - partIdx = 0		(H-87)

NOTE – When Log2ParMrgLevel is greater than 2 and nCbS is equal to 8, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for merging candidates from neighbouring prediction unit partitions in subclause 8.5.3.2.2 is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the partition index partIdx as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, and mvLXB2, with X being 0 or 1.

  - The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause H.8.5.3.2.7 is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col.The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

      - availableFlagCol = availableFlagL0Col		(H-88)

      - predFlagL0Col = availableFlagL0Col		(H-89)

      - predFlagL1Col = 0		(H-90)

  - When slice_type is equal to B, the derivation process for temporal luma motion vector prediction in subclause H.8.5.3.2.7 is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

      - availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(H-91)

      - predFlagL1Col = availableFlagL1Col		(H-92)

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause H.8.5.3.2.10 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, as the inputs and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXivMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively)..

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], the following applies.

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause H.8.5.3.2.13 is invoked with the luma locations ( xCb, yCb ) as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on DepthFlag, the following applies.

    - If DepthFlag is equal to 0, the variable availableFlagT is set equal to 0.

    - Otherwise ( DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause H.8.5.3.2.14 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as the inputs and the outputs are the flag availableFlagT, the prediction utilization flags predFlagL0T and predFlagL1T, the reference indices refIdxL0T and refIdxL1T, and the motion vectors mvL0T and mvL1T.

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies:

      - When the following condition is true,

        - availableFlagT = = 0  &&  availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1  &&  predFlagLXN  !=  predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  mvLXN  !=  mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  refIdxLXN  !=  refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the following applies:

      - When the following condition is true ,

        - availableFlagT  = =  0  &&  availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

        - availableFlagN  = =  1  &&  predFlagLXN  !=  predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  mvLXN  !=  mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  refIdxLXN  !=  refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW − 1 ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

      - availableFlagA1  = =  0,

      - predFlagLXA1  !=  predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXA1  !=  mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXA1  !=  refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

      - availableFlagB1  = =  0,

      - predFlagLXB1  !=  predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXB1  !=  mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXB1  !=  refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagVSP is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to VSP, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal 1 and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH ] and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagIvMCShift is equal to 1 and numMergeCand is less than 6, and one or more of the following conditions are true,

      - availableFlagIvMC  = =  0,

      - predFlagLXMC  !=  predFlagLXMCShift  (with X being replaced by 0 and 1),

      - mvLXMC  !=  mvLXIvMCShift (with X being replaced by 0 and 1),

      - refIdxLXMC  !=  refIdxLXMCShift (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvMCShift, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - A variable availableFlagIvDCShift is set to 0 and when all of the following conditions are true

      - DepthFlag is equal to 0, [Ed (CY): this condition is already assumed thus could be removed.]

      - availableFlagIvMCShift is equal to 0,

      - numMergeCand is less than 6,

the derivation process for the shifted disparity merging candidate as specified in subclause H.8.5.3.2.15 is invoked with the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in mergeCandList, mergeCandList, mergeCandIsVspFlag, and numMergeCand as the inputs and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift. When availableFlagIvDCShift is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvDCShift, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - The variable numOrigMergeCand is set equal to numMergeCand.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause H.8.5.3.2.3 is invoked with mergeCandList, mergeCandIsVspFlag, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand, and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk, and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + k ] is set equal to 0.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.3.2.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + numCombMergeCand + m ] is set equal to 0.

  - The following assignments are made with N being the candidate at position merge_idx[ xOrigP ][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(H-93)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(H-94)

      - refIdxLX = refIdxLXN		(H-95)

      - predFlagLX = predFlagLXN		(H-96)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies

      - refIdxL1 = −1		(H-97)

      - predFlagL1 = 0		(H-98)

  - The variable vspModeFlag is set equal to mergeCandIsVspFlag[ merge_idx[ xPb][ yPb ] ]

  - When vspModeFlag is equal to 1, the following applies:

    - The variables xVsp and yVsp are derived depending on the value of mergeCandList[ merge_idx[ xPb][ yPb ] ] as specified in Table H-9.

    - The variable vspRefViewIdx is set equal RefViewIdx[ xVsp ][ yVsp ] and the variable vspMvDisp is set equal to MvDisp[ xVsp ][ yVsp ].

    - For x in the range of xPb to ( xPb + nPbW − 1 ), inclusive, the following applies:

      - For y in the range of yPb to ( yPb + nPbH − 1 ), inclusive, the following applies:

        - RefViewIdx[ x ][ y ] = vspRefViewIdx		(H-99)

        - MvDisp[ x ][ y ] = vspMvDisp		(H-100)

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If one of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - predFlagLXIvMC  = =  1  &&  mergeCandList[ merge_idx[ xPb][ yPb ] ]  =  =  IvMC

      - predFlagLXIvMCShift  = =  1  &&  and mergeCandList[ merge_idx[ xPb][ yPb ] ]  = =  IvMCShift

[Ed. (GT): PredMode[ xCb ][ yCb ]  = =  MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

Table H-9 – Specification of the xVsp and yVsp depending on N = mergeCandList[ merge_idx[ xPb][ yPb ] ]

| N | A1 | B1 | B0 | VSP | A0 | B2 |
| --- | --- | --- | --- | --- | --- | --- |
| xVsp | xPb − 1 | xPb + nPbW − 1 | xPb + nPbW | xPb | xPb − 1 | xPb − 1 |
| yVsp | yPb + nPbH − 1 | yPb − 1 | yPb − 1 | yPb | yPb + nPbH | yPb − 1 |

        - Derivation process for spatial merge candidates

The specifications in subclause 8.5.2.1.2 apply, with the following changes:

The following paragraph

- availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

is removed.

        - Derivation process for combined bi-predictive merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- a list mergeCandIsVspFlag,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk added into mergeCandList during the invokation of this process,

- the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk added into mergeCandList during the invokation of this process,

- the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk added into mergeCandList during the invokation of this process.

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCand, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable combIdx is set equal to 0, the variable combStop is set equal to FALSE, and the following steps are repeated until combStop is equal to TRUE:

  - The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-6.

  - The following assignments are made, with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList:

    - l0Cand = mergeCandList[ l0CandIdx ]

    - l1Cand = mergeCandList[ l1CandIdx ]

  - When all of the following conditions are true:

    - mergeCandIsVspFlag[ l0CandIdx ] = = 0,

    - mergeCandIsVspFlag[ l1CandIdx ] = = 0,

    - predFlagL0l0Cand  = =  1

    - predFlagL1l1Cand  = =  1

    - ( DiffPicOrderCnt( RefPicList0[ refIdxL0l0Cand ], RefPicList1[ refIdxL1l1Cand ] )  !=  0 )  | |  
( mvL0l0Cand  !=  mvL1l1Cand )

the candidate combCandk with k equal to ( numCurrMergeCand − numInputMergeCand ) is added at the end of mergeCandList, i.e. mergeCandList[ numCurrMergeCand ] is set equal to combCandk, and the reference indices, the prediction list utilization flags, and the motion vectors of combCandk are derived as follows and numCurrMergeCand is incremented by 1:

      - refIdxL0combCandk = refIdxL0l0Cand		(H-101)

      - refIdxL1combCandk = refIdxL1l1Cand		(H-102)

      - predFlagL0combCandk = 1		(H-103)

      - predFlagL1combCandk = 1		(H-104)

      - mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]		(H-105)

      - mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]		(H-106)

      - mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]		(H-107)

      - mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]		(H-108)

      - numCurrMergeCand = numCurrMergeCand + 1	(H-109)

  - The variable combIdx is incremented by 1.

  - When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numCurrMergeCand is equal to MaxNumMergeCand, combStop is set equal to TRUE.

        - Derivation process for zero motion vector merging candidates

The specifications in subclause 8.5.3.2.4 apply.

        - Derivation process for luma motion vector prediction

The specifications in subclause 8.5.3.2.5 apply.

        - Derivation process for motion vector predictor candidates

The specifications in subclause 8.5.3.1.6 apply.

        - Derivation process for temporal luma motion vector prediction

The specifications in subclause 8.5.3.2.7 apply, with the following modifications:

- All invocations of the process specified in subclause 8.5.3.2.8 are replaced with invocations of the process specified in subclause H.8.5.3.2.8.

        - Derivation process for collocated motion vectors

Inputs to this process are:

- a variable currPb specifying the current prediction block,

- a variable colPic specifying the collocated picture,

- a variable colPb specifying the collocated prediction block inside the collocated picture specified by colPic,

- a luma location ( xColPb, yColPb ) specifying the top-left sample of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by colPic,

- a reference index refIdxLX, with X being 0 or 1.

Outputs of this process are:

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagLXCol[ x ][ y ], mvLXCol[ x ][ y ], and refIdxLXCol[ x ][ y ] are set equal to the corresponding arrays of the collocated picture specified by colPic, PredFlagLX[ x ][ y ], MvLX[ x ][ y ], and RefIdxLX[ x ][ y ], respectively, with X being the value of X this process is invoked for.

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colPb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows:

  - If predFlagL0Col[ xColPb ][ yColPb ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL1Col[ xColPb ][ yColPb ], refIdxL1Col[ xColPb ][ yColPb ], and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL0Col[ xColPb ][ yColPb ], refIdxL0Col[ xColPb ][ yColPb ], and L0, respectively.

  - Otherwise (predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 1), the following assignments are made:

    - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColPb ][ yColPb ], refIdxLXCol[ xColPb ][ yColPb ] and LX, respectively.

    - Otherwise, mvCol, refIdxCol, and listCol are set equal to mvLNCol[ xColPb ][ yColPb ], refIdxLNCol[ xColPb ][ yColPb ], and LN, respectively, with N being the value of collocated_from_l0_flag.

and mvLXCol and availableFlagLXCol are derived as follows:

    - The variables curIvFlag and colIvFlag, specifying whether inter-view prediction is utilized for the current and collocated PU are derived as:

        - curIvFlag = LongTermRefPic( currPic, currPb, refIdxLX, LX )	(H-110)

        - colIvFlag = LongTermRefPic( colPic, colPb, refIdxCol, listCol )	(H-111)

    - When MvHevcCompatibilityFlag is equal to 0, curIvFlag is not equal to colIvFlag, and AltRefIdxLX is not equal to −1, the variables refIdxCol and colIvFlag are modified as follows:

        - refIdxLX = AltRefIdxLX		(H-112)

- curIvFlag = LongTermRefPic( currPic, currPb, refIdxLX, LX )	(H-113)

    - The motion vector mvLXCol is modified as follows.

    - If curIvFlag is not equal to colIvFlag, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

    - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing prediction block currPb in the picture colPic, and the following applies:

        - The variables colDiff and currDiff specifying a POC or ViewId difference are derived as follows.

          - If curIvFlag is equal to 0 or ( ( ViewIdx  != 0  )  &&  iv_mv_scaling_flag ) is equal to 0, the following applies.

              - colDiff = DiffPicOrderCnt( colPic, refPicListCol[ refIdxCol ] )	(H-114)

              - currDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(H-115)

          - Otherwise, ( curIvFlag is equal to 1 and ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 1), the following applies.

              - colDiff = DiffViewId( colPic, refPicListCol[ refIdxCol ] )	(H-116)

              - currDiff = DiffViewId( currPic, RefPicListX[ refIdxLX ] )	(H-117)

        - If colDiff is equal to currDiff, mvLXCol is derived as follows:

            - mvLXCol = mvCol		(H-118)

        - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows:

            - tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(H-119)

            - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(H-120)

            - mvLXCol = Clip3( −32768, 32767, Sign( distScaleFactor * mvCol ) * 
	( ( Abs( distScaleFactor * mvCol ) + 127 )  >>  8 ) )	(H-121)

where td and tb are derived as follows:

            - td = Clip3( −128, 127, colDiff )		(H-122)

            - tb = Clip3( −128, 127, currDiff )		(H-123)

        - Derivation process for chroma motion vectors

The specifications in subclause 8.5.2.1.9 apply.

        - Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC, predFlagLXIvMCShift and predFlagLXIvDC,

- the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC,

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies;

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause H.8.5.3.2.11 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ] and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(H-124)

      - predFlagL0IvMC = availableFlagL0IvMC		(H-125)

      - predFlagL1IvMC = availableFlagL1IvMC		(H-126)

The shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause H.8.5.3.2.11 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X , the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ), and the reference index refIdxLX being equal to −1, and as the inputs and the outputs are the flag availableFlagLXIvMCShift, the motion vector mvLXIvMCShift and the reference index refIdxLXIvMCShift.

  - The availability flag availableFlagIvMCShift, and the prediction utilization flags predFlagL0IvMCShift and predFlagL1IvMCShift are derived by

      - availableFlagIvMCShift = availableFlagL0IvMCShift | | availableFlagL1IvMCShift	(H-127)

      - predFlagL0IvMCShift = availableFlagL0IvMCShift	(H-128)

      - predFlagL1IvMCShift = availableFlagL1IvMCShift	(H-129)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a disparity inter-view motion vector candidate as specified in subclause H.8.5.3.2.12 is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ], and the prediction list indication X, as the inputs and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(H-130)

      - predFlagL0IvDC = availableFlagL0IvDC		(H-131)

      - predFlagL1IvDC = availableFlagL1IvDC		(H-132)

        - Derivation process for a temporal inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The reference layer luma location ( xRef, yRef ) is derived by

  - xRefFull = xPb + ( ( nPbW – 1 )  >>  1 ) + ( ( mvDisp[ 0 ] + 2 )  >>  2 )	(H-133)

  - yRefFull = yPb + ( ( nPbH – 1 )  >>  1 ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 )	(H-134)

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, (  xRefFull  >>  3 )  <<  3 )	(H-135)

  - yRef = Clip3( 0, PicHeightInSamplesL – 1, (  yRefFull  >>  3 )  <<  3 )	(H-136)

The variable ivRefPic is set equal to the picture with ViewIdx equal to refViewIdx in the current access unit.

The variable ivRefPb specifies the luma prediction block covering the location given by ( xRef, yRef ) inside the inter-view reference picture specified by ivRefPic.

The luma location ( xIvRefPb, yIvRefPb ) is set equal to the top-left sample of the inter-view reference luma prediction block specified by ivRefPb relative to the top-left luma sample of the inter-view reference picture specified by ivRefPic.

When ivRefPb is not coded in an intra prediction mode, the following applies, for Y in the range of X to (1 – X), inclusive:

- The variables refPicListLYIvRef, predFlagLYIvRef[ x ][ y ], mvLYIvRef[ x ][ y ], and refIdxLYIvRef[ x ][ y ] are set equal to the corresponding variables of the inter-view reference picture specified by ivRefPic, RefPicListLY,, PredFlagLY[ x ][ y ], MvLY[ x ][ y ], and RefIdxLY[ x ][ y ], respectively.

- When predFlagLYIvRef[ xIvRefPb ][ yIvRefPb ] is equal to 1, the following applies for each i from 0 to num_ref_idx_lX_active_minus1, inclusive:

  - When PicOrderCnt( refPicListLYIvRef[ refIdxLYIvRef[ xIvRefPb ][ yIvRefPb ] ]) is equal to PicOrderCnt( RefPicListLX[ i ] ) and availableFlagLXInterView is equal to 0, the following applies.

      - availableFlagLXInterView = 1 		(H-137)

      - mvLXInterView = mvLYIvRef[ xIvRefPb ][ yIvRefPb ]	(H-138)

      - refIdxLX = i		(H-139)

        - Derivation process for a disparity inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a  reference view index refViewIdx,

- a disparity vector mvDisp,

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the disparity inter-view motion vector candidate is available,

- a disparity inter-view motion vector candidate mvLXInterView,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

For each i from 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

  - When PicOrderCnt( RefPicListX[ i ] ) is equal to the PicOrderCntVal, ViewIdx( RefPicListX[ i ] ) is equal to refViewIdx and availableFlagLXInterView is equal to 0 the following applies:

        - availableFlagLXInterView = 1 		(H-140)

        - mvLXInterView[ 0 ] = mvDisp[ 0 ]		(H-141)

        - mvLXInterView[ 1 ] = 0		(H-142)

        - refIdxLX = i		(H-143)

        - Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

- The variable availableFlagVSP is set equal to 1, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, the variables refIdxL0VSP and refIdxL1VSP are set equal to −1 and the variable refViewAvailableFlag is set equal to 0.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX – 1, inclusive, the following applies:

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xCb ][ yCb ], the following applies:

        - refViewAvailableFlag = 1		(H-144)

        - predFlagLXVSP = 1		(H-145)

        - mvLXVSP = MvDisp[ xCb ][ yCb ]		(H-146)

        - refIdxLXVSP = i		(H-147)

        - Y = 1 – X		(H-148)

When the current slice is a B slice and refViewAvailableFlag is equal to 1, refViewAvailableFlag is set equal to 0 and the following applies:

  - For i in the range of 0 to NumRefPicsLY – 1, inclusive, the following applies.

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListY[ i ] ) is not equal to RefViewIdx[ xCb ][ yCb ] and ViewIdx( RefPicListY[ i ] ) is not equal to ViewIdx, the following applies:

      - The variables refViewAvailableFlag, predFlagLYVSP, mvLYVSP, and refIdxLYVSP are derived as specified in the following:

        - refViewAvailableFlag = 1		(H-149)

        - predFlagLYVSP = 1		(H-150)

        - mvLYVSP =  MvDisp[ xCb ][ yCb ]		(H-151)

        - refIdxLYVSP = i		(H-152)

      - When iv_mv_scaling_flag is equal to 1, mvLYVSP is modified as specified in the following:

        - td = Clip3( −128, 127, ViewId – view_id_val[ RefViewIdx[ xCb ][ yCb ] ) ]	(H-153)

        - tb = Clip3( −128, 127, ViewId – ViewId( RefPicListY[ i ] ) )	(H-154)

        - tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(H-155)

        - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(H-156)

- mvLYVSP[ 0 ] = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLYVSP[ 0 ] ) 
	* ( (Abs( distScaleFactor * mvLYVSP[ 0 ] ) + 127 )  >>  8 ) )	(H-157)

        - mvLYVSP[ 1 ] = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLYVSP[ 1 ] ) 
	* ( (Abs( distScaleFactor * mvLYVSP[ 1 ] ) + 127 )  >>  8 ) )	(H-158)

        - Derivation process for the texture merging candidate

This process is not invoked when DepthFlag is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

- a flag availableFlagT specifying whether the texture merging candidate is available,

- the prediction utilization flags predFlagL0T and predFlagL1T,

- the reference indices refIdxL0T and refIdxL1T (when availableFlagT is equal to 1),

- the motion vectors mvL0T and mvL1T (when  availableFlagT is equal to 1).

The variable availableFlagT is set equal to 0. The variables predFlagL0T and predFlagL1T are set equal to 0. The variables refIdxL0T and refIdxL1T are set equal to −1. Both components of the motion vectors mvL0T and mvL1T are set equal to 0.

The texture luma location ( xRef, yRef ) is derived by:

  - xRefFull = xPb + ( ( nPbW – 1 )  >>  1 )		(H-159)

  - yRefFull = yPb + ( ( nPbH – 1 )  >>  1 ) 		(H-160)

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, ( xRefFull  >>  3 )  <<  3 )	(H-161)

  - yRef = Clip3( 0, PicHeightInSamplesL – 1,( yRefFull  >>  3 )  <<  3 )	(H-162)

  - [ Ed. (GT): Is clipping necessary? ]

Let textPic be the picture with PicOrderCntVal and ViewIdx equal to PicOrderCnt and ViewIdx of the current picture and DepthFlag being equal to 0 and let textPU the PU at covering the position ( xRef, yRef ) in textPic.

For X in the range of 0 to 1, inclusive, the following applies:

  - The variable textPredFlagLX is set equal to PredFlagLX of textPic. The variable textRefIdxLX is set equal to RefIdxLX of textPic. The variable textMvLX is set equal to the MvLX of textPic. The variable availableFlag is set equal to 0.

  - When X is equal to 0 or the current slice is a B slice, for i in the range of 0 to NumRefPicsLX − 1, inclusive, the following applies:

    - When all of the following conditions are true, availableFlag is set equal to 1,

      - textPredFlagLX[ xRef ][ yRef ] is equal to 1

      - PicOrderCnt( RefPicListX[ i ] ) is equal to PicOrderCnt( textPic )

      - ViewIdx( RefPicListX[ i ] ) is equal to ViewIdx( textPic )

    - When predFlagLXT is equal to 0 and availableFlag is equal to 1, the following applies:

        - mvLXT[ 0 ] = ( textMvLX[ xRef ][ yRef ][ 0 ] + 2 )  >>  2	(H-163)

        - mvLXT[ 1 ] = ( textMvLX[ xRef ][ yRef ][ 1 ] + 2 )  >>  2	(H-164)

        - refIdxLX = i		(H-165)

        - predFlagLXT = 1		(H-166)

        - availableFlagT = 1		(H-167)

        - Derivation process for the shifted disparity merging candidate

This process is not invoked when DepthFlag is equal to 1.

Inputs to this process are:

- the availability flags availableFlagN,

- the reference indices refIdxL0N and refIdxL1N,

- the prediction list utilization flags predFlagL0N and predFlagL1N,

- the motion vectors mvL0N and mvL1N,

- a merging candidate list mergeCandList,

- a list mergeCandIsVspFlag

- the variable numMergeCand specifying the number of merge candidates in list mergeCandList

Outputs of this process are:

- the flag availableFlagIvDCShift, specifying whether shifted disparity merging candidate is available

- the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift,

- the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift,

- the motion vectors mvL0IvDCShift and mvL1IvDCShift.

- The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to numMergeCand - 1, inclusive, the following applies:

  - The variable N is set equal to mergeCandList[ i ].

  - When availableFlagIvDCShift is equal to 0 and availableFlagN is equal to 1, the candidate N is not equal to IvMC or IvDC, and mergeCandIsVspFlag[ i ] is not equal to 0, predFlagL0N is equal to 1 and ViewIdx( RefPicList0[ refIdxL0N ] ) is not equal to ViewIdx, the following applies:

    - availableFlagIvDCShift is set equal to 1

    - predFlagLXIvDCShift is set equal to predFlagLXN, ( with X being replaced by 0 and 1 )

    - refIdxLXIvDCShift is set equal to refIdxLXN, ( with X being replaced by 0 and 1 )

    - mvL0IvDCShift[ 0 ] is set equal to mvL0N[ 0 ] + 4

    - mvL0IvDCShift[ 1 ] is set equal to ( view_synthesis_pred_flag[ nuh_layer_id ] ? 0 : mvL0N[ 1 ] )

    - mvL1IvDCShift = mvL1N

When availableFlagIvDCShift is equal to 0 and availableFlagIvDC is equal to 1, availableFlagIvDCShift is set to 1 and the following applies for X being 0 to 1, inclusive:

- predFlagLXIvDCShift is set equal to predFlagLXIvDC,

- refIdxLXIvDCShift is set equal to refIdxLXIvDC,

- mvLXIvDCShift[ 0 ] is set equal to mvL0IvDC[ 0 ] + 4

- mvLXIvDCShift[ 1 ] is set equal to mvL0IvDC[ 1 ]

      - Decoding process for inter prediction samples

        - General

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCbS specifying the size of the current luma coding block,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- the luma motion vectors mvL0 and mvL1,

- the chroma motion vectors mvCL0 and mvCL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

- an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

- an (nCbSC)x(nCbSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSC is derived as specified below,

- an (nCbSC)x(nCbSC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSC is derived as specified below.

- The variable nCbSL is set equal to nCbS and the variable nCbSC is set equal to nCbS  >>  1.

- If VspModeFlag[ xCb + xBl ][ yCb + yBl ] is equal to 0, the following ordered steps apply:

  - Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and predSampleL0Cb, predSampleL1Cb, predSampleL0Cr, and predSampleL1Cr be (nPbW / 2)x(nPbH / 2) arrays of predicted chroma sample values.

  - For X being each of 0 and 1, when predFlagLX is equal to 1, the following applies:

    - When predFlagLX is equal to 1, the following applies.

      - The variable resPredFlag is derived as specified in the following:

        - resPredFlag = 	( iv_res_pred_weight_idx  !=  0 ) 	
	( PicOrderCnt( RefPicListX[ refIdxLX ] )  !=  PicOrderCntVal ) 	(H-168)

      - If resPredFlag is equal to 1, the bilinear sample interpolation and residual prediction process as specified in subclause H.8.5.3.3.7 is invoked with the luma locations ( xCb, yCb ), ( xBl, yBl ), the size of the current luma coding block nCbS, the width and the height of the current luma prediction block nPbW, nPbH, the prediction list indication X, the prediction list utilization flag predFlagLX, the reference index refIdxLX, and the motion vectors mvLX, mvCLX, as the inputs and the outputs are the arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr.

      - Otherwise, ( resPredFlag is equal to 0 ), the following applies:

        - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.3.3.2 with refIdxLX as input.

        - If DepthFlag is equal to 0, the arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in subclause 8.5.3.3.3 with the luma locations ( xCb, yCb ) and ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH, the motion vectors mvLX and mvCLX, and the reference arrays refPicLXL, refPicLXCb, and refPicLXCr as inputs.

        - Otherwise, (DepthFlag is equal to 1), arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the full sample interpolation process specified in subclause H.8.5.3.3.5 with the luma locations ( xCb, yCb ), ( xBl, yBl ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr given as input.

  - Depending on ic_flag, the array predSamplesL is derived as specified in the following:

    - If ic_flag is equal to 0, the following applies.

      - The array predSampleL of the prediction samples of luma component is derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the luma prediction block width nPbW, the luma prediction block height nPbH, and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 0 as inputs.

    - Otherwise ( ic_flag is equal to 1), the following applies.

      - The array predSampleL of the prediction samples of luma component is derived by invoking the illumination compensated sample prediction process specified in subclause H.8.5.3.3.6, with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, the luma location ( xBl, yBl ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvL0, mvL1 and cIdx equal to 0 given as input.

  - Depending on ic_flag and nPbW, the arrays predSampleCb, and predSampleCr are derived as specified in the following:

    - If ic_flag is equal to 0 or nPbW is not greater than 8, the following applies:

      - The array predSampleCb of the prediction samples of component Cb is derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the chroma prediction block width nPbWCb set equal to nPbW / 2, the chroma prediction block height nPbHCb set equal to nPbH / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 as inputs.

      - The array predSampleCr of the prediction samples of component Cr is derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the chroma prediction block width nPbWCr set equal to nPbW / 2, the chroma prediction block height nPbHCr set equal to nPbH / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 as inputs.

    - Otherwise ( ic_flag is equal to 1 and nPbW is greater than 8), the following applies:

      - The array predSampleCb of the prediction samples of component Cb is derived by invoking the illumination compensated sample prediction process specified in subclause H.8.5.3.3.6, with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, with the chroma location ( xBl/2, yBl/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW / 2, nPbHCb set equal to nPbH / 2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 1 given as input.

      - The array predSampleCr of the prediction samples of component Cr is derived by invoking the illumination compensated sample prediction process specified in subclause H.8.5.3.3.6, with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, with the chroma location ( xBl / 2, yBl / 2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW / 2, nPbHCr set equal to nPbH / 2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 2 given as input.

- Otherwise, ( VspModeFlag[ xCb + xBl ][ yCb + yBl ] is equal to 1 ), the following applies:

  - For X in the range of 0 to 1, inclusive, the following applies.

    - When predFlagLX is equal to 1, the arrays predSamplesL ,predSampleCb, and predSampleCr are derived by invoking the view synthesis prediction process as specified in subclause H.8.5.3.3.8, with the luma locations ( xCb, yCb ), ( xBl, yBl ), the width and the height of the current luma prediction block nPbW, nPbH, the prediction list indicator X and the reference index refIdxLX as the inputs and the outputs are the sample arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr.

  - The array predSampleL of the prediction samples of luma component is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the luma location ( xBl, yBl ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 0 given as input.

  - The array predSampleCb of the prediction samples of component Cb is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xBl/2, yBl/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW / 2, nPbHCb set equal to nPbH / 2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 given as input.

  - The array predSampleCr of the prediction samples of component Cr is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xBl / 2, yBl / 2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW / 2, nPbHCr set equal to nPbH / 2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 given as input.

        - Reference picture selection process

The specifications in subclause 8.5.3.3.2 apply.

        - Fractional sample interpolation process

The specifications in subclause 8.5.3.3.3 apply.

        - Weighted sample prediction process

The specifications in subclause 8.5.3.3.4 apply.

        - Full sample interpolation process

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top left sample of the current luma coding block,

- the width and height of the prediction block, nPbW and nPbH, in luma-sample units,

- a luma motion vector mvLX given in quarter-luma-sample units,

- a chroma motion vector mvCLX given in eighth-chroma-sample units,

- the reference picture sample arrays refPicLXL, refPicLXCb, and refPicLXCr.

Outputs of this process are:

- a (nPbW)x(nPbH) array predSampleLXL of prediction luma sample values,

- two (nPbW/2)x(nPbH/2) arrays predSampleLXCb, and predSampleLXCr of prediction chroma sample values.

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the given reference sample arrays is derived by

  - xP = xCb + xBl		(H-169)

  - yP = yCb + yBl		(H-170)

Let ( xIntL, yIntL ) be a luma location given in full-sample units specifying sample locations inside the reference sample arrays refPicLXL.

For each luma sample location ( xL = 0..( nPbW − 1 ), yL = 0..( nPbH−1 ) ) inside the prediction luma sample array predSampleLXL, the corresponding prediction luma sample value predSampleLXL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, are derived as specified in the following:

  - xIntL = xP + mvLX[ 0 ] + xL		(H-171)

  - yIntL = yP + mvLX[ 1 ] + yL		(H-172)

- The prediction luma sample value predSampleLXL[ xL ][ yL ] is derived as specified in the following:

  - predSampleLXL[ xL ][ yL ] = refPicLXL[ xIntL ][ yIntL ]		(H-173)

For each chroma sample location ( xC = 0..( nPbW / 2 − 1 ), yC = 0..( nPbH / 2 −1 ) ) inside the prediction chroma sample arrays predSampleLXCb and predSampleLXCr, the corresponding prediction chroma sample values predSampleLXCb[ xC ][ yC ] and predSampleLXCr[ xC ][ yC ] are set to be equal to ( 1  <<  ( BitDepthC − 1 ) ).

[Ed. (GT): In current software and draft chroma planes are also present for depth. A general discussion is needed to specify how chroma planes are handled. (#12)]

        - Illumination compensated sample prediction process

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- the size of current luma coding block nCbS,

- a location ( xBl, yBl ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices, refIdxL0 and refIdxL1,

- motion vector mvL0 and mvL1

- colour component index, cIdx,

Outputs of this process are:

- the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

- The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

- The variable offset1 is derived as follows.

  - If shift1 is greater than 0, offset1 set equal to 1  <<  ( shift1 − 1 ).

  - Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

- The variable offset2 is set equal to 1  <<  ( shift2 − 1 ).

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause H.8.5.3.3.6.1 is invoked with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as the inputs and the outputs are the flags puIcFlagL0 and puIcFlagL1 and the variables icWeightL0 and icWeightL1 specifying weights for illumination compensation, the variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as follows:

- For X in the range of 0 to 1, inclusive, the following applies:

  - When predFlagLX is equal to 1 the following applies:

      - clipPredVal = 
Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLX[ x ][ y ] + offset1 )  >>  shift1 )	(H-174)

      - predValX = !puIcFlagLX ? clipPredVal : 
( Clip3( 0, ( 1  <<  bitDepth ) − 1, ( clipPredVal * icWeightLX )  >>  5 ) + icOffsetLX )	(H-175)

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 1, the following applies:

    - predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predVal0 + predVal1 + offset2 )  >>  shift2 )	(H-176)

- Otherwise ( predFlagL0 is equal to 0 or predFlagL1 is equal to 0 ), the following applies:

    - predSamples[ x ][ y ] = predFlagL0 ? predVal0 : predVal1	(H-177)

          - Derivation process for illumination compensation mode availability and parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top left sample of the current picture,

- the size of the current luma coding block, nCbS,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices refIdxL0 and refIdxL1,

- motion vectors mvL0 and mvL1

- a bit depth of samples, bitDepth.

- a variable cIdx specifying colour component index.

Outputs of this process are:

- flags puIcFlagL0 and puIcFlagL1 specifying whether illumination compensation is enabled.

- variables icWeightL0 and icWeightL1 specifying weights for illumination compensation

- variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation

The variables puIcFlagL0 and PuIcFlagL1 are set equal to 0, the variables icWeightL0 and icWeightL1 are set equal to 1, and the variables icOffsetL0 and icOffsetL1 are set equal to 0.

The variables nCS specifying the current luma or chroma coding block size, and the location ( xC, yC ) specifying the top left sample of the current luma or chroma coding block is derived as follows.

  - nCS = ( cIdx = = 0 ) ? nCbS : nCbS / 2 		(H-178)

  - ( xC, yC ) =  ( cIdx = = 0 ) ? ( xCb, yCb ) : ( xCb /2 , yCb / 2 ) 	(H-179)

The variable availFlagCurAboveRow specifying the availability of above neighbouring row samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xN, yN ) set equal to ( xCb, yCb − 1 ) as the input and the output is assigned to availFlagCurAboveRow.

The variable availFlagCurLeftCol specifying the availability of left neighbouring column samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xN, yN ) set equal to ( xCb − 1, yCb ) as the input and the output is assigned to availFlagCurLeftCol.

[Ed. (GT) The availability derivation is specified as performed in the software. However, the check of the availability of left and above PU might not be sufficient to guarantee the availability of the whole left column or above row. A check of availability similar to that used for intra prediction might be a better solution. ]

When availFlagCurAboveRow is equal to 0 and availFlagCurLeftCol is equal to 0 the whole derivation process of this subclause terminates.

For X being replaced by 0 and 1, when predFlagLX is equal to 1, the variable puIcFlagLX is derived by the following ordered steps.

  - The variable refPicLX specifying the reference picture from reference picture list X is set equal to RefPicListX[ refIdxLX ] .

  - If ViewIdx( refPicLX ) is not equal to ViewIdx, the variable puIvPredFlagLX specifying whether inter-view prediction from list X is utilized is set equal to 1, otherwise (predFlagLX is equal to 0 or ViewIdx( RefPicListX[ refIdxLX ] )  is equal to ViewIdx ), puIvPredFlagLX is set equal to 0.

  - If puIvPredFlagLX is equal to 0, the variable puIcFlagLX is set equal to 0, otherwise (puIvPredFlagLX is equal to 1 ) the following applies:

    - The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as

        - xRLX = xC + ( ( mvLX[ 0 ] + ( cIdx ? 4 : 2 ) )  >>  ( 2 + ( cIdx ? 1 : 0 ) ) )	(H-180)

        - yRLX = yC + ( ( mvLX[ 1 ] + ( cIdx ? 4 : 2 ) )  >>  ( 2 + ( cIdx ? 1 : 0 ) ) )	(H-181)

    - The variable availFlagAboveRowLX specifying whether the above neighbouring row samples of the current block and the reference block are available is derived as specified in the following:

        - availFlagAboveRowLX = ( yRLX > 0 )  &&  availFlagCurAboveRow 	(H-182)

    - The variable availFlagLeftColLX specifying whether the left neighbouring column samples of the current block and the reference block are available is derived as specified in the following:

        - availFlagLeftColLX = ( xRLX > 0 )  &&  availFlagCurLeftCol	(H-183)

    - The variable puIcFlagLX is derived as follows:

        - puIcFlagLX = availFlagAboveRowLX  | |  availFlagLeftColLX 	(H-184)

Depending on the colour component cIdx, the variable curRecSamples specifying the reconstructed picture samples of the current picture is derived as

    - curRecSamples = ( !cidx ) ? RecSamplesL : ( ( icdx == 1 ) ? RecSamplesCb : RecSamplesCr )	(H-185)

- [Ed. (GT). The reconstructed samples before deblocking filter RecSamplesL, RecSamplesCb and RecSamplesCr as used above although not explicitly defined. However, they should be defined in the base spec. ]

- For X being replaced by 0 and 1, when puIcFlagLX is equal to 1, the variables icWeightLX, and icOffsetLX are derived by the following ordered steps:

  - Depending on the colour component cIdx, the variable refRecSamples specifying the reconstructed picture samples of the reference picture is derived as specified in the following.

    - If cIdx is equal to 0, refRecSamples is set equal to reconstructed picture sample array SL of picture refPicLX.

    - Otherwise, if cIdx is equal to 1, refRecSamples is set equal to the reconstructed chroma sample array SCb of picture refPicLX.

    - Otherwise (cIdx is equal to 2), refRecSamples is set equal to the reconstructed chroma sample array SCr of picture refPicLX.

  - The lists curNeighSampleListLX and refNeighSampleListLX specifying the neighbouring samples in the current picture and the reference picture are derived as specified in the following.

    - The variable numNeighSamplesLX specifying the number of elements in curNeighSampleListLX and in refNeighSampleLX is set equal to 0.

    - The variable leftNeighOffLX specifying the offset of the left neighbouring samples in curNeighSampleListLX and refNeighSampleLX is derived as

        - leftNeighOffLX = availFlagAboveRowLX ? 0 : nCS 	(H-186)

    - For i ranging from 0 to nCS − 1, inclusive the following applies.

      - When availFlagAboveRowLX is equal to 1 the following applies.

          - curNeighSampleListLX[ i ] = curRecSamples[ xC + i][ yC − 1 ] 	(H-187)

          - refNeighSampleListLX[ i ] = refRecSamples[ xRLX + i ][ yRLX − 1 ]	(H-188)

          - numNeighSamplesLX  +=  1		(H-189)

      - When availFlagLeftColLX is equal to 1 the following applies

          - curNeighSampleListLX[ i + leftNeighOffLX ] = curRecSamples[ xC − 1][ yC + i ] 	(H-190)

          - refNeighSampleListLX[ i + leftNeighOffLX ] = refRecSamples[ xRLX − 1][ yRLX + i ] 	(H-191)

          - numNeighSamplesLX  +=  1		(H-192)

  - The derivation process for illumination compensation parameters as specified in subclause H.8.5.3.3.6.2 is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample list, the number of neighbouring samples numNeighSamlesLX and the size of the current luma coding block nCSl as inputs and the illumination parameters icWeightLX, and icOffsetLX as outputs.

          - Derivation process for illumination compensation parameters

Inputs to this process are:

- a list curSampleList specifying the current samples, ,

- a list refSampleList specifying the reference samples,

- a variable numSamples specifying the number of elements in curSampleList and refSampleList.

- a bit depth of samples, bitDepth.

- the size of the current luma coding block nCSl

Outputs of this process are:

- a variable icWeight specifying a weight for illumination compensation,

- a variable icOffset specifying a offset for illumination compensation,

The variable precShift is set equal to Max( 0, bitDepth − 12 ).

The variables sumRef, sumCur, sumRefSquare and sumProdRefCur are set equal to 0 and the following applies for i ranging from 0 to numSamples / 2− 1, inclusive:

  - sumRef  +=  refSampleList[ 2 * i ]		(H-193)

  - sumCur  +=  curSampleList[ 2 * i ]		(H-194)

  - sumRefSquare  +=  ( refSampleList[ 2 * i ] * refSampleList[ 2 * i ] )  >>  precShift	(H-195)

  - sumProdRefCur  +=  ( refSampleList[ 2 * i ] * curSampleList[ 2 * i ] )  >>  precShift	(H-196)

The variable avgShift and avgOffset are derived as follows:

  - avgShift = Log2( numSamples / 2)		(H-197)

  - avgOffset = 1  <<  ( avgShift − 1 )		(H-198)

The variables numerDiv and denomDiv are derived as follows:

  - denomDiv= ( ( sumRefSquare + ( sumRefSquare  >>  7 ) )  <<  avgShift )
		 – ( sumRef * sumRef )  >>  precShift	(H-199)

  - numerDiv= Clip3( 0, 2 * denomDiv, ( ( sumProdRefCur + ( sumRefSquare  >>  7 ) )  <<  avgShift )
		 – ( sumRef * sumCur )  >>  precShift )	(H-200)

The variables shiftNumer and shiftDenom are derived as follows:

  - shiftDenom = Max( 0, Floor( Log2( Abs( denomDiv ) ) ) − 5 )	(H-201)

  - shiftNumer = Max( 0, shiftDenom − 12 )		(H-202)

The variables sNumerDiv and sDenomDiv are derived as follows:

  - sDenomDiv = denomDiv  >>  shiftDenom		(H-203)

  - sNumerDiv = numerDiv  >>  shiftNumer		(H-204)

The value of variable divCoeff  is derived from Table H-10 depending on sDenomDiv and the variables icWeight, and icOffset are derived as follows:

  - icWeight = ( sNumerDiv * divCoeff )  >>  ( shiftDenom – shiftNumer + 10 )	(H-205)

- icOffset = ( sumCur – ( ( icWeight * sumRef )  >>  5 ) + avgOffset )  >>  avgShift	(H-206)

Table H-10 – Specification of divCoeff depending on sDenomDiv

| sDenomDiv | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| divCoeff | 0 | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 |
| sDenomDiv | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| divCoeff | 2521 | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 |
| sDenomDiv | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| divCoeff | 1260 | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 |
| sDenomDiv | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| divCoeff | 840 | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 |
| sDenomDiv | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |  |
| divCoeff | 630 | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 |  |

        - Bilinear sample interpolation and residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCbS specifying the size of the current luma coding block,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- the prediction list indication X,

- the prediction list utilization flag predFlagLX,

- the reference index refIdxLX,

- the motion vectors mvLX, mvCLX

Output of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

The motion vector mvLX is scaled as specified in the following:

  - tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(H-207)

  - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(H-208)

  - mvLX =	Clip3( −32768, 32767, Sign( distScaleFactor * mvLX ) * 
	( ( Abs( distScaleFactor * mvLX ) + 127 ) >> 8 ) )	(H-209)

where td and tb are derived as:

  - td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(H-210)

  - tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ 0 ] ) )	(H-211)

The location ( xP, yP )  is derived by:

  - xP = xCb + xBl		(H-212)

  - yP = yCb + yBl		(H-213)

The motion vector mvCLX is set equal to mvLX.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr  are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX equal to 0 as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause H.8.5.3.3.7.1 with the luma locations ( xCb, yCb ), ( xBl, yBl ), , the luma prediction block width nPbW, the luma prediction block height nPbH,, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. as the inputs.

The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr  are derived as specified in the following:

- Let currIvRefPic be the picture with PicOrderCnt( currIvRefPic ) equal to PicOrderCntVal and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays currIvRefPicLXL, currIvRefPicLXCb, and currIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture currIvRefPic.

- The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause H.8.5.3.3.7.1 with the luma locations ( xCb, yCb ) and ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH, the motion vectors mvLX equal to MvDisp[ xP ][ yP ] and mvCLX equal to MvDisp[ xP ][ yP ], and the, and the reference arrays with currIvRefPicLXL, currIvRefPicLXCb and currIvRefPicLXCr as the inputs.

The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr  are derived as specified in the following:

- Let refIvRefPic be the picture with PicOrderCnt( refIvRefPic ) equal to PicOrderCnt( RefPicListX[ 0 ] ) and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays refIvRefPicLXL, refIvRefPicLXCb, and refIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture refIvRefPic.

- The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause H.8.5.3.3.7.1 with the luma locations ( xCb, yCb ), ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH,, the motion vector mvLX equal to ( mvLX + MvDisp[ xP ][ yP ] ) and the motion vector mvCLX equal to ( mvCLX + MvDisp[ xP ][ yP ] ), and the reference arrays with refIvRefPicLXL, refIvRefPicLXCb and refIvRefPicLXCr as the inputs.

The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 predSamplesLXL[ x ][ y ] + 
	( ( currIvSamplesLXL[ x ][ y ] − refIvSamplesLXL[ x ][ y ] )  >>  shiftVal )	(H-214)

The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 predSamplesLXCb[ x ][ y ] + 
	( ( currIvSamplesLXCb[ x ][ y ] − refIvSamplesLXCb[ x ][ y ] )  >>  shiftVal )	(H-215)

The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

  - predSamplesLXCr[ x ][ y ] =	 predSamplesLXCr[ x ][ y ] + 
	( ( currIvSamplesLXCr[ x ][ y ] − refIvSamplesLXCr[ x ][ y ] )  >>  shiftVal )	(H-216)

          - Bilinear sample interpolation process

The specifications in subclause 8.5.3.3.3.1 apply with the following modifications:

- All invocations of the process specified in subclause 8.5.3.3.3.2 are replaced with invocations of the process specified in subclause H.8.5.3.3.7.2 with chromaFlag equal to 0 as additional input.

- All invocations of the process specified in subclause 8.5.3.3.3.3 are replaced with invocations of the process specified in subclause H.8.5.3.3.7.2 with chromaFlag equal to 1 as additional input.

          - Bilinear luma and chroma sample interpolation process

Inputs to this process are:

- a location in full-sample units ( xInt, yInt ),

- a location offset in fractional-sample units ( xFrac, yFrac),

- a sample reference array refPicLX,

- a flag chromaFlag.

- Output of this process is a predicted sample value predPartLX[ x ][ y ].

- In Figure H-1, the positions labelled with A, B, C, and D represent samples at full-sample locations inside the given two-dimensional array refPicLX of samples.

Figure H-1 Fractional sample position dependent variables in bi-linear interpolation 
and surrounding integer position samples A, B, C, and D

- The variable picWidthInSamples is set equal to pic_width_in_luma_samples and the variable picHeightInSamples is set equal to pic_height_in_luma_samples.

- If chromaFlag is equal 0, xFrac is set equal to ( xFrac << 1 ) and yFrac is set equal to ( yFrac << 1 ).

- Otherwise ( chromaFlag is equal to 1 ), picWidthInSamples is set equal to ( picWidthInSamples / SubWidthC) and picHeightInSamples is set equal to ( picHeightInSamples / SubHeightC ).

The coordinates of positions A, B, C and D are derived as follows:.

  - xA = Clip3( 0, picWidthInSamples − 1, xInt )		(H-217)

  - xB = Clip3( 0, picWidthInSamples − 1, xInt + 1 )		(H-218)

  - xC = Clip3( 0, picWidthInSamples − 1, xInt )		(H-219)

  - xD = Clip3( 0, picWidthInSamples − 1, xInt + 1 ) 		(H-220)

  - yA = Clip3( 0, picHeightInSamples − 1, yInt )		(H-221)

  - yB = Clip3( 0, picHeightInSamples − 1, yInt )		(H-222)

  - yC = Clip3( 0, picHeightInSamples − 1, yInt + 1 )	 	(H-223)

  - yD = Clip3( 0, picHeightInSamples − 1, yInt + 1 )		(H-224)

The value of predPartLX[ x ][ y ] is derived as specified in the following:

  - predPartLX[ x ][ y ] = (	refPicLX[ xA ][ yA ] * ( 8 − xFrac ) * ( 8 − yFrac ) + 
	refPicLX[ xB ][ yB ] * ( 8 − yFrac ) * xFrac + 
	refPicLX[ xC ][ yC ] * ( 8 − xFrac ) * yFrac + 
	refPicLX[ xD ][ yD ] * xFrac * yFrac ) >> 6	(H-225)

        - View synthesis prediction process

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- a location ( xBl, yBl ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- the prediction list indicator X

- the reference index refIdxLX

Outputs of this process are:

–	an array predSamplesL of luma prediction samples,

–	an array predSamplesCb of chroma prediction samples for the component Cb

–	an array predSamplesCr of chroma prediction samples for the component Cr

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the current picture is derived by:

  - xP = xCb + xBl		(H-226)

  - yP = yCb + yBl		(H-227)

The reference picture consisting of an ordered two-dimensional array refPicL of luma samples and two ordered two-dimensional arrays refPicCb and refPicCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

The variable refViewIdx is set equal to the ViewIdx( RefPicListX[ refIdxLX ] ) and the variable depthViewIdx is set equal to RefViewIdx[ xCb + xBl ][ yCb + yBl ]. The variable mvDisp is set equal to MvDisp[ xCb + xBl ][ yCb + yBl ]

The derivation process for a disparity sample array as specified in section H.8.5.5.2 is invoked with the luma location ( xP, yP ), the disparity vector mvDisp, the variable refViewIdx, the variable depthViewIdx, the variable nPSW equal to ( nPbW  = =  12 ? 16 : nPbW ), the variable nPSH equal to ( nPbH  = =  12 ? 16 : nPbH ), and the variable splitFlag equal to ( nPbW > 4  &&  nPbH > 4 ) as the inputs, and the output is the array disparitySamples of size (nPSW)x(nPSH).

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units.

For each luma sample location ( xL = 0..nPbW − 1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSamplesL, the corresponding prediction luma sample value predSamplesL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

  - xIntL = xP + xL+ disparitySamples[ xL ][ yL ]		(H-228)

  - yIntL = yP + yL		(H-229)

  - xFracL = disparitySamples[ xL ][ yL ] & 3		(H-230)

  - yFracL = 0		(H-231)

- The prediction luma sample value predSamplesL[ xL][ yL ] is derived by invoking the process specified in subclause 8.5.3.3.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicL given as input.

[Ed. (GT): As for inter prediction the treatment of colour planes for depth needs to be discussed. In software colour planes are set to 128 in VSP process. (#12)]

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-eighth sample units.

For each chroma sample location ( xC = 0..nPbW / 2−1, yC = 0..nPbH /2 −1 ) inside the prediction chroma sample arrays predSamplesCb and predSamplesCr, the corresponding prediction chroma sample values predSampleLXCb[ xC ][ yC ] and predSamplesCr[ xC ][ yC ] are derived as follows:

- The variables xIntC, yIntC, xFracC, and yFracC are derived by

  - xIntC = ( xP / 2 ) + xC + disparitySamples[ xC  <<  1 ][ yC  <<  1 ]	(H-232)

  - yIntC = ( yP / 2 ) + yC		(H-233)

  - xFracC = disparitySamples[ xC  <<  1][ yC   <<  1 ] & 7		(H-234)

  - yFracC = 0		(H-235)

- The prediction sample value predSamplesCb[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.3.3.3.3 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCb given as input.

- The prediction sample value predSamplesCr[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.3.3.3.3 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCr given as input.

    - Decoding process for the residual signal of coding units coded in inter prediction mode

      - General

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Outputs of this process are:

- an (nCbSL)x(nCbSL) array resSamplesL of luma residual samples, where nCbSL is derived as specified below,

- an (nCbSC)x(nCbSC) array resSamplesCb of chroma residual samples for the component Cb, where nCbSC is derived as specified below,

- an (nCbSC)x(nCbSC) array resSamplesCr of chroma residual samples for the component Cr, where nCbSC is derived as specified below.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to nCbSL  >>  1.

Let resSamplesL be an (nCbSL)x(nCbSL) array of luma residual samples and let resSamplesCb and resSamplesCr be two (nCbSC)x(nCbSC) arrays of chroma residual samples.

If inter_sdc_flag is equal to 0, the following applies, depending on the value of rqt_root_cbf, the following applies:

  - If rqt_root_cbf is equal to 0 or skip_flag[ xCb ][ yCb ] is equal to 1, all samples of the (nCbSL)x(nCbSL) array resSamplesL and all samples of the two (nCbSC)x(nCbSC) arrays resSamplesCb and resSamplesCr are set equal to 0.

  - Otherwise (rqt_root_cbf is equal to 1), the following ordered steps apply:

    - The decoding process for luma residual blocks as specified in subclause H.8.5.4.2 below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable nCbS set equal to nCbSL, and the (nCbSL)x(nCbSL) array resSamplesL as inputs, and the output is a modified version of the (nCbSL)x(nCbSL) array resSamplesL.

    - The decoding process for chroma residual blocks as specified in subclause H.8.5.4.3 below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 1, the variable nCbS set equal to nCbSC, and the (nCbSC)x(nCbSC) array resSamplesCb as inputs, and the output is a modified version of the (nCbSC)x(nCbSC) array resSamplesCb.

    - The decoding process for chroma residual blocks as specified in subclause H.8.5.4.3 below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 2, the variable nCbS set equal to nCbSC, and the (nCbSC)x(nCbSC) array resSamplesCr as inputs, and the output is a modified version of the (nCbSC)x(nCbSC) array resSamplesCr.

- Otherwise (inter_sdc_flag is equal to 1), the decoding process for simplified depth coded residual blocks as specified in subclause H.8.5.4.4 is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable nCbS set equal to nCbSL, and the (nCbSL)x(nCbSL) array resSamplesL as inputs, and the output is a modified version of the (nCbSL)x(nCbSL) array resSamplesL.

For x in the range of 0 to nCbSL − 1 and y in the range of 0 to nCbSL − 1, the following applies:

  - ResSamplesL[ xCb + x ][ yCb + y ] is set equal to resSamplesL[ x ][ y ].

For x in the range of 0 to nCbSC − 1 and y in the range of 0 to nCbSC − 1, the following applies:

  - ResSamplesCb[ xCb /2 + x ][ yCb /2 + x] is set equal to resSamplesCb[ x ][ y ].

  - ResSamplesCr[ xCb /2 + x ][ yCb /2 + x ] is set equal to resSamplesCr[ x ][ y ].

      - Decoding process for luma residual blocks

The specification in subclause 8.5.4.2 applies.

      - Decoding process for chroma residual blocks

The specification in subclause 8.5.4.3 applies.

      - Decoding process for simplified depth coded residual blocks

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable nCbS specifying the size of the current luma coding block,

–	a (nCbS)x(nCbS) array resSamples of luma residual samples.

Output of this process is:

–	a modified version of the (nCbS)x(nCbS) array of luma residual samples.

The values of the variables xOff, yOff, and interSdcResiIdx[ i ] for i in the range of 0 to 3, inclusive, depending on the value of PartMode are specified in Table H-11.

Table H-11 – Specification of the variables xOff, yOff, and interSdcResiIdx[ i ]

| PartMode | xOff | yOff | interSdcResiIdx[ i ] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | i = 0 | i = 1 | i = 2 | i = 3 |
| PART_2Nx2N | nCbS | nCbS | 0 | 0 | 0 | 0 |
| PART_2NxN | nCbS | ( nCbS >> 1 ) | 0 | 0 | 1 | 1 |
| PART_2NxnU | nCbS | ( nCbS >> 2 ) | 0 | 0 | 1 | 1 |
| PART_2NxnD | nCbS | ( nCbS >> 1 ) + ( nCbS >> 2 ) | 0 | 0 | 1 | 1 |
| PART_Nx2N | ( nCbS >> 1 ) | nCbS | 0 | 1 | 0 | 1 |
| PART_nLx2N | ( nCbS >> 2 ) | nCbS | 0 | 1 | 0 | 1 |
| PART_nRx2N | ( nCbS >> 1 ) + ( nCbS >> 2 ) | nCbS | 0 | 1 | 0 | 1 |
| PART_NxN | ( nCbS >> 1 ) | ( nCbS >> 1) | 0 | 1 | 2 | 3 |

For x in the range of 0 to nCbS the following applies:

- For y in the range of 0 to nCbS the following applies:

  - The variable i is derived as specified in the following:

    - If x is less than xOff and y is less than yOff, i is set equal to 0.

    - Otherwise, if x greater than or equal to xOff and y is less than to yOff, i is set equal to 1.

    - Otherwise, if x less than xOff and y is greater than or equal to yOff, i is set equal to 2.

    - Otherwise, ( x is greater than or equal to xOff and y is greater than or equal to yOff), i is set equal to 3.

  - The value of resSamples[ x ][ y ] is set equal to InterSdcResi[ xCb ][ yCb ][ interSdcResiIdx[ i ] ]

    - Derivation process for disparity vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows:

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - PredMode[ xCb ][ yCb ] is equal to MODE_SKIP.

  - PredMode[ xCb ][ yCb ] is equal to MODE_INTER and merge_flag[ xCb ][ yCb ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

The derivation process for a disparity vector from temporal neighbour block as specified in subclause H.8.5.5.1 is invoked with the luma location ( xCb, yCb ), and the variable nCbS as inputs, and the outputs are the flag availableDV, the disparity vector mvDisp and the reference view order index refViewIdx.

When availableDV is equal to 0, for each N being A1, B1 and ( xN, yN ) being ( xCb − 1,  yCb + nCbS − 1 ), ( xCb + nCbS − 1,  yCb − 1 ), respectively, the following ordered steps apply.

  - When yCb − 1 is less than ( ( yCb  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ), the following applies.

        - xB1 = ( ( xB1  >>  3 )  <<  3 ) + ( ( xB1  >>  3 ) & 1) * 7	(H-236)

  - The derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to the ( xCb, yCb ) and the luma location ( xN, yN ) as the input and the output assigned to availableN.

  - When availableN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_INTRA, availableN is set equal to 0. [Ed. (GT): 2+3 correspond to 6.4.2 for CU when ( xN, yN ) outside CU. Cross-check appreciated. ]

  - When all of the following conditions are true, availableN is set equal to 0.

    - checkParallelMergeFlag is equal to 1

    - ( xCb  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to
( xN  >>  ( log2_parallel_merge_level_minus2 + 2) )

    - ( yCb  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to ( yN  >>  ( log2_parallel_merge_level_minus2 + 2) ).

  - The flag availableIvpMvSearchFlagN is set equal to availableN.

  - When one of the following conditions is true, N is equal to B1 and ( ( yN  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ) is less than ( ( yCb >> Log2CtbSizeY )  <<  Log2CtbSizeY), availableIvpMvSearchFlagN is set equal to 0.

  - The flag availableFlagIvpMvN is set equal to 0.

  - For each X from 0 to 1, the following applies:

    - When availableDV is equal to 0, availableN is equal to 1, RefIdxLX[ xN ][ yN ] is greater than or equal to 0, and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture of the current picture, the following applies:

          - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(H-237)

          - mvDisp = MvLXN[ xN ][ yN ]		(H-238)

          - availableDV = 1		(H-239)

      - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

        - When availableIvpMvSearchFlagN is equal to 1, availableFlagIvpMvN is equal to 0, and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

            - ivpMvDispN = MvRefinedDisp[ xN ][ yN ]	(H-240)

            - refViewIdxN = RefViewIdx[ xN ][ yN ]	(H-241)

            - availableFlagIvpMvN = 1		(H-242)

When availableDV is equal to 0 for each N being A1 and B1, the following applies.

  - When availableDV is equal to 0 and availableFlagIvpMvN is equal to 1, the following applies:

      - mvDisp = ivpMvDispN		(H-243)

      - refViewIdx = refViewIdxN		(H-244)

      - availableDV = 1		(H-245)

When availableDV is equal to 0, refViewIdx is set equal to 0, and mvDisp is set equal to ( 0, 0 ). The variable mvRefinedDisp is set equal to mvDisp.

[Ed. (GT): It is currently an issue under non-CTC that a view with refViewIdx equal to 0 might not be available in the reference picture list. In software a preliminary fix has been incorporated, to enable simulations in IBP configuration. ]

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - The derivation process for a disparity sample array as specified in subclause H.8.5.5.2 is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS,  and the variable splitFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

  - MvDisp[ x ][ y ] = mvDisp		(H-246)

  - MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(H-247)

  - RefViewIdx[ x ][ y ] = refViewIdx		(H-248)

  - DefaultDispFlag[ x ][ y ] = !availableDV		(H-249)

      - Derivation process for a disparity vector from temporal neighbour blocks

Inputs to this process are

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

Outputs of this process are

- the disparity vector mvDisp,

- the reference view order index refViewIdx,

- the availability flag availableFlag.

The luma location ( xCCtr , yCCtr) specifying the centre position of the current luma coding block is derived as follows:

  - xCCtr = xCb + ( nCbS  >>  1 )		(H-250)

  - yCCtr = yCb + ( nCbS  >>  1 )		(H-251)

The flag availableFlag is set equal to 0, and mvDisp is set equal to ( 0, 0 ).

For i from 0 to NumDdvCandPics − 1, inclusive, the following ordered steps apply and the whole decoding process of this sub-clause terminates once availableFlag is set to 1.

  - Let colPu the prediction unit in DdvCandPicsList[ i ] covering the position ( ( xCCtr  >>  4 )  <<  4 ,  ( yCCtr  >>  4 )  <<  4 ).

  - The position ( xPCol, yPCol ) is set equal to the position of the top-left sample of colPu relative to the top-left luma sample of the DdvCandPicsList[ i ].

  - If slice_type is equal to B, the variable dir is set equal to collocated_from_l0_flag, otherwise, dir is set equal to 1 – collocated_from_l0_flag. [Ed. (GT) In software L0 is always checked first. Moreover the number of checks depends on the slice_type the of collocated slice. ]

  - For each X from dir to 1 – dir, inclusive, the following applies:

    - The variables candPicRefPicList, candPredFlag, candRefIdx, and candMV are set equal to the variables RefPicListX, RefIdxLX, and MvLX of DdvCandPicsList[ i ], respectively.

    - When colPu is not coded in an intra prediction mode and candPredFlag[ xPCol ][ yPCol ] is equal to 1, the following applies:

      - The variable candRefViewIdx is set equal to the ViewIdx( candPicRefPicList[ candRefIdx[ xPCol ][ yPCol ] ] ).

      - When candRefViewIdx is not equal to the ViewIdx( DdvCandPicsList[ i ] ) and there is an inter-view reference picture with ViewIdx equal to candViewIdx in RefPicList0 or RefPicList1, the following applies:

          - refViewIdx = candRefViewIdx		(H-252)

          - mvDisp = candMV[ xPCol ][ yPCol ]		(H-253)

          - availableFlag = 1		(H-254)

      - Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPic the picture in the current access unit with ViewIdx( refDepPic ) equal to ViewIdx and DepthFlag( refDepPic ) equal to 1.

Let refDepPels be an array of reconstructed depth samples refDepPic. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = xP + ( ( mvDisp[ 0 ] + 2 )  >>  2 )		(H-255)

  - yTL = yP + ( ( mvDisp[ 1 ] + 2 )  >>  2 )		(H-256)

- The variables nSubBlkW and nSubBlkH are derived as specified in the following:

  - nSubBlkW = splitFlag ? 8 : nPSW		(H-257)

  - nSubBlkH = splitFlag ? 8 : nPSH		(H-258)

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) – 1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW) – 1 ), inclusive, the following applies:

    - The variables xB, yB, xP0, yP0, xP1, yP1, are derived as specified in the following:

xB = sBx * nSubBlkW
			yB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB )
			yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB )
			xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB + nSubBlkW – 1 )
			yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB + nSubBlkH – 1 )

    - The variable nSubSubBlkW is set equal to nSubBlkW and the variable nSubSubBlkH is set equal to nSubBlkH.

    - When splitFlag is equal to 1, nSubSubBlkW and nSubSubBlkW are modified as follows:

      - The variable horSplitFlag is derived as specified in the following:

        - horSplitFlag = ( refDepPels[ xP0 ][ yP0 ] > refDepPels[ xP1 ][ yP1 ] )
	  = =  ( refDepPels[ xP1 ][ yP0 ] > refDepPels[ xP0 ][ yP1] ) )	(H-259)

        - nSubSubBlkW = horSplitFlag ? nSubSubBlkW : ( nSubSubBlkW  >>  1 )		(H-260)

        - nSubSubBlkH = horSplitFlag ? ( nSubSubBlkH  >>  1 ) : nSubSubBlkW		(H-261)

    - The derivation process for a disparity sample block as specified in subclause H.8.5.5.2.1 is invoked with the luma location ( xB, yB ), variables nSubBlkW and nSubBlkH, the array of reconstructed depth samples refDepPels, the luma location ( xTL, yTL ), the variables nSubSubBlkW and nSubSubBlkW, the view order index refViewIdx, and the array disparitySamples as the inputs, and the output is the modified array disparitySamples.

        - Derivation process for a disparity sample sub block

- Inputs to this process are:

- a luma location ( xB, yB ) of the top-left luma sample of the current sub block relative to the top-left luma sample of the current prediction unit,

- variables nSubBlkW and nSubBlkH specifying the width and the height, respectively, of the current sub block.

- an array of reconstructed depth samples refDepPels

- a luma location ( xTL, yTL ) of the top-left luma sample of the current reference block relative to the top-left luma sample of the array of reconstructed depth samples,

- variable nSubSubBlkW and nSubSubBlkW specifying the conversion precision of the corresponding depth samples,

- a view order index refViewIdx specifying a reference view,

- an array disparitySamples of disparities values.

Outputs of this process are:

- an modified array disparitySamples of disparities values.

The array disparitySamples is modified as specified in the following:

- For sBy in the range of 0 to ( ( nSubBlkH / nSubSubBlkH) –1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nSubBlkW / nSubSubBlkW) –1 ), inclusive, the following applies:

    - The variable maxDep is set equal to –1 and modified as specified in the following.

xSubB = xB + sBx * nSubSubBlkW
		ySubB = yB + sBy * nSubSubBlkH
		xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xSubB )
		yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + ySubB ) 
		xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xSubB + nSubSubBlkW – 1 )
		yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + ySubB + nSubSubBlkH – 1 )
		maxDep = Max( maxDep, refDepPels[ xP0 ][ yP0 ] )
		maxDep = Max( maxDep, refDepPels[ xP0 ][ yP1] )
		maxDep = Max( maxDep, refDepPels[ xP1][ yP0 ] )
		maxDep = Max( maxDep, refDepPels[ xP1][ yP1 ] )

    - The values of the array depthSamples are modified as specified in the following:

for ( yOff = 0; yOff < nSubSubBlkH; yOff++ )
			for( xOff = 0; xOff < nSubSubBlkW; xOff++ ) {
				x = xSubB + xOff 
				y = ySubB + yOff 
				disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]
			}

  - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 apply.

  - In-loop filter process

The specifications in subclause 8.7 apply.

- Parsing process

  - General

The specifications in clause 9.1 apply with the following modifications

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause H.7.3 .

–	All invocations of the process specified in subclause 9.1 are replaced with invocations of the process specified in subclause H.9.1.

  - Parsing process for 0-th order Exp-Golomb codes

    - General

The specifications in subclause 9.2.1 apply with the following modifications .

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause H.7.3.

–	All invocations of the process specified in subclause 9.2.2 are replaced with invocations of the process specified in subclause H.9.2.2.

    - Mapping process for signed Exp-Golomb codes

The specifications in subclause 9.2.1 apply with the following modifications.

–	All references to the process specified in subclause 9.2 are replaced with references to the process specified in subclause H.9.2.1. [Ed. (GT) Reference in base spec to 9.2 is wrong.]

  - CABAC parsing process for slice segment data

    - General

The specifications in subclause 9.3.1 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.8.1 to through 7.3.8.11 are replaced with references to the process specified in subclauses H.7.3.8.1 to H.7.3.8.11

–	All invocations of the process specified in subclause 9.3.2 are replaced with invocations of the process specified in subclause H.9.3.2.

–	All invocations of the process specified in subclause 9.3.3 are replaced with invocations of the process specified in subclause H.9.3.3.

–	All invocations of the process specified in subclause 9.3.4 are replaced with invocations of the process specified in subclause H.9.3.4 .

–		All invocations of the process specified in subclause 9.3.2.3 are replaced with invocations of the process specified in subclause H.9.3.2.3.

    - Initialization process

      - General

The specifications in subclause 9.3.1 apply with the following modifications.

–	All invocations of the process specified in subclause 9.3.2.2 are replaced with invocations of the process specified in subclause H.9.3.2.2.

–	All invocations of the process specified in subclause 9.3.2.4 are replaced with invocations of the process specified in subclause H.9.3.2.4.

      - Initialization process for context variables

The specifications in subclause 9.3.2.2 apply with the following modifications.

- All references to the process specified in subclauses7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses H.7.3.8.1 to H.7.3.8.11.

- Table H-12 is appended to the end of Table 9-4.

- Table H-13 to Table H-24 are appended to the end of the subclause.

Table H-12 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) depth_mode_parameters( ) | depth_intra_mode | Table H-21 | 0..7 | 8..15 | 16..23 |
|  | wedge_full_tab_idx | Table H-13 | 0 | 1 | 2 |
|  | wedge_predtex_tab_idx | Table H-14 | 0 | 1 | 2 |
|  | edge_code | Table H-16 | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Table H-17 |  | 0..3 | 4..7 |
|  | ic_flag | Table H-18 |  | 0 | 1 |
|  | edge_dc_flag | Table H-23 | 0 | 1 | 2 |
|  | edge_dc_abs | Table H-24 | 0 | 1 | 2 |
|  | sdc_residual_flag | Table H-19 | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | prefix: Table H-20 suffix: na | prefix:0 suffix: na | prefix:1 suffix: na | prefix:0 suffix: na |
|  | inter_sdc_flag | Table H-25 | 0 | 1 | 2 |
|  | inter_sdc_resi_abs_minus1 | Table H-26 | 0 | 1 | 2 |
|  | inter_sdc_resi_sign_flag | Table H-27 | 0 | 1 | 2 |

[Ed (GT). Tables need to be sorted. ].

Table H-13 – Values of initValue for wedge_full_tab_idx ctxIdx

| Initialization variable | ctxIdx of wedge_full_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-14– Values of initValue for wedge_predtex_tab_idx ctxIdx

| Initialization variable | ctxIdx of wedge_predtex_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-15 – Values of initValue for dmm_dc_abs ctxIdx

| Initialization variable | ctxIdx of dmm_dc_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-16 – Values of initValue for edge_code ctxIdx

| Initialization variable | ctxIdx of edge_code |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-17 – Values of initValue for iv_res_pred_weight_idx ctxIdx

| Initialization variable | ctxIdx of iv_res_pred_weight_idx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table H-18 – Values of initValue for ic_flag ctxIdx

| Initialization variable | ctxIdx of ic_flag |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 197 | 197 |

Table H-19 – Values of initValue for sdc_residual_flag ctxIdx

| Initialization variable | ctxIdx of sdc_residual_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-20 – Values of initValue for sdc_residual_abs_minus1 ctxIdx

| Initialization variable | ctxIdx of sdc_residual_abs_minus1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | prefix: 155 suffix : na | prefix: 155 suffix : na | prefix: 155 suffix : na |

Table H-21 – Values of initValue for depth_intra_mode ctxIdx

| Initialization variable | ctxIdx of depth_intra_mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 0 | 0 | 64 | 168 | 168 | 124 | 154 | 0 |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 0 | 64 | 0 | 183 | 154 | 108 | 0 | 0 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 64 | 0 | 154 | 154 | 168 | 109 | 0 | 0 |

Table H-22 – Values of initValue for dmm_dc_flag ctxIdx

| Initialization variable | ctxIdx of dmm_dc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 0 | 0 | 64 |

Table H-23 – Values of initValue for edge_dc_flag ctxIdx

| Initialization variable | ctxIdx of edge_dc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-24 – Values of initValue for edge_dc_abs ctxIdx

| Initialization variable | ctxIdx of edge_dc_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-25 – Values of initValue for inter_sdc_flag ctxIdx

| Initialization variable | ctxIdx of inter_sdc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-26 – Values of initValue for inter_sdc_resi_abs_minus1 ctxIdx

| Initialization variable | ctxIdx of inter_sdc_resi_abs_minus1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-27 – Values of initValue for inter_sdc_resi_sign_flag ctxIdx

| Initialization variable | ctxIdx of inter_sdc_resi_sign_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

      - Storage process for context variables

The specifications in subclause 9.3.2.3 apply with the following modifications

- All references to the process specified in subclauses 7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses H.7.3.8.1 to H.7.3.8.11

      - Synchronization process for context variables

The specifications in subclause 9.3.2.4 apply with the following modifications

–	All references to the process specified in subclauses 7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses H.7.3.8.1 to H.7.3.8.11

      - Initialization process for the arithmetic decoding engine

The specifications in subclause 9.3.2.5 apply.

    - Binarization process

      - General

The specifications in subclause 9.3.3.1 apply with the following modifications.

- Table H-28 is appended to the end of Table 9-32.

Table H-28 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | ic_flag | FL | cMax = 1 |
|  | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | inter_sdc_flag | FL | cMax = 1 |
|  | inter_sdc_resi_abs_minus1 | UEG0 | [Ed. (GT) To be specified] |
|  | inter_sdc_resi_sign_flag | FL | cMax = 1 |
| depth_mode_parameters( ) | depth_intra_mode_set_indication_flag | FL | cMax = 1 |
|  | depth_intra_mode | TR | cMax = depthIntraModeMaxLen, cRiceParam = 0 |
|  | edge_start_left_flag [Ed. (GT) Missing! ] |  |  |
|  | edge_start_position [Ed. (GT) Missing! ] |  |  |
|  | num_edge_codes_minus1 [Ed. (GT) Missing! ] |  |  |
|  | edge_code | TR | cMax = 6, cRiceParam = 0 |
|  | dmm_dc_flag | FL | cMax = 1 |
|  | dmm_dc_abs | UEG0 | [Ed. (GT) To be specified] |
|  | dmm_dc_sign_flag | FL | cMax = 1 |
|  | edge_dc_flag | FL | cMax = 1 |
|  | edge_dc_abs | FL | cMax = 1 |
|  | edge_dc_sign_flag | FL | cMax = 1 |
|  | sdc_residual_flag | FL | cMax = 1 |
|  | sdc_residual_sign_flag | FL | cMax = 1 |
|  | sdc_residual_abs_minus1 | H.9.3.3.10 |  |

The binarization for the syntax element wedge_partition_type is specified in Error: Reference source not found

| wedge_scalar[ x0 ][ y0 ] | Bin string |
| --- | --- |
| 0 | 1 |
| 1 | 00 |
| 2 | 01 |

The binarization for the syntax element wedge_scalar is specified in Error: Reference source not found

| wedge_scalar[ x0 ][ y0 ] | Bin string |
| --- | --- |
| 0 | 0 |
| 1 | 100 |
| 2 | 101 |
| 3 | 110 |
| 4 | 111 |

The binarization for the syntax element wedge_abs_offset is specified in Error: Reference source not found depending on the value of wedge_partition_type and wedge_scalar

| wedge_scalar[ x0 ][ y0 ] | wedge_scalar[ x0 ][ y0 ] | Bin string |
| --- | --- | --- |
| 1 | - | FL, cMax = 2n - 1 |
| 2 | - | FL, cMax = 2n - 1 |
| - | 0 | FL, cMax = 4n - 1 |
| - | 1 | FL, cMax = 2n - 1 |
| - | 2 | FL, cMax = 2n - 1 |
| - | 3 | FL, cMax = 2n - 1 |
| - | 4 | FL, cMax = 2n - 1 |

Table H-29 –Values of wedgeFullTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 7 | 10 | 11 | 11 | 13 |

Table H-30 –Values of wedgePredTexTabIdxBits[ log2PUSize ]

| Initialization variable | wedgePredTexTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 4 | 7 | 8 | 8 | 0 |

      - Truncated Rice (TR) binarization process

The specifications in subclause 9.3.3.2 apply.

      - k-th order Exp-Golomb (EGk) binarization process

The specifications in subclause 9.3.3.3 apply.

      - Fixed-length (FL) binarization process

The specifications in subclause 9.3.3.4 apply.

      - Binarization process for part_mode

The specifications in subclause 9.3.3.5 apply.

      - Binarization process for intra_chroma_pred_mode

The specifications in subclause 9.3.3.6 apply.

      - Binarization process for inter_pred_idc

The specifications in subclause 9.3.3.7 apply.

      - Binarization process for cu_qp_delta_abs

The specifications in subclause 9.3.3.8 apply.

      - Binarization process for coeff_abs_level_remaining

The specifications in subclause 9.3.3.9 apply.

      - Binarization process for sdc_residual_abs_minus1

- Input to this process is a request for the a syntax element sdc_residual_abs_minus1,

- Output of this process is the binarization of the syntax element.

- The bin string is a concatenation of a prefix bin string and, when present, a suffix bin string.

- The variable numDepthValues is derived as follows:

  - numDepthValues = 	dlt_flag[ nuh_layer_id ] ? 		(H-262)
	num_depth_values_in_dlt[ nuh_layer_id ] : ( 1  <<  BitDepthY ) − 1

- The variable cMaxPrefix is derived as follows:

  - cMaxPrefix =  ( numDepthValues * 3 )  >>  2 )

- For the derivation of the prefix bin string, the following applies:

- If sdc_residual_abs_minus1 is less than cMaxPrefix, the prefix bin string is a bit string of length sdc_residual_abs_minus1+ 1 indexed by binIdx. The bins for binIdx less than sdc_residual_abs_minus1 are equal to 1. The bin with binIdx equal to sdc_residual_abs_minus1 is equal to 0.

- Otherwise, the prefix bin string is a bit string of length cMaxPrefix with all bins being equal to 1.

When sdc_residual_abs_minus1 is greater than cMaxPrefix, the suffix of the bin string is present and it is derived as follows:

  - The suffix value suffixVal, is derived as follows:

    - suffixVal = sdc_residual_abs_minus1 − cMaxPrefix 	(H-263)

  - The suffix of the bin string is specified by Fixed-length (FL) binarization process as specified in subclause with suffixVal and cMax equal to ( numDepthValues − cMaxPrefix ) as the inputs.

    - Decoding process flow

      - General

The specifications in subclause 9.3.4.1 apply. with the following modifications.

–	All references to the process specified in subclause 9.3.3 are replaced with references to the process specified in subclause  H.9.3.3.

–	All invocations of the process specified in subclause 9.3.4.2 are replaced with invocations of the process specified in subclause H.9.3.4.2.

–	All invocations of the process specified in subclause 9.3.4.3 are replaced with invocations of the process specified in subclause H.9.3.4.3.

      - Derivation process for ctxTable, ctxIdx and bypassFlag

        - General

The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- Table H-31 is appended to the end of Table 9-37.

Table H-31 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_dc_flag | 0 | na | na | na | na | na |
| dmm_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| edge_dc_flag | 0 | na | na | na | na | na |
| edge_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| edge_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| edge_code | 0 | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| sdc_residual_flag | bypass | na | na | na | na | na |
| sdc_residual_abs_minus1 | prefix: 0 suffix: bypass | prefix: 0 suffix: bypass | prefix: 0 suffix: bypass | prefix: 0 suffix: bypass | prefix: 0 suffix: bypass | prefix: 0 suffix: bypass |
| inter_sdc_flag | 0 | 0 | 0 | 0 | 0 | 0 |
| inter_sdc_resi_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| inter_sdc_resi_sign_flag | 0 | 0 | 0 | 0 | 0 | 0 |

        - Derivation process of ctxInc using left and above syntax elements

The specifications in subclause 9.3.4.2.2 apply with the following modifications.

–	Table H-32 is appended to the end of Table 9-38.

Table H-32 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| iv_res_pred_weight_idx | iv_res_pred_weight_idx [ xL ][ yL ] | iv_res_pred_weight_idx [ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

        - .Derivation process of ctxInc for the syntax elements last_sig_coeff_x_prefix and last_sig_coeff_y_prefix

The specifications in subclause 9.3.4.2.3 apply.

        - Derivation process of ctxInc for the syntax element coded_sub_block_flag

The specifications in subclause 9.3.4.2.4 apply.

        - Derivation process of ctxInc for the syntax element sig_coeff_flag

The specifications in subclause 9.3.4.2.5 apply.

        - Derivation process of ctxInc for the syntax element coeff_abs_level_greater1_flag

The specifications in subclause 9.3.4.2.6 apply.

        - Derivation process of ctxInc for the syntax element coeff_abs_level_greater2_flag

The specifications in subclause 9.3.4.2.7 apply.

      - Arithmetic decoding process

The specifications in subclause 9.3.4.3 apply. with the following modifications.

–	All references to the process specified in subclause 9.3.4.2 are replaced with references to the process specified in subclause H.9.3.4.2.

    - Arithmetic encoding process (informative)

The specifications in subclause 9.3.5 apply with the following modifications.

- All references to the process specified in subclause 9.3.4.3 are replaced with references to the process specified in subclause H.9.3.4.3.

- Sub-bitstream extraction process

The specifications in clause 10 apply.

- Profiles and levels

The specifications in Annex A apply, with the following modification:

TBD

- Byte stream format

The specifications in Annex B apply.

- Hypothetical reference decoder

The specifications in clause F.13 apply.

- Supplemental enhancement information

  - General

The specifications in clause F.14 apply.

  - SEI payload syntax

The specifications in subclause G.14.2 together with the extensions and modifications specified in this subclause apply.

    - Alternative depth information SEI message syntax

| alternative_depth_info ( payloadSize ) { | Descriptor |
| --- | --- |
| alternative_depth_info_cancel_flag | u(1) |
| if( alternative_depth_info_cancel_flag = = 0 ) { |  |
| depth_type | u(2) |
| if( depth_type = = 1 ) { |  |
| min_offset_x_int | se(v) |
| min_offset_x_frac | u(8) |
| max_offset_x_int | se(v) |
| max_offset_x_frac | u(8) |
| offset_y_present_flag | u(1) |
| if( offset_y_present_flag ){ |  |
| min_offset_y_int | se(v) |
| min_offset_y_frac | u(8) |
| max_offset_y_int | se(v) |
| max_offset_y_frac | u(8) |
| } |  |
| warp_map_size_present_flag | u(1) |
| if( warp_map_size_present_flag ) { |  |
| warp_map_width_minus2 | ue(v) |
| warp_map_height_minus2 | ue(v) |
| } |  |
| } |  |
| if( depth_type = = 0 ) { |  |
| num_residual_texture_views_minus1 | ue(v) |
| residual_depth_flag | u(1) |
| } |  |
| } |  |
| } |  |

  - SEI payload semantics

The specifications in subclause G.14.3 together with the extensions and modifications specified in this subclause apply.

    - Alternative depth information SEI message semantics

The alternative depth information SEI message indicates that decoded depth samples have to be interpreted as an alternative depth format. To discriminate different alternative depth formats, a depth_type syntax element is used. The information of the alternative depth information SEI message persists in output order until any of the following are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in an access unit containing an alternative depth information SEI message is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

alternative_depth_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous alternative depth information SEI message in output order. alternative_depth_info_cancel_flag equal to 0 indicates that alternative depth information follows.

depth_type identifies an alternative depth type according to Table H-33. A value of depth_type is equal to 0 indicates that this SEI message signals Global View and Depth (GVD) information. A value of depth_type is equal to 1 indicates that decoded depth samples can be used to derive a warp map and view synthesis can be performed by image-domain warping. .

Values of depth_type that are not listed in Table H-33 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore alternative depth information SEI messages that contain reserved values of depth_type.

Table H-33 – Interpretation of depth_type

| Value | Description |
| --- | --- |
| 0 | Global view and depth |
| 1 | Warp map |

NOTE 2  − When depth_type is equal to 0, decoding processes for inter-view prediction and decoding processes with depth-texture interaction should be disabled.

min_offset_x_int, min_offset_x_frac specify the integer and the fractional part of the minimum offset for the horizontal direction of a warp map.

The variable minOffsetX is derived as follows:

  - minOffsetX = min_offset_x_int + min_offset_x_frac  256	(H-264)

max_offset_x_int, max_offset_x_frac specify the integer and the fractional part of the maximum offset for the horizontal direction of a warp map.

The variable maxOffsetX value is derived as follows:

  - maxOffsetX = max_offset_x_int + max_offset_x_frac  256

offset_y_present_flag equal to 1 specifies that min_offset_y_int, min_offset_y_frac, max_offset_y_int and max_offset_y_frac are present. offset_y_present_flag equal to 0 specifies that min_offset_y_int, min_offset_y_frac, max_offset_y_int and max_offset_y_frac are not present.

min_offset_y_int, min_offset_y_frac specify the integer and the fractional part of the minimum offset for the vertical direction of a warp map. When not present, min_offset_y_int and min_offset_y_frac are inferred to be equal to 0.

The variable  minOffsetY value is derived as follows:

  - minOffsetY = min_offset_y_int + min_offset_y_frac  256	(H-265)

max_offset_y_int, max_offset_y_frac specify the integer and the fractional part of the maximum offset for the vertical direction of a warp map. When not present, max_offset_y_int and max_offset_y_frac are inferred to be equal to 0.

The variable maxOffsetY value is derived as follows:

  - maxOffsetY = max_offset_y_int + max_offset_y_frac  256	(H-266)

warp_map_size_present_flag equal to 1 specifies that a new warp map size is present, which is valid for the current and all following warp maps in output order until a new message with warp_map_size_present_flag equal to 1 is received or alternative_depth_info_cancel_flag is equal to.. warp_map_size_present_flag  equal to 0 specifies that the warp map size is not changed.

warp_map_width_minus2 plus 2 specifies the width of the warp map. The value of warp_map_width_minus2 shall be in the range of 0 to pic_width_in_luma_samples − 2, inclusive. The variable warpMapWidth is set equal to ( warp_map_width_minus2 + 2 )

warp_map_height_minus2 plus 2 specifies the height of the warp map. The value of warp_map_height_minus2 shall be in the range of 0 to ( pic_height_in_luma_samples  >>  offset_y_present_flag ) − 2, inclusive. The variable warpMapHeight is set equal to ( warp_map_height_minus2 + 2 )

- The variables deltaX, deltaY, scaleX and scale Y are derived as specified in the following:

  - deltaX = pic_width_in_luma_samples  ( warpMapWidth − 1 )	(H-267)

  - deltaY = pic_height_in_luma_samples  ( warpMapHeight − 1 )	(H-268)

  - scaleX = ( maxOffsetX − minOffsetX ) / ( ( 1  <<  BitDepthY ) − 1 )	(H-269)

  - scaleY = ( maxOffsetY − minOffsetY ) / ( ( 1  <<  BitDepthY ) − 1 )	(H-270)

Let recSamples[ x ][ y ] correspond to the reconstructed sample array SL of a depth view component. The corresponding horizontal warp map component w[ x ][ y ][ 0 ] and the corresponding vertical warp map component w[ x ][ y ][ 1 ] for recSamples[ x ][ y ] are derived as specified in the following:

for( x = 0; x < warpMapWidth ; x++ )
	 for( y = 0; y < warpMapHeight; y++){
		w[ x ][ y ][ 0 ] = x * deltaX + minOffsetX + scaleX * recSamples[ x ][ y ]
		if( offset_y_present_flag ) 
			w[ x ][ y ][ 1 ] =	y * deltaY + minOffsetY + 
							scaleY * recSamples[ x ][ y + pic_height_in_luma_samples / 2 ]
		else
			w[ x ][ y ][ 1 ] =	y * deltaY
	}

A warp map w[ x ][ y ] is derived for each input view using reconstructed samples of its corresponding depth view component, i.e. each input view has an associated warp map and vice versa. A warp map specifies a sparse set of positional correspondences. These correspondences identify semantically corresponding image locations between two views of the same time instance, i.e. the associated input view and a neighbouring input view which is identified as follows.

When the warp map w[ x ][ y ] is associated with the leftmost input view, then the warp map specifies for each sub-pel position (x*deltaX, y*deltaY ) in this input view a corresponding sub-pel position (2 * w[ x ][ y ][ 0 ], 2 * w[ x ][ y ][ 1 ]) in the closest input view on the right.

When the warp map w[ x ][ y ] is associated with an input view different to the leftmost input view, then the warp map specifies for each sub-pel position ( x * deltaX, y*deltaY ) in this input view a corresponding sub-pel position ( 2 * w[ x ][ y ][ 0 ], 2 * w[ x ][ y ][ 1 ] ) in the closest input view on the left.

NOTE 3  − A sample dense set of positional correspondences can be derived e.g. by bilinear interpolation.

num_residual_texture_views_minus1 plus1 specifies the number of sub-residual texture views packed in the residual  texture layer. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that the number of sub-residual depth views packed in the residual depth layer is equal to num_residual_texture_views_minus1 + 1. residual_depth_flag equal to 0 specifies the number of sub-residual depth views is equal to 0 and that no residual depth layer  is present in the bitstream.

When GVD information are signalled by the SEI message, information of multiple texture views are packed into two layers and information of multiple depth views is packed into one or two layers.

The two layers containing the texture views are the base texture layer and the residual texture layer. The base texture layer contains a base texture view in full resolution (e.g. the view from a central camera position) and is the layer with ViewIdx equal to 0 and DepthFlag equal to 0. The residual texture layer contains packed information of up to four additional views in quarter resolution (sub-residual texture views) and is the layer with ViewIdx equal to 1 and DepthFlag equal to 0.

- Each sub-residual texture view is derived by applying the following to an additional texture input view:

  - Project the base texture view to the position of the additional texture input view using the decoded base depth view ( and the decoded residual depth view, when residual_depth_flag is equal to 1).

  - Create a picture containing samples of the additional texture view that are located at positions not covered by projected sample positions of the base texture view.

  - Decimate the created picture by a factor of two in horizontal and vertical direction by discarding odd sample positions.

With increasing order of camera IDs (which are specified by external means), the order of sub-residual texture views within the residual texture layer is top-right, top-left, bottom-left, bottom-right.

NOTE 4  − An example is shown in Figure H-2. The variable N is set equal to ( num_residual_texture_views_minus1 + 2 ). The base texture view (B) is the input from central camera ( camera ID = 3 ). In the case N = 2, camera ID = 2 and 3 are used. In the case N = 3, camera ID = 2, 3 and 4 are used. In the case N = 4, camera ID = 1, 2, 3 and 4 are used. In the case N = 5, camera ID = 1, 2, 3, 4 and 5 are used. The N − 1 input texture views with camera ID not equal to 3 are converted to N − 1 quarter-size sub-residual texture views (R-x) and packed in the residual texture layer in order top-left (R-2), bottom-left (R-4), top-right (R-1) and bottom-right (R-5). Their top-left co-ordinates in the residual texture layer of width = W and height = H is shown in Table H-34. The residual texture layer represents occluded area or out-of-frame area of the base texture view when it is projected to the N − 1 input texture views by GVD process.

Figure H-2 Relation between camera ID and GVD texture/depth and packing of texture/depth views to the base and residual texture/depth layer

- Table H-34– Top-left corner co-ordinates of GVD sub-residual views packed in a residual layer
of width = W and height=H.

| R-1 | R-2 | R-4 | R-5 |
| --- | --- | --- | --- |
| ( W / 2, 0 ) | ( 0, 0) | ( 0, H / 2 ) | ( W/2, H / 2 ) |

The two layers containing the depth views are the base depth layer and the residual depth layer. The base depth layer contains a depth map in full resolution (base depth view) and is the layer with ViewIdx equal to 0 and DepthFlag equal to 1. The base depth view is generated as specified in the following:

  - Project all depth maps associated with the additional texture views to the position of the base view.

  - Derive the median of samples values projected to the same position.

When residual_depth_flag is equal to 0 and the number of input texture views is even (2 or 4), the position of the base depth view is the center of input views (e.g. camera ID = 2.5 when only cameras 2 and 3, or cameras 1, 2, 3 and 4 are present in the above example) and all depth maps are projected to this position, when the base depth view is generated.

When residual_depth_flag is equal to 1, the residual depth layer contains packed information derived from up to four depth views associated with the additional texture views in quarter resolution (sub-residual depth views). The residual depth layer is the layer with ViewIdx equal to 1 and DepthFlag equal to 1.

- Each sub-residual depth view is derived by applying the following to an additional depth input view:

  - Project the base depth view to the position of the additional depth input view using the decoded base depth view.

  - Create a picture containing samples of the additional depth view that are located at positions not covered by projected sample positions of the base depth view.

  - Decimate created the picture by a factor of two in horizontal and vertical direction by discarding odd sample positions.

The order of sub-residual depth views within the residual depth layer corresponds to the order of sub-residual texture layers in the residual texture layer.

- Video usability information

The specifications in clause G.15 apply.