| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0156 |
| --- | --- |

| Title: | MV-HEVC HLS: Indication of sampling grid position |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Ye Yan Univ. of Science and Technology of China | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution studies alternatives for arranging the processing chain of downsampling to vertically half resolution for output on a polarizing stereoscopic display and MV-HEVC encoding/decoding of stereoscopic content with inter-view prediction. It concludes that coding vertically half-resolution sequences improved the RD performance for polarizing displays compared to the MV-HEVC anchor settings by about 3% in BD bitrate. Furthermore, the contribution concludes that the use of interleaved sampling grid positions between views improved the BD bitrate by 0.2 %-unit on average and by 2.5 %-unit at maximum relative the use of aligned sampling grid positions. It is proposed to include vertical sampling grid position information in the VPS extension.

# Introduction

Passive stereoscopic 3D displays utilize polarized light in a manner that even pixel lines represent one view and have a polarizing filter of a certain orientation, while odd pixel lines represent another view and have a polarizing filter of an orthogonal orientation. When viewed with polarizing viewing glasses, each eye sees one view and half of the pixel lines of the display.

This contribution studies alternatives for arranging the processing chain of downsampling to vertically half resolution for output on a polarizing display and MV-HEVC encoding/decoding of stereoscopic content with inter-view prediction. In the targeted use case an encoder has knowledge of the receiver-side display arrangement, i.e. the display resolution and that the receiver has a polarizing display. Such knowledge could be obtained for example through capability exchange, such as SDP Offer/Answer negotiation.

The following alternatives, illustrated in Figures 1 to 3, are studied:

- Anchor: coding is performed at full resolution, and downsampling is done as post-processing in the decoder side.

- Vertically half-resolution coding with aligned sampling grid: Vertical downsampling is performed as pre-processing in the encoder side. MV-HEVC coding/decoding is applied. One of the decoded views is resampled as post-processing in the decoder side to obtain even sample lines from the decoded odd sample lines.

- Vertically half-resolution coding with interleaved sampling grid: Vertical downsampling is performed as pre-processing in the encoder side. One of the views is sampled at odd pixel lines and the other view is sampled at even pixel lines. MV-HEVC coding/decoding is applied without modifications. As the sampling grid positions of the views differ, inter-view prediction would typically have a vertical motion vector component with a magnitude of half-pixel in units of coded luma samples.

Figure 1. Anchor: coding at full resolution, downsampling as post-processing in the decoder side.

Figure 2. Vertically half-resolution coding with aligned sampling grid.

Figure 3. Vertically half-resolution coding with interleaved sampling grid.

# Simulations

SHM 2.1 was used for the simulations. JCT-3V common test conditions (JCT3V-D1100) for 2-view MV-HEVC were used for the anchor coding. The PSNR was derived against vertically half-resolution uncompressed sequences to reflect the perceived quality on a polarizing display. In all down- and upsampling operations, the filters in SHM were used with proper phase shifts.

The simulation results can be found in Table 1.

Table 1. Results of vertically half-resolution coding compared to the anchor.
a) Aligned sampling grid					b) Interleaved sampling grid

The following remarks are made about the results:

- On average and in 6 out of 7 sequences, coding of vertically half-resolution sequences improves the RD performance when compared to the anchor.

- On average and in 2 out of 6 sequences (where half resolution improves the RD performance), the use of the interleaved sampling grid outperforms the use of the aligned sampling grid. In S01 the improvement is about 2.5 %-units of BD bitrate.

It is therefore concluded that the optimal relative vertical sampling grid position for stereoscopic coding for polarizing displays is content-dependent.

# Proposal

In order to enable the use of interleaved sampling grid, related indications should be included in the MV-HEVC bitstream syntax. Options for the syntax structure to contain the indications include:

- VPS extension (or its VUI part).

VPS is envisioned to be used in capability exchange and session negotiation. The targeted use case for the vertically half-resolution coding with interleaved sampling grid requires the knowledge of the far-end display arrangement. Hence, having the sampling grid position indications included in VPS would support the use case. For example, an offerer in the SDP Offer/Answer procedure can include both a VPS of the full-resolution bitstream and a VPS of the vertically half-resolution bitstream with interleaved sampling grid positions as options in the offer, and the answerer can pick the option which better suits its display characteristics.

- SPS extension (or VUI extension).

The indication of the sampling grid position is not proposed to affect the decoding process. Hence, while the sampling grid position is typically sequence-level information, it is felt that SPS is not a correct container syntax structure.

- SEI message

The indication of the sampling grid position is not proposed to affect the decoding process, and hence the use an SEI message would be justified. However, as SEI messages are typically not part of capability exchange and as the bit count of sampling grid position is low (when considering the total bit count of VPS and when considering the overhead caused by NAL unit and SEI message headers), the VPS extension approach is preferred.

The syntax and semantics for the VPS extension option is therefore provided below. The semantics are similar to sampling grid position signaling of the frame packing arrangement SEI message. However, to make the overhead as small as possible, rather than using units of 1/16th of the luma sample grid spacing constantly (as in the frame packing arrangement SEI message), the units are specified (1/2th, 1/4th, 1/8th, or 1/16th of the luma sample grid spacing) and the length of the sampling grid position syntax element (1, 2, 3, or 4 bits respectively).

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if( NumViews > 1 ) { |  |
| view_id_len_minus1 | u(4) |
| grid_pos_present_flag | u(1) |
| if( grid_post_present_flag ) |  |
| grid_pos_len_minus1 | u(2) |
| } |  |
| for( i = 0; i < NumViews; i++ ) { |  |
| view_id_val[ i ] | u(v) |
| if( i > 0 ) |  |
| grid_pos_y[ i ] | u(v) |
| } |  |
| … |  |

grid_pos_present_flag equal to 0 specifies that the grid_pos_len_minus1 and grid_pos_y[ i ] syntax elements are not present and that all views have the same vertical sampling grid alignment positioning. grid_pos_present_flag equal to 1 specifies that the grid_pos_len_minus1 and grid_pos_y[ i ] syntax elements are present.

grid_pos_len_minus1 plus 1 specifies the length of grid_pos_y[ i ] syntax element.

grid_pos_y[ i ] specifies the vertical sampling grid alignment positioning of the i-th view relative to vertical sampling grid position of the 0th view as follows. Let the variable posLen be equal to grid_pos_len_minus1 + 1. Let the variable pow2PosLen be equal to 1 << posLen. grid_pos_y[ i ] specifies the vertical sampling grid alignment positioning of the upmost sample line of the i-th view relative to the upmost sample line of the 0th view in units of one pow2PosLen-th of the luma sample grid spacing. The length of the grid_pos_y[ i ] syntax element is posLen.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The authors are not aware of University of Science and Technology of China having any current or pending patent rights relating to the technology described in this contribution.