| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CN, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0159 |
| --- | --- |

| Title: | CE5: Fast depth lookup table application method to intra modes for depth data |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Jie Jia Sehoon Yea, | Email: | jin78.heo@lge.com, jie.jia@lge.com sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

Depth lookup table (DLT) application method was proposed in the last meeting. It used an alternative residual generation method for all intra modes except SDC modes. DLT application method generates two different residual types: 1) residual value between original and prediction pixel values and 2) residual index between original and prediction pixel indexes generated by DLT and then selects the best intra mode between them in terms of rate distortion optimization (RDO). However, in order to select the best prediction mode DLT application method requires RDO process twice for each candidate mode. Consequently, it leads to encoder complexity increase. Therefore, this contribution proposes a fast depth lookup table application method in an alternative residual generation method.

# Introduction

Applying depth look-up table to intra modes of depth map was introduced at the last meeting [1] and it provides -0.29% and -0.54% coding gains on synthesized view in random access and all-intra configuration. However, since the method fully checked the cost values of two different residual types for each mode to select the best prediction mode, it takes long time to find the best prediction mode. Therefore, this contribution proposes a fast depth look-up table application method to reduce the number of candidates used in RDP process.

# Proposed Method

In order to use the advantage of DLT used in SDC, depth lookup table (DLT) application method proposes an alternative residual generation method. The method generates two different residual types: one is residual value and the other is residual index. The former is the difference between the original and the prediction pixel values directly and the latter is the difference between the original and the prediction pixel index through DLT. Let org[x], pred[x] and resi[x] be the original value, intra predicted value and residual at position x respectively, and let DLT[a] be the index of depth value a. The method generates two different residual types: residual value and residual index as follows:

resi_value[x] = org[x] – pred[x]                              (1)

resi_index[x] = DLT[org[x]] – DLT[pred[x]]                        (2)

Rate distortion optimized selection between residual value and residual index in 3D-HEVC is performed at encoder and one flag is encoded for each intra coded (excluding SDC) depth CU to indicate which method is used.

At decoder, Let rec[x] be the reconstructed value, and let IDLT[a] be the depth value of index a. The rec[x] is constructed as based on the resi_value and resi_index:

rec[x] = resi_value[x] + pred[x]                              (3)

rec[x] = IDLT[ DLT[pred[x]] + resi_index[x] ]                        (4)

In order to select the best prediction mode in two different residual types, DLT application method should perform RDO process twice in one prediction mode. As a result, it causes increase of encoder complexity. Therefore, this contribution proposes three fast depth lookup table application methods.

## Method I: applying depth lookup table to 2Nx2N block size

In the current intra coding process, the intra prediction is employed at both NxN and 2Nx2N block size. However, in order to reduce encoder complexity the proposed depth lookup table method is used only to 2Nx2N block size.

## Method II: Method I + reduction of the number of candidate modes

In order to select the best prediction mode, two different RDO processes; simplified RDO and full RDO are used. First, the simplified RDO selects eight candidate modes among conventional 35 intra prediction modes and then addes intra modes related to the detph intra tools; depth modeling mode (DMM) and region boundary chain (RBC) mode as candidate modes. Finally, the best prediction mode is selected among selected candidate modes obtained from the simplified RDO for the current block in terms of full RDO.

In order to reduce the complexity, the proposed method reduces the number of candidate modes through simlified RDO from eight to four and then addes depth intra tool modes to candidate modes list. Consequently, the proposed method can reduce the number of candidate modes chekced in full RDO process.

## Method III: applying depth lookup table to particualr modes

This method applied depth lookup table method only to particular intra modes; DMM, RBC, DC, vertical, and horizontal modes which are not used interpolation process to construct the prediction samples. Therefore, if the current mode is a mode among the predefined particualr modes, residual indexes are generated by DLT. Otherwise, residual values are constructed. After constructing the residaual data, transform and quantization are performed and then, quantized transform coefficients are entered into entropy coder. This proposed method has two advantages; unnecessariness of a flag bit and less complexity compared to Method I and Method II.

# Experimental Results

The proposed method is integrated into HTM 8.0 reference software and three tests are conducted under random access (CTC) and intra-only configuration.

## Test 1: Results of the proposed method I

The proposed method I provides 0.2% and 0.3% coding gains on synthesized view for CTC and AI cases, respectively.

Table 1 Result for 3-view scenario under CTC

Table 2 Result for 3-view scenario under intra-only configuration

## Test 2: Results of the proposed method II

The proposed method II provides 0.1% and 0.2% coding gains on synthesized view for CTC and AI cases, respectively.

Table 3 Result for 3-view scenario under CTC

Table 4 Result for 3-view scenario under intra-only configuration

## Test 3: Results of the proposed method III

The proposed method III provides 0.1% and 0.2% coding gains on synthesized view for CTC and AI cases, respectively.

Table 5 Result for 3-view scenario under CTC

Table 6 Result for 3-view scenario under intra-only configuration

# Working Draft Modifications

## H.8.6 Scaling, transformation and array construction process prior to deblocking filter process

### H.8.6.1 Derivation process for quantization parameters

The specifications in subclause 8.6.1 apply.

### H.8.6.2 Scaling and transformation process

The specifications in subclause 8.6.2 apply.

### H.8.6.3 Scaling process for transform coefficients

The specifications in subclause 8.6.3 apply.

### H.8.6.4 Transformation process for scaled transform coefficients

The specifications in subclause 8.6.4 apply.

### H.8.6.5 Picture construction process prior to in-loop filter process

Inputs to this process are:

–	a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nCurrS specifying the size of the current block,

–	a variable cIdx specifying the colour component of the current block,

–	an (nCurrS)x(nCurrS) array predSamples specifying the predicted samples of the current block,

–	an (nCurrS)x(nCurrS) array resSamples specifying the residual samples of the current block.

Depending on the value of the colour component cIdx, the following assignments are made:

–	If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y.

–	Otherwise, if cIdx is equal to 1, recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C.

–	Otherwise (cIdx is equal to 2), recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C.

Depending on the intra prediction mode, The (nCurrS)x(nCurrS) block of the reconstructed sample array recSamples at location ( xCurr, yCurr ) is derived as follows:

–	if predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, INTRA_ANGULAR26, INTRA_DMM_WFULL, INTRA_DMM_WPREDTEX, INTRA_DMM_CPREDTEX, or INTRA_CHAIN and SdcFlag[ xCurr ][ yCurr ] is equal to 0 and dlt_flag[ nuh_layer_id ] is equal to 1,

recSamples[ xCurr + i ][ yCurr + j ] = Idx2DepthValue[ clip3( 0, num_depth_values_in_dlt[ nuh_layer_id ] – 1, DepthValue2Idx[ predSamples[ i ][ j ] ] + resSamples[ i ][ j ] ) ] with i = 0..nCurrS − 1, j = 0..nCurrS − 1

–	Otherwise

The (nCurrS)x(nCurrS) block of the reconstructed sample array recSamples at location ( xCurr, yCurr ) is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = clipCidx1( predSamples[ i ][ j ] + resSamples[ i ][ j ] )	(8-1)
	with i = 0..nCurrS − 1, j = 0..nCurrS − 1

# Conclusion

This contribution proposes a fast depth lookup table application method to further employ the advantage of DLT to all intra modes except for SDC modes. The proposed method provides 0.2% and 0.3% bit savings for random access (CTC) and all-intra configuration.

# Reference

- H. Liu, J. Jia, “CE 6.h related: Applying Depth Look-up Table to Intra Modes of Depth Map,” JCT3V-E0157, Vienna, AT, July 2013.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).