| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CN, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0160 |
| --- | --- |

| Title: | Non-CE: Illumination compensation flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jin Heo, Sehoon Yea, | Email: | jin78.heo@lge.com, sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents an adaptive context model selection method for illumination compensation (IC) flag. This contribution adaptively uses three context models for IC CU flag coding derived from neighbouring blocks. The proposed method provides 0.2% coding gain in the common test condition (CTC).

# Introduction

In the current 3D-HEVC [2], a CU-level flag (i.e. ic_flag) [1] for illumination compensation (IC) is adaptively signalled for a block predicted from an adjacent view to compensate the luminance discrepancy among views. When ic_flag is encoded, only one context model is used. This contribution adaptively selects the proper context model according to the context models derived from neighbouring blocks among three context models.

# Proposed Method

In this contribution, three context models are used to encode ic_flag with CABAC engine. The context index increment is adaptively calculated by the ic_flag derived from neighbouring blocks; the above CU and the left CU.

context index increment = (ic_flag(Above) != 0) ? 1 : 0 + (ic_flag(Left) != 0) ? 1 : 0       (1)

# Experimental Results

The proposed method is integrated into HTM 8.0 reference software and simulation is conducted under common test conditions (CTC).

The proposed method provides 0.2% coding gain on synthesized view for CTC.

Table 1 Result for 3-view scenario under CTC

# Working Draft Modifications

H.9.3.2.2 Initialization process for context variables

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) depth_mode_parameters( ) | depth_intra_mode | Error: Reference source not found | 0..7 | 8..15 | 16..23 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_predtex_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_code | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ic_flag | Table Error: Reference source not found-2 |  | 00..2 | 13..5 |
|  | edge_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | prefix: Error: Reference source not found suffix: na | prefix:0 suffix: na | prefix:1 suffix: na | prefix:0 suffix: na |
|  | inter_sdc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | inter_sdc_resi_abs_minus1 | Error: Reference source not found | 0 | 1 | 2 |
|  | inter_sdc_resi_sign_flag | Error: Reference source not found | 0 | 1 | 2 |

Table Error: Reference source not found-2 – Values of initValue for ic_flag ctxIdx

| Initialization variable | ctxIdx of ic_flag |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 197 | 197 |

| Initialization variable | ic_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

The specifications in subclause 9.3.4.2.2 apply with the following modifications.

–	Table Error: Reference source not found-3 is appended to the end of Table 9-38.

Table Error: Reference source not found-3 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| iv_res_pred_weight_idx | iv_res_pred_weight_idx [ xL ][ yL ] | iv_res_pred_weight_idx [ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| ic_flag | ic_flag[ xL ][ yL ] | ic_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

# Conclusion

This contribution presents an adaptive context model selection method for illumination compensation (IC) flag. This contribution adaptively uses three context models for IC CU flag coding derived from neighbouring blocks. The proposed method provides 0.2% coding gain in the common test condition (CTC).

# Reference

- H. Liu, J. Jung, J. Sung, etc., (LG), “3D-CE 2.h: Results of Illumination Compensation for Inter-View Prediction,” JCT3V-B0045, Shanghai, China, October, 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).