| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CN, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0161 |
| --- | --- |

| Title: | CE4 related: Coding of weighting factor of advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Sunmi Yoo, Sehoon Yea, | Email: | jin78.heo@lge.com, sunmi.yoo@lge.com sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

Advanced residual prediction (ARP) was proposed to further improve the coding efficiency of inter-view residual prediction. In order to reduce the prediction error between residue of the current block of the picture in the current view and the corresponding block of the picture in the reference view, an adaptive weighting factor is used. This contribution modified the initial value of context model for ARP weighting factors based on the distribution of each weighting factor. The proposed method provides 0.3% coding gain under the common test condition (CTC).

# Proposed Method

It is proposed that values of four contexts are differently initialized for the CABAC coding of the ARP [1] weighting factor while all values of four contexts are set to the same value in the current design. The initial values of four contexts are determined based on the distribution of each weighting factor.

The working draft modifications are below.

Table Error: Reference source not found-1 – Values of initValue for iv_res_pred_weight_idx ctxIdx

| Initialization variable | ctxIdx of iv_res_pred_weight_idx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 154 162 | 154 153 | 154 154 | 154 162 | 154 162 | 154 153 | 154 154 | 154 162 |

# Experimental Results

The proposed method is integrated into HTM 8.0 reference software and simulation is conducted under common test conditions (CTC).

The proposed method provides 0.3% coding gain on synthesized view for CTC.

Table 1 Result for 3-view scenario under CTC

# Conclusion

This contribution modified the initial value of context model for ARP weighting factors based on the distribution of each weighting factor. The proposed method provides 0.3% coding gain under the common test condition (CTC).

# Reference

- L. Zhang, Y. Chen, X. Li, and M. Karczewicz, “CE 4: Advanced residual prediction for multiview coding,” JCT3V-D0177, Incheon, KR, April, 2013.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).