| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0163 |
| --- | --- |

| Title: | CE2 related: Derivation of motion-aware temporal disparity vector |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Sehoon Yea | Tel: Email: | junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In 3D-HEVC, a disparity vector is derived from a disparity vector of temporal or spatial neighboring blocks. Especially, a location of temporal neighboring blocks is fixed as a center position of the co-located CU. If there is temporal motion, the co-located CU does not correspond with the current CU. This contribution proposes motion-aware temporal disparity vector derivation method. It is proposed to derive a disparity vector of a temporal block which is derived by a motion vector of neighboring blocks in the current CU. Experimental results show that the proposed method can achieve 0.1% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In 3D-HEVC, a disparity vector is derived from a motion vector of temporal, spatial and a disparity vector associated with MCP coded neighboring blocks for inter-view prediction. Once a disparity motion vector is found, the derivation process for the disparity vector is terminated.

Figure 1. The conventional disparity vector derivation from temporal pictures

Figure 1 shows the conventional disparity vector derivation from temporal picture. The disparity vector (DVc') is derived from co-located block (C') in temporal picture. The position of the corresponding block is always fixed. There is no consideration of whether current block has any motion. In the case of having some motion in the current block, an inaccurate disparity vector could be derived from temporal co-located block.

# Proposed method

In this contribution, we propose motion-aware temporal disparity derivation method. The proposed method is that the position of corresponding block in temporal picture is derived by spatial neighboring block motion vector (NBMV). Figure 2 shows an example of the proposed disparity vector derivation from temporal pictures. In the proposed method, a disparity (DVc") is derived from motion-aware corresponding block (C") which is shifted by a motion vector (MVL) of neighboring left or above blocks.

Figure 2. The proposed disparity vector derivation from temporal pictures

The spatial neighboring blocks for NBMV process are visited order of left and above. A motion vector is derived as follows,

Step1) If POC of a reference picture for a spatial neighboring block is equal to POC of a temporal candidate picture for a temporal disparity vector derivation, a motion vector of the spatial block sets the motion vector of neighboring block and NBMV process is terminated.

Step2) If the motion vector of neighboring block is not found during step1, motion vector of a spatial neighboring block is scaled by difference between If POC of a reference picture for a spatial neighboring block and POC of a temporal candidate picture for temporal disparity vector derivation.

# Experimental results

The results indicate that the proposal has a slightly coding gains compared to HTM-8.0 using common test conditions [1]. Encoding and decoding time are measured on a Linux cluster and is expected to be similar as for HTM-8.0.

Table 1. Performance of the proposed method compared to HTM-8.0

# Conclusion

This contribution proposes motion-aware disparity derivation for temporal pictures. The proposal achieved 0.1% gains under common test conditions.

# Reference

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-E1100, Vienna, AT, Aug. 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).