| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0166 |
| --- | --- |

| Title: | JCT3V – Inter-view MV-based rate prediction for rate control of 3D multi-view video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document Approved by the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Lim, Hyunho Jo, Donggyu Sim 512 Bima-building, Kwangwoon Univ. Wolgye-dong, Seoul, Korea. | Tel: Email: | +82-10-2744-6098 limwoong@kw.ac.kr idjhh@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

# Abstract

A rate control algorithm is an essential part of the practical video encoder for various multimedia services and the latest reference software of HEVC, HM-12.0, includes two rate control algorithm s based on URQ (Unified Rate Quantization) and R-lambda model [1-3]. In the last meeting in Vienna, ‘Inter-view MAD prediction for rate control of 3D multi-video coding’ has been proposed, adopted and integrated on the latest test model for multi-view video coding, HTM8.2. After the last meeting, we invested further to improve the performance of the rate control of the extended view and propose inter-view MV-based rate prediction algorithm. By using the proposed method, we achieved xx% coding gain for the extended view compared to the previous inter-view MAD prediction algorithm.

# Introduction

Our proposed URQ model-based rate control algorithm, described in JCTVC-H0213 [1], was adopted in HM reference software at the 8th San Jose meeting in February 2012. Some improvements were reported in JCTVC-I0094 [2]. At the meeting in Shanghai, China, another rate control algorithm based on R-lambda model was newly adopted in the HM reference software for single-view coding [3]. After URQ and R-lambda model-based rate control has proposed, there is no rate control algorithm in the multi-view reference software and no further study though it is very important in the practical use. In the last Vienna meeting, to improve the performance of the rate control for multi-view video coding, we introduced the view-wise bitrate allocation and inter-view texture complexity prediction [8].

The inter-view MAD prediction algorithm was designed based on an assumption that the ratio of texture complexity between views is similar. However, generally the encoding configurations of the base view and extended view is different. Especially, for example, the QP value of the base view is lower than the extended view because the extended view exploits not only the temporal correlation but also the inter-view correlation by referencing the reconstructed base view picture. The conventional rate control model has a consistency in each view and the temporal correlation is generally higher than inter-view correlation. The reason why we employed inter-view texture complexity instead of the texture complexity of temporally collocated block is that the motion of the extended view can be reflected from the motion of the base view. However, as aforementioned, there is the temporal consistency in the extended view to preserve to keep the certain bitrate and the quality even if there is a correlation among views. Therefore, in this contribution, we propose the inter-view MV-based rate prediction algorithm which predicts the target bitrate from the temporal corresponding, not collocated, block by referencing the motion information of the depth map-based inter-view corresponding block.

# The proposed inter-view MV-based rate prediction algorithm

The current rate control algorithm implemented on HM reference software only considers the temporal texture complexity or bit weight to predict the best QP for the current unit, frame or CTU, for single-view video. In multi-view video coding, motion information of views is highly correlated and the correlation can be exploited to reference the temporal texture complexity or bit weight.

Figure 1. An example of inter-view MV-based rate prediction

As shown in Fig. 1, the current single-view rate control algorithm exploits the temporal texture complexity or bit weight form the collocated unit in the previously encoded frame. However, when the current unit contains the moving object, the prediction accuracy of the texture complexity or bit weight is low along how fast the moving objects are, the distance between the current picture and the temporal reference picture. In contrast to the base view, all the extended views can reference the inter-view picture acquired at the same time by slightly translated cameras. Those inter-view reference pictures are geometrically high correlated to the current picture. Therefore, we can predict the motion information and apply the motion to the current unit to find the corresponding temporal unit. By using the motion-based temporal texture or bit weight, the prediction accuracy of QP for the current unit can be improved.

The encoder with a rate control algorithm should generate bits to conform to the pre-specified bitrate. To do this, the rate control algorithm calculates the available bits and restricts the amount of available bits at a GOP level, frame level, and CTU level. The algorithm also decides the best QP for the current unit, typically by using the texture complexity or bit weight of a collocated unit in the previously encoded frame. This means that the algorithm should predict the to-be-generated bits for the current unit, by making use of the information from the previously-encoded units or frames. However, the information from the temporally collocated unit may be inaccurate when the unit contains some objects with fast motions between frames.

Figure 2. Overall structure of depth map-based inter-view MV prediction for a coding unit

In the current 3D multi-view video encoding algorithm, the base view is coded independently, but extended views are coded by referencing the reconstructed picture of base view. The picture in the base view and a picture in extended view with the same picture order count (POC) are acquired at the same time. Therefore, the disparities between object locations in these pictures depend only on the object’s distance from the cameras. By using the depth map, we can calculate the disparity of a coding unit between the views, and thereby find the corresponding position in the other view. Since the two pictures with the same POC are highly correlated, the motion vectors are close to each other. Patent rights declaration(s)

# Experimental results of the proposed method

The proposed rate control for multi-view video coding was evaluated under the CBR condition. Table 1 shows the average percentage error in the actual generated bitrate compared to the target bitrate. We show the results for the base view (view 0) and extended views (view 1 and view 2) for two cases: (1) when R-lambda model-based rate control and inter-view MAD prediction are on, and (2) when R-lambda model-based rate control and the proposed algorithms are on. As seen in the table, the proposed method shows better bitrate accuracy for 0.3% and 0.31% compared to inter-view MAD prediction.

Table 1. Percentage error in the actual bitrate (%) of the proposed method compared to the R-lambda model-based rate control with inter-view MAD prediction

| Sequence | R-lambda model-based rate control + inter-view MAD prediction |  |  |  | R-lambda model-based rate control + the proposed methods |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | average view0 | average view1 | average view2 | total | average view0 | average view1 | average view2 | total |
| Balloons | 1.51 | 1.47 | 1.80 | 1.58 | 1.51 | 0.78 | 1.00 | 1.30 |
| Kendo | 0.17 | 0.98 | 1.06 | 0.42 | 0.17 | 0.53 | 0.62 | 0.28 |
| Newspaper | 3.32 | 1.22 | 1.77 | 2.67 | 3.32 | 1.22 | 1.77 | 2.67 |
| GTFly | 0.89 | 0.71 | 1.01 | 0.86 | 0.89 | 0.71 | 1.01 | 0.86 |
| Poznanhall2 | 3.86 | 1.53 | 2.11 | 3.10 | 3.86 | 1.53 | 2.11 | 3.10 |
| Poznanstreet | 6.42 | 4.68 | 4.72 | 5.96 | 6.42 | 4.01 | 4.06 | 5.75 |
| Dancer | 0.93 | 1.36 | 0.94 | 0.95 | 0.93 | 1.09 | 0.70 | 0.88 |
| Average | 2.44 | 1.71 | 1.92 | 2.22 | 2.44 | 1.41 | 1.61 | 2.12 |

Table 2 shows the BDrate performance of the proposed algorithm. By using the proposed algorithm, 0.56% and 0.45% bitrate reduction were achieved for each extended view with R-lambda model-based rate control with inter-view MV-based rate prediction.

Table 2. BDrate of the proposed algorithms compared to the R-lambda model-based rate control with inter-view MAD prediction

| Sequence | Bdrate of R-lambda with inter-view MAD prediction vs. R-lambda + the proposed |  |
| --- | --- | --- |
|  | average view1 | average view2 |
| Balloons | -0.42 | -0.17 |
| Kendo | 2.35 | 2.44 |
| Newspaper | 0.62 | 0.36 |
| GTFly | -0.66 | -0.44 |
| Poznanhall2 | -4.95 | -4.64 |
| Poznanstreet | -1.57 | -1.62 |
| Dancer | 0.69 | 0.93 |
| Average | -0.56 | -0.45 |

# Conclusion

We proposed a rate control algorithm for 3D multi-view video coding, inter-view MV-based rate prediction. The proposed algorithm is implemented on HTM8.2 to evaluate the rate control performance. For extended views, the proposed algorithm references the texture complexity or bitrate weight from the corresponding temporal region based on the motion vector derived by the disparity-compensated region in the base view to consider the motion, instead of the collocated unit. The proposed algorithm achieves higher bitrate accuracy and coding performance compared to the previous inter-view MAD prediction. Hence, we recommend the proposed method to be adopted to the current rate control scheme for multi-view video coding.

# References

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Rate control based on unified RQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H0213 (m23088), San José, CA, USA, Feb. 2012.

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Improvement of the rate control based on pixel-based URQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0094 (m24333), Geneva, Switzerland, Apr.-May 2012.

- B. Li, H. Li, L. Li and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0103, Shanghai, China, Oct. 2012.

- W. Lim, H. Choi, J. Nam, H. Jo, D. Sim and I. Bajić, “Rate control for 3D multi-view video coding,” JCT3V of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT3V-A0151, Stockholm, Sweden, Jul. 2012.

- W. Lim, H. Choi, J. Nam, H. Jo, D. Sim and I. Bajić, “Improvement of the rate control for 3D multi-view video coding,” JCT3V of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT3V-B0030, Shanghai, China, Oct. 2012.

- W. Lim, D. Sim and I. Bajić, “Improvement of the rate control for 3D multi-view video coding,” JCT3V of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT3V-C0090, Geneva, CH, Jan. 2013.

- W. Lim, H. Jo, D. Sim and I. Bajić, “The rate control schemes for 3D multi-view video coding,” JCT3V of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT3V-D0111, Incheon, KR, Apr. 2013.

- W. Lim, H. Jo, I. Bajić and D. Sim, “Inter-view MAD prediction for rate control of 3D multi-view video coding,” JCT3V of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT3V-E0227, Vienna, AT, Jul. 2013.

# Patent rights declaration(s)

KWU does not have any current or pending patent rights relating to the technology described in this contribution.