| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0169 |
| --- | --- |

| Title: | MV-HEVC HLS: depth representation information SEI message for auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the depth representation information SEI message for auxiliary pictures. The contribution is technically identical to the SEI message option presented in JCT3V-E0102. The JCT-3V decision on JCT3V-E0102 was, quoting JCT3V-E_Notes_d8.doc: “Decision: Adopt (and emphasize need further coordination with JCT-VC on auxiliary picture structure and constraints)”.

# Introduction

JCT-VC agreed to take contribution JCTVC-N0063/JCT3V-E0049 “REXT/MV-HEVC/SHVC HLS: Auxiliary picture layers” as a starting point for strong consideration and further development in AHGs on SHVC HLS and RExt. In the Incheon meeting MPEG Requirements met jointly with JCT-VC and JCT-3V and concluded that the same mechanism is intended to be used for both alpha channel and depth map sequences. This position was reinforced in the Vienna meeting (e.g. MPEG N13767 “Disposition of Comments on ISO/IEC 23008-2:201x/PDAM2”).

Contribution JCTVC-O0041/JVT3V-F0031 revises the above-mentioned JCTVC-N0063/JCT3V-E0049. Among other things, JCTVC-O0041/JVT3V-F0031 proposes that each type of auxiliary picture may have a different SEI message to indicate characteristics of the auxiliary pictures. A separate SEI message per each auxiliary picture type is justified, as the characteristics of the auxiliary pictures are specific to the type – for example, alpha planes have different parameters than depth maps. The SEI message approach is justified, as the characteristics of auxiliary pictures do not affect the decoding process. Moreover, the SEI message approach allows changing the characteristics at any picture. For example, in some sequences it is desirable to change depth range parameters picture by picture.

This contribution proposes the depth representation information SEI message for auxiliary pictures. The contribution is technically identical to the SEI message option presented in JCT3V-E0102. The JCT-3V decision on JCT3V-E0102 was, quoting JCT3V-E_Notes_d8.doc: “Decision: Adopt (and emphasize need further coordination with JCT-VC on auxiliary picture structure and constraints)”.

# Background: relation to depth representation information SEI message of MVC+D

The motivation to use the depth representation information SEI message of MVC+D as a basis for the proposal was that the SEI message is used for a similar purpose in MVC+D and hence apparently contains all the necessary information. Furthermore, the syntax of the SEI message was followed as much as possible.

The main differences compared to the proposed syntax and the depth representation information SEI message of MVC+D can be summarized as follows:

- The SEI message documents the properties of all depth views in the bitstream. The proposed syntax structure is specified in a manner that it documents the properties of a single view/picture. The scalable nesting SEI message can include the proposed SEI message and indicate which auxiliary layers it applies to.

- The Z-axis reference view was not included in the proposed syntax and the units for the Z values are not specified, because these values are meaningful only relative to extrinsic camera parameters and at the moment there is no mechanism to indicate the extrinsic camera parameters in MV-HEVC.

# Proposed specification text

| depth_representation_info_sei( payloadSize ) { | Descriptor |
| --- | --- |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( d_min_flag ¦ ¦ d_max_flag ) |  |
| disparity_ref_view_id | ue(v) |
| if( z_near_flag ) |  |
| depth_ext_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| depth_ext_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| depth_ext_element( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| depth_ext_element( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| } |  |
| } |  |

| depth_ext_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table XXX. In Table XXX, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

- Table XXX – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to 255, inclusive. |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to 255, inclusive. |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to 255, inclusive. |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to 255, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

The variables in the x column of Table YYY are derived as follows from the respective variables in the s, e, n, and v columns of Table YYY as follows.

–	If 0 < e < 127, x = ( −1 )s * 2e−31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x = ( −1 )s * 2−( 30+v ) * n.

NOTE – The above specification is similar to that found in IEC 60559.

- Table YYY – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary picture.

The units for the ZNear and ZFar values, if present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 + 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] specifies the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity.

NOTE – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. Variable DepthLUT[ i ], as specified below, is used to transform coded depth sample values from nonlinear representation to the linear representation – disparity normalized in range from 0 to 255. The shape of this transform is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

Variable DepthLUT[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

depth_nonlinear_representation_model[ 0 ] = 0
depth_nonlinear_representation_model[depth_nonlinear_representation_num_minus1 + 2 ] = 0
for( k=0; k<= depth_nonlinear_representation_num_minus1 + 1; ++k ) {
	pos1 = ( 255 * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 ) )
	dev2 = depth_nonlinear_representation_model[ k+1 ]

	x1 = pos1 − dev1
	y1 = pos1 + dev1
	x2 = pos2 − dev2
	y2 = pos2 + dev2

	for ( x = max( x1, 0 ); x <= min( x2, 255 ); ++x )
			DepthLUT[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to 255, inclusive, represents disparity that is uniformly quantized into the range of 0 to 255, inclusive.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).