| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0171 |
| --- | --- |

| Title: | CE5 related: Fix for DMM/RBC reference sample filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Karsten Müller | Email: | philipp.merkle@hhi.fraunhofer.de, karsten.mueller@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes aligning the 3D-HEVC specification and fixing the 3D-HTM software in terms of using the unfiltered reference samples for segment DC value prediction of DMM and RBC.

# Introduction

First, it is not specified in current 3D-HEVC that the unfiltered reference samples are used for predicting segment DC values of DMM and RBC blocks.

Second, the current 3D-HTM reference software contains an inconsistency for RBC, using unfiltered reference samples in encoder search of segment DC offsets, but filtered reference samples in the prediction process.

# Proposed method

## Draft specification alignment

Specifying the use of unfiltered reference samples for DMM and RBC requires changing the specification in section H.8.4.4.2.1 of Annex H of JCT3V-E1001 [3] as follows:

… Depending on the value of predModeIntra, the following ordered steps apply: - When cIdx is equal to 0 and predModeIntra is in the range of 0 to 34, the filtering process of neighbouring samples specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nTbS as inputs, and the output is reassigned to the sample array p. - The intra sample prediction process according to predModeIntra applies as follows: …

## Software fix

For RBC, the segment DC prediction is changed such that the unfiltered instead of the filtered reference samples are used for prediction of segment DC values. This fixes the inconsistency of using different reference samples for encoder search and prediction for the RBC mode. Consequently, only unfiltered reference samples are used for all DMM and RBC prediction, which reduces the complexity of these modes as the filtering process does not need to be carried out (see section 2.1).

# Experimental results

According to the CE5 description in JCT3V-E1105 [1] and the common test conditions in JCT3V-E1100 [2], the fixed software HTM-8.0 is evaluated for random access (CTC) and all-intra configuration.

Table 1. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.03% | 102.1% | 103.3% | 101.7% |
| Kendo | 0.0% | 0.0% | -0.01% | 101.5% | 106.3% | 100.8% |
| Newspapercc | 0.0% | 0.0% | 0.01% | 100.3% | 106.3% | 99.9% |
| GhostTownFly | 0.0% | 0.0% | -0.01% | 100.3% | 100.5% | 99.9% |
| PoznanHall2 | 0.0% | 0.0% | -0.05% | 99.0% | 99.0% | 99.5% |
| PoznanStreet | 0.0% | 0.0% | -0.04% | 100.2% | 99.8% | 99.3% |
| UndoDancer | 0.0% | 0.0% | -0.02% | 100.1% | 102.0% | 100.3% |
| 1024x768 | 0.0% | 0.0% | -0.01% | 101.3% | 105.3% | 100.8% |
| 1920x1088 | 0.0% | 0.0% | -0.03% | 99.9% | 100.3% | 99.7% |
| average | 0.0% | 0.0% | -0.02% | 100.5% | 102.5% | 100.2% |
| Shark | 0.0% | 0.0% | -0.02% | 99.8% | 109.0% | 99.2% |

Table 2. Result for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.01% | 99.8% | 100.7% | 100.1% |
| Kendo | 0.0% | 0.0% | -0.01% | 99.7% | 97.6% | 98.7% |
| Newspapercc | 0.0% | 0.0% | -0.03% | 99.3% | 101.5% | 99.5% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 99.4% | 97.6% | 98.4% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | 100.1% | 106.7% | 101.2% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 99.9% | 102.7% | 100.2% |
| UndoDancer | 0.0% | 0.0% | -0.01% | 100.0% | 100.7% | 98.8% |
| 1024x768 | 0.0% | 0.0% | -0.02% | 99.6% | 99.9% | 99.5% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 99.9% | 101.9% | 99.6% |
| average | 0.0% | 0.0% | -0.01% | 99.8% | 101.1% | 99.6% |
| Shark | 0.0% | 0.0% | -0.01% | 99.5% | 99.6% | 97.3% |

# Cross check

The cross check of the proposed alignment was done by Qualcomm. They did not find any issues with the source code and the simulation results perfectly matched.

# Conclusion

The results show that the proposed alignment has no negative impact on the coding performance and should therefore be adopted to the 3D-HEVC specification and the 3D-HTM reference software.

# References

- F. Jäger, " Description of Core Experiment 5 (CE5) on Depth Intra Modes", JCT3V-E1105, Vienna, Austria, August 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-E1100, Vienna, Austria, August 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” JCT3V-E1001, Vienna, Austria, August 2013.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).