| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0172_r1 |
| --- | --- |

| Title: | AHG12: MVC+D reference software and conformance bitstreams update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Usman Hakeem Andrey Norkin Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution proposes to update the MVC+D reference software with implementation of 3D reference displays information SEI (a part of MVC+D). The contribution also proposes to include a conformance bitstream that uses the reference display information SEI message. The generated bitstream and the reference software implementation are based on 3D-ATMv9.0.

# Introduction

The 3D reference displays SEI message was adopted to MVC+D at the JCT-3V meeting in Stockholm [1]. This contribution proposes to update the MVC+D reference software with the 3D reference displays information SEI implementation based on 3D-ATMv9.0 [2]. It is also proposed to include a conformance bitstream that uses the 3D reference displays information SEI into the conformance bitstreams set [3].

# Modifications to 3D-ATM software and its usage

It is proposed to update the MVC+D reference software with the implementation of the 3D reference display SEI message (which is a part of MVC+D since the Stockholm meeting). The SEI message is implemented on top of 3D-ATMv9.0 and the software implementation is attached to the proposal.

The changes made to the code are present under REF_DISP_SEI_A0163 define statement in defines.h file. A new configuration file “3dv_reference_display.cfg” is added to 3D-ATM, which contains the reference display parameters, such as precision for reference baseline, display width and viewing distance  as well as the values for these parameters. The parameters can be specified separately for a number of displays. The reference display configuration file name should be specified in the encoder config file with the “SEI_ReferenceDispayFile” parameter. The decoder is able to decode the reference displays SEI message and save its content into the specified file which can be specified in decoder.cfg.

# Conformance bistream generation

It is proposed to include a conformance bitstream that uses a 3D reference displays SEI message in order to facilitate implementation of the SEI message by video codecs developers. It is also proposed to include the printout of the SEI message values together with the conform bitstream.

The conformance bitstream is generated with 3D-ATMv9.0 for the Balloons sequence (1024x768@30hz) at qp 30 for texture and depth. Three texture and 3 depth maps are coded for balloons sequence containing 17  frames. The conformance bistream is using multiview high profile with post dilation filter turned off using common test conditions. For the provided conformance bitstream, the reference display sei is sent with first IDR picture along with the multiview aquisition SEI. The conformance bistream along with its md5 checksum and the SEI message output can be uploaded to http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/MVCD/.

# Conclusions

It is proposed to update the MVC+D reference software with the implementation of the 3D reference displays SEI message adopted to MVC+D at the JCT-3V meeting in Stockholm [1].

It is also proposed to add a conformance bitstream for MVC+D that contains the 3D reference displays SEI message and the text output of the SEI message content.

# References

- A. Norkin, I. Girdzijauskas, Y. Zhao, L. Yu, “Show-case and syntax for SEI message on reference display information signaling”, JCT3V-A0163, Stockholm, Sweden, July 2012.

- http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv9.0

- T. Suzuki, D. Rusanovskyy, “Draft 4 of MVC plus depth conformance”, JCT3V-E1006, Vienna, August. 2013

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).