| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. –1 Nov. 2013 | Document: JCT3V-F0173 |
| --- | --- |

| Title: | CE1.h related: View Synthesis Prediction with DV difference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Lu Yu | Tel: Email: | +86 571 87953107 felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution describes design of view synthesis prediction (VSP) with disparity vector (DV) difference. A DV difference, i.e., deltaDV, is introduced to current design of VSP to compensate the DV that used to retrieve the virtual depth block. The overall bitrate savings on the coded and synthesized views are -0.x% compared to HTM-8.0 and the decoding time is 99.6% for all sequences on average. The maximum bitrate savings of texture on dependent left and right views are -x.x% and -x.x%, respectively.

# Introduction

In the current design of VSP in 3D-HEVC, the virtual depth block is located by the DV inherited from neighboring blocks. Then the virtual depth block is used to convert DVs to find the corresponding blocks (4x8 or 8x4) in reference view. However, the inherited DV might not be the best DV for the VSP module. So a DV difference is introduced to compensate the inherited DV to provide a better virtual depth block for VSP module.

# DV Difference for VSP modes

In this proposal the DV used to locate the virtual depth block, i.e., VspDV, is recalculated by:

VspDV = inheritedDV + deltaDV

where the inheritedDV (as shown in Fig. 1) is the DV inherited from neighboring blocks and the deltaDV (as shown in Fig. 1) is the newly introduced DV difference.

The the refined VspDV, a refined depth block is located and use as the virtual depth block for the current texture block.

The deltaDV is searched on the encoder side within a smaller window than the normal motion estimation (ME) and transmitted in the bitstream.

Fig 1. Illustration of the application of deltaDV in VSP

# Simulation results and discussions

The proposed method was implemented into HTM 8.0. Simulations are conducted according to the common test conditions as defined in [1].

Table 1 shows the performance of the proposed method against HTM8.0. The total BD-rate gains are:

- 0.x% for video PSNR vs. video bitrate

- 0.x% for video PSNR vs. total bitrate

- 0.x% for synthesis PSNR vs. total bitrate

The maximum BD-rate gains for synthesis PSNR vs. total bitrate is x.x%.

The decoding complexity of proposed method is xx.xx% of the anchor.

Table 1. Performance of proposed method compared to HTM8.0 anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons |  |  |  |  |  |  |  |  |
| Kendo |  |  |  |  |  |  |  |  |
| Newspaper_CC |  |  |  |  |  |  |  |  |
| GT_Fly |  |  |  |  |  |  |  |  |
| Poznan_Hall2 |  |  |  |  |  |  |  |  |
| Poznan_Street |  |  |  |  |  |  |  |  |
| Undo_Dancer |  |  |  |  |  |  |  |  |
| 1024x768 |  |  |  |  |  |  |  |  |
| 1920x1088 |  |  |  |  |  |  |  |  |
| average |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, a DV difference is introduced to the current design of VSP framework. Better synthesis images are generated by a relocated virtual depth block. It achieves -0.x% gain for synthesis PSNR vs. total bitrates.

# References

- D. Rusanovskyy, K. Muller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-E1100, Vienna, AT, July. 2013.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).