| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, Switzerland, 25 Oct –1 Nov. 2013 | Document: JCT3V- F0174 |
| --- | --- |

| Title: | 3D-CE3.h related: On Split Flag Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen1 Jianhua Zheng2 Fan Liang1 | Tel: Email: | +86-10-60612141 zhengjianhua@hisilicon.com chhuanb@mail2.sysu.edu.cn |
| Source: | Sun Yat-sen University1, HiSilicon Technologies2 |  |  |

_____________________________

# Abstract

This contribution provided a strategy to reduce the split flag signalling for dependent texture views. Based on JCT3V-E0173, the proposal introduces a modified CU splitting termination strategy in both encoder and decoder for split_cu_flag prediction. Experimental results show that the proposed method can effective to reduce split flag signalling overhead and achieves 0.36% , 0.28% and 0.15% averaging BD-rate coding gains for texture views, overall video and synthesized views under common test condition, respectively.

# Introduction

JCT3V-E0173 proposes an early CU splitting termination algorithm for dependent texture views. When the CU splitting depth of the current CU is equal to or larger than the maximum depth of the CU splitting depths of its inter-view corresponding blocks and Skip mode is selected as the best prediction mode for the current CU, the splitting progress is terminated. In this proposal, we modified the terminated conditions to make it available for the decoder.

# Proposed method

## Split flag prediction

In this contribution, we propose a method to predict split_cu_flag by exploiting the inter-view correlation as depicted in Fig. 1. The following conditions are applied in both encoder and decoder.

Condition 2.2.1

All the five inter-view neighbouring blocks (the corresponding block of the current CU and the four direct adjacent blocks of the corresponding block in the reference view as shown in Fig. 1) are coded as merge modes.

Condition 2.2.2

The CU splitting depth of the current CU is equal to or larger than the maximum depth of the CU splitting depths of the five inter-view neighboring blocks.

Fig. 1.  The five inter-view neighboring blocks of the current CU

## Split flag signaling and Parsing

In encoder, if both conditions 2.2.1 and 2.2.2 are true, no further CU splitting is need for the current CU, hence split_cu_flag need not be signaled in bitstream, else split_cu_flag is still be signaled in bitstream.

In decoder, if split_cu_flag is not present, then examine the same conditions 2.2.1 and 2.2.2. If both conditions 2.2.1 and 2.2.2 are true, it is decided no further CU splitting for the current CU.

## Specification Changes

- H.7.4.9.4  Coding quadtree semantics

The specifications in subclause 7.4.9.4 apply.

The variable predSplitCuFlag specifying whether the split_cu_flag is predicted by inter-component prediction is derived as specified in the following.

  - Derivate the bVFMerge flag and iVFDepth ;

  - bIVFMerge is true when all the five inter-view neighbouring blocks are coded as merge mode; iVFDepth specifies the max depth of the above-mentioned corresponding blocks.

  - If slice_type is not equal to I , current slice is dependent texture view, bVFMerge is true and ctDpeth is not less than iVFDepth, predSplitCuFlag is set equal to 1;

  - Else If slice_type is not equal to I, RapPicFlag is equal to 0, and lim_qt_pred_flag[ nuh_layer_id ] is equal to 1, predSplitCuFlag is set equal to ( TextureCtDepth[ x0 ][ y0 ]  <=  ctDepth )

  - Otherwise (slice_type is equal to I or RapPicFlag is equal to 1 or lim_qt_pred_flag[ nuh_layer_id ] is equal to 0), predSplitCuFlag is set equal to 0.

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than MinCbLog2SizeY and predSplitCuFlag is equal to 0 , the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	Otherwise (log2CbSize is equal to MinCbLog2SizeY or predSplitCuFlag is equal to 1), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

# Experimental results

The proposed method is implemented in HTM8.0. The coding efficiency is evaluated in random access (CTC) configurations. Table 1 shows the results. The results show the proposed method achieves overall 0.36%, 0.28% and 0.15% BD-rate coding gains for texture views, overall video and synthesized views, respectively.

Table 1. Experimental results of prediction of split flag under CTC condition

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.7% | -0.5% | -0.4% | -0.4% | -0.2% | 114.8% | 97.5% | 96.1% |
| Kendo | 0.0% | -0.7% | -0.5% | -0.4% | -0.5% | -0.2% | 111.9% | 90.2% | 100.0% |
| Newspaper_CC | 0.0% | -0.4% | -0.3% | -0.2% | -0.2% | -0.1% | 104.0% | 95.6% | 98.0% |
| GT_Fly | 0.0% | 0.2% | 0.1% | -0.2% | -0.2% | 0.0% | 113.1% | 82.5% | 94.5% |
| Poznan_Hall2 | 0.0% | -0.7% | -0.8% | -0.4% | -0.4% | -0.3% | 106.1% | 95.1% | 100.6% |
| Poznan_Street | 0.0% | -0.5% | -0.7% | -0.3% | -0.3% | -0.2% | 105.3% | 88.4% | 94.3% |
| Undo_Dancer | 0.0% | 0.1% | 0.2% | -0.1% | -0.1% | 0.1% | 108.1% | 95.5% | 100.6% |
| 1024x768 | 0.0% | -0.6% | -0.4% | -0.3% | -0.4% | -0.2% | 110.2% | 94.4% | 98.0% |
| 1920x1088 | 0.0% | -0.2% | -0.3% | -0.2% | -0.2% | -0.1% | 108.2% | 90.4% | 97.5% |
| average | 0.00% | -0.36% | -0.36% | -0.28% | -0.28% | -0.15% | 109.1% | 92.1% | 97.7% |
| Shark | 0.00% | 0.10% | 0.33% | -0.11% | -0.10% | -0.02% | 110.8% | 101.2% | 94.8% |

# Conclusion

This contribution proposes a simple method to predict split flag and reduce split flag signalling overhead for dependent texture views. The proposed method can achieve overall 0.36%, 0.28% and 0.15% BD-rate coding gains for texture views, overall video and synthesized views, respectively.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies and Sun Yat-sen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-8.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- Na Zhang, Yi-Wen Chen, “3D-CE3.h related: Fast encoder decision for texture coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E0173, August, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, August, 2013.