| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0176 |
| --- | --- |

| Title: | CE1-related: Motion parameters stored at VSP-coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, JAPAN | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

The current WD specifies that disparity parameters utilized to fetch virtual depth map is stored in the motion storage for the blocks where view synthesis prediction is applied. However, these disparity parameters are different from the disparity parameters which are actually utilized on generating prediction signals. This contribution proposes to store the finally used disparity parameters for each sub-PU. Experiments reportedly show that about 0.1% bitrate reduction for both coded and synthesized views without significant increase of decoder complexity.

# Introduction

The procedure of view synthesis prediction (VSP) is specified as two-step disparity compensation in the current WD [1]. Figure 1 shows the current VSP design. The first disparity compensation is PU-based processing to generate virtual depth map for the current PU. The second disparity compensation is sub-PU-wise processing to generate actual predictor for the coding target picture.

Figure 1: View synthesis prediction in 3D-HEVC

In the current WD, the first disparity parameter is stored in the motion storage at the blocks where VSP is applied. This contribution proposes to store the second disparity parameters in the motion storage.

# Proposal

The proposed method is to store the second disparity parameters, which are actually utilized for prediction, in the motion storage instead of the first disparity parameters, which are utilized to fetch virtual depth map as shown in Figure 2. The stored disparity parameters are subjected to the other processes including de-blocking filter, AMVP, TMVP, and NBDV derivation as specified in the current WD.

Figure 2: Disparity parameters stored in motion memory

# Experiments

The proposed methods are built on the top of the reference software 3DV-HTM v8.0. And the experiments are conducted according to the common test condition [2].

TABLE 1 shows the simulation results by the proposed method. As it can be seen, the proposed method achieves about 0.1% bitrate reduction on average for both coded and synthesized views.

Since the proposed change is only the selection of disparity parameter to be stored and the disparity parameters proposed to be stored are already derived in the current process, the impact on the decoder complexity might be negligible, and the proposed method might be more reasonable in terms of the concept of motion storage.

TABLE 1: Simulation results

# Conclusion

In this document, it is proposed to store disparity parameters utilized to generate prediction signals for VSP coded blocks. Although the improvement is minor, but the proposed method is better aligned to the concept of the other stored motion parameters and there is no significant decoder complexity increase. We recommend adopting the proposed method.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “3D-HEVC Draft Text 1”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E1001, September 2013, Vienna, Austria.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E1100, August 2013, Vienna, Austria.