| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0177r1 |
| --- | --- |

| Title: | CE1-related: View synthesis prediction via motion field synthesis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, JAPAN | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution proposes view synthesis prediction (VSP) based on inter-view motion field synthesis. While the current VSP directly synthesized prediction signals using disparity information derived from depth map, the proposed VSP first synthesizes motion vector field using depth map and then generates prediction signals based on the synthesized motion vector field. Experiments reportedly show 0.6% and 0.4% bitrate reduction on average for the coded and synthesized views, respectively. The maximum bitrate reduction, 11% for each dependent view and 1.6% for the synthesized views, is achieved on GT_Fly sequence.

# Introduction

View synthesis prediction (VSP) is one of the 3D-HEVC coding tools which utilize depth map. The current VSP directly generates prediction signals by synthesizing views using depth map [1]. This means that the current VSP utilize only inter-view correlations. However, multiview video contains both temporal and inter-view correlations.

This contribution proposes a novel VSP which utilize both temporal and inter-view correlations by simplifying the method proposed in JCT3V-E0246.

# Proposal

The proposed method consists of three steps. The first step is generation of depth map; this step is the same as the first step of the current VSP. The second step is motion vector field synthesis by utilizing inter-view correlations. Instead of image signals, motion parameters are synthesized for each sub-PU based on disparity vector derived from depth map as shown in Figure 1. The third step is prediction signal generation by sub-PU-wise motion compensated prediction using the synthesized motion parameter field.

Figure 1: Proposed VSP via motion field synthesis.

# Experiments

The proposed methods are built on the top of the reference software 3DV-HTM v8.0. And the experiments are conducted according to the common test condition [2]. Simulations were conducted with two sub-PU sizes, 4x4 and 8x8.

TABLE 1 shows the simulation results by the proposed approaches. As it can be seen, the proposed VSP with 4x4 sub-PU achieves about 0.6% and 0.4% bitrate reductions for coded and synthesized views, respectively.

TABLE 1: Simulation results

- HTM v8.0 vs. Proposed VSP (4x4 sub-PU)

- HTM v8.0 vs. Proposed VSP (8x8 sub-PU)

# Conclusion

In this document, view synthesis prediction based on inter-view motion field synthesis is proposed. Experiments show good gain although there is significant impact on decoder runtime. We recommend further study in CE.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “3D-HEVC Draft Text 1”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E1001, September 2013, Vienna, Austria.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E1100, August 2013, Vienna, Austria.