| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0189 |
| --- | --- |

| Title: | 3D-CE4.h related: Explicit disparity vector oriented inter-view motion prediction and residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shiori Sugimoto*, Shinya Shimizu (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2866 sugimoto.shiori@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

3D-HEVC supports inter-view motion prediction and advanced residual prediction using a derived disparity vector, where the disparity vector is derived from neighbor blocks. This contribution proposes an explicit disparity vector oriented inter-view motion prediction and advance residual prediction. The experimental results show that the proposed method can achieve 0.8% BD-rate saving for video 1 and video 2, and 0.3% and 0.2% BD-rate saving for coded and synthesized views with no complexity increase.

# Introduction

The inter-view motion prediction and the advanced residual prediction (ARP) in the current test model is illustrated in Figure 1[1].

Figure 1

The disparity vector (DV) to locate the corresponding block in the inter-view reference picture is derived from neighboring blocks used for the both of methods. Therefore, its precision depends on the precision of the derived DV.

Figure 2

# Proposal

In this contribution, it is proposed to code the DV which is used for inter-view motion prediction and APR. Figure 2 shows the comparison of the combination of inter-view motion prediction and ARP in the current test model and the proposed method.

The proposed method can be implemented as the extension of ARP in the current test model in the case that the reference picture for motion compensated prediction (MCP) is the inter-view reference picture. In the current test model, a flag for ARP is coded but ARP is never performed in such a case. Therefore, the proposed tool never conflict with any existing tools.

The proposed method generates the prediction signals by motion compensated prediction (MCP) instead of disparity compensation (DCP) on condition that ARP flag is flagged. The motion parameters for MCP are derived by inter-view motion prediction using the explicit DV. It also updates the prediction signals by ARP using the derived motion parameters and the explicit DV.

The prediction signals is generated as follows.

- The corresponding block in the inter-view reference picture is located using the explicit DV.

- If the corresponding block is coded by MCP, the motion parameters are copied from coded motion parameters and the prediction signals is generated by MCP and ARP. If not, the prediction signal is generated by DCP.

  - If the current PU is coded by bi-prediction, a single set of motion parameters are derived from one of reference picture lists used in the corresponding block. The priority is L0, but it has no parameters in L0, the parameters in L1 are used. Then, the prediction signal is generated by bi-prediction of MCP+ARP and other prediction.

  - If not, two set of motion parameters are derived from both of reference picture lists used in the corresponding block. Then, the prediction signal is generated by bi-prediction of MCP+ARP. If the corresponding block is coded by uni-prediction, the prediction signal is generated by uni-prediction of MCP+ARP.

# Experimental results

The proposed methods are built on the top of the reference software 3DV-HTM v8.1. And the experiments are conducted according to the common test condition [2].

Table 1 shows the simulation results by the proposed explicit DV-oriented inter-view motion prediction and ARP. It can be seen that the proposed method can achieve some improvements with no complexity increase.

Table 1

# Conclusion

This contribution proposes an explicit DV-oriented inter-view motion prediction and ARP. The experimental results show the average bitrate reduction of 0.3% and 0.2% for coded and synthesized views, respectively. Although the current improvement is minor, but it causes no complexity increase. It is suggested to adopt this method to the next version of HTM.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, Aug, 2013.