| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0190 |
| --- | --- |

| Title: | 3D-CE4.h related: Motion vector oriented disparity vector derivation for residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shiori Sugimoto*, Shinya Shimizu (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2866 sugimoto.shiori@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

3D-HEVC supports advanced residual prediction using a derived disparity vector, where the disparity vector is derived from neighbor blocks. This contribution proposes a motion vector oriented disparity derivation from the temporal reference picture for advance residual prediction. The experimental results show that the proposed method can achieve 0.1% BD-rate saving for coded and synthesized views with no complexity increase.

# Introduction

The advanced residual prediction (ARP) in the current test model is illustrated in Figure 1[1]. The disparity vector (DV) to locate the corresponding block in the inter-view reference picture is derived from neighboring blocks used for this method. Therefore, its precision depends on the precision of the derived DV.

# Proposal

In this contribution, it is proposed to derive DV which is used for APR from coded temporal reference picture. The DV is derived from the motion parameters in the coded reference block located by motion parameters. The motion parameters are also used in motion compensation prediction (MCP). Figure 1 also shows the proposed method.

The proposed method can be used for ARP as one of disparity vector derivation. The prediction signals are generated as follows.

- The reference block in the temporal reference picture is located using the motion parameters.

- If the reference block is coded by DCP, the disparity parameters are copied from coded disparity parameters. And the prediction signal is generated by MCP and ARP using the DV derived from the reference block. If the reference block is coded by bi-prediction of DCP, the disparity parameters in list L0 are selected.

  - If the reference block is coded by MCP, same process is called recursively until the block coded by DCP is found. If the current reference block is coded bi-prediction, the motion parameters in list L0 are selected. If no DCP blocks are found at last, the prediction signal is generated by MCP and ARP using the DV derived using other derivation method in current test model.

Figure 1

# Experimental results

The proposed methods are built on the top of the reference software 3DV-HTM v8.1. And the experiments are conducted according to the common test condition [2].

Table 1 shows the simulation results by the proposed Motion vector oriented disparity vector derivation for residual prediction. It can be seen that the proposed method can achieve some improvements with no complexity increase.

Table 1

# Conclusion

This contribution proposes a Motion vector oriented disparity vector derivation for residual prediction . The experimental results show the average bitrate reduction of 0.1% for coded and synthesized views, respectively. Although the current improvement is minor, but it causes no complexity increase. It is suggested to adopt this method to the next version of HTM.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, Aug, 2013.