| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0213 |
| --- | --- |

| Title: | Cross-check of Illumination Compensation for View Synthesis Prediction Candidate (JCT3V-F0146) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of VSP prediction on VSP merge candidate with ic_flag being equal to 1. In current 3D-HEVC, the ic_flag, in the VSP merge candidate, is useless since the predictor is identical regardless of ic_flag in the case of VSP merge candidate.  In the test method, conventional DCP prediction and illumination compensation is performed in the case of VSP merge candidate with ic_flag being equal to 1 to differentiate it from the the case with ic_flag being equal to 0. The experiment results perfectly match with those provided by the proponent. The software was also reviewed and verified that it follows the description. It is reported that the BD-rage gain on average is 0.1%, 0.1% and 0.1% in texture, video and synthesis respectively.

- Note: VSP prediction and illumination compensation is not performed simultaneously so that there is no additional complexity in terms of prediction.

# Introduction

Table 1. Test method

| Condition |  | HTM80 | Test (JCT3V-F0146) |
| --- | --- | --- | --- |
| vsp_flag == 0 | ic_flag == 0 | DCP | DCP (same as HTM80) |
| vsp_flag == 0 | ic_flag == 1 | DCP + IC | DCP + IC (same as HTM80) |
| vsp_flag == 1 | ic_flag == 0 | VSP | VSP (same as HTM80) |
| vsp_flag == 1 | ic_flag == 1 | VSP | DCP+IC |

# Experimental results

Table 2. Test results (anchor: HTM8.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.5% | -0.2% | -0.2% | -0.1% | -0.1% | 100.2% | 101.1% | 101.9% |
| Kendo | 0.0% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% | 100.2% | 98.6% | 100.4% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.5% | 105.2% | 101.9% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 99.2% | 99.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.6% | 99.1% | 99.7% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 98.4% | 99.8% |
| Undo_Dancer | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 99.9% | 101.1% | 100.1% |
| 1024x768 | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 100.3% | 101.6% | 101.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 99.5% | 99.7% |
| average | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 100.0% | 100.4% | 100.5% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 89.9% | 98.8% |

# References

[1] M. W. Park, J. Y. Lee, C. Kim, “Illumination Compensation for View Synthesis Prediction Candidate for 3D-HEVC” JCT3V-F0146, JCT3V 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).