| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0214 |
| --- | --- |

| Title: | Cross-check of Removal of IC in depth coding and IC flag signalling (JCT3V-F0151) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of removal of illumination compensation for depth and ic_flag signaling in VPS. The experiment results perfectly match with those provided by the proponent. The software was also reviewed and verified that it follows the description. It is reported that by removing illumination compensation for depth, 0.00 %, 0.05 % and 0.02 % BD-rage gain on average is observed in texture, video and synthesis respectively.

- The other part, or “adding ic_flag in VPS”, is considered to be in accordance to other prediction tools such as VSP, ARP and depth refinement, which also have those gating signallings in VPS. It is also reported that the signaling inclusion doesn’t change coding performance.

# Experimental results

Table 1. Removal of depth IC (anchor: HTM8.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 92.3% | 99.1% | 98.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 91.5% | 97.1% | 97.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 91.6% | 100.6% | 100.7% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 98.8% | 99.4% | 99.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 90.7% | 97.1% | 97.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 91.2% | 96.7% | 97.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 101.0% | 98.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 91.8% | 98.9% | 98.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 95.2% | 98.5% | 98.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 93.8% | 98.7% | 98.5% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 98.3% | 88.8% | 97.0% |

Table 1. Removal of depth IC and ic_flag in VSP (anchor: HTM8.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 92.3% | 99.1% | 98.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 91.5% | 97.1% | 97.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 91.6% | 100.6% | 100.7% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 98.8% | 99.4% | 99.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 90.7% | 97.1% | 97.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 91.2% | 96.7% | 97.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 101.0% | 98.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 91.8% | 98.9% | 98.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 95.2% | 98.5% | 98.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 93.8% | 98.7% | 98.5% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 98.3% | 88.8% | 97.0% |

# References

[1] M. W. Park, J. Y. Lee, C. Kim, “Removal of IC in depth coding and IC flag signalling in 3D-HEVC” JCT3V-F0151, JCT3V 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).