| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0218 |
| --- | --- |

| Title: | Crosscheck of Default Disparity Vector Improvement by Samsung (JCT3V-F0143) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang, Kai Zhang MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang } @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution verified the results of Default Disparity Vector Improvement proposed in JCT3V-F0143 by Samsung.

# Introduction

In order to simplify the NBDV derivation process, it is proposed to remove the DV-MCP process from the derivation process. In addition to this, it is proposed to change the default disparity vector from (0, 0) to the disparity vector corresponding to the depth value 128, which means the middle value of the depth value range, and to utilize the default disparity vector for the residual prediction in order to make up for the coding performance of DV-MCP.

The proposed NBDV derivation process can be illustrated as follows.

- A variable availableDV, which indicates whether DV is found or not, is set to 0.

- For 2 temporal and 2 spatial neighboring blocks (co-located block in co-located picture and RAP picture, and A1 and B1 blocks), if availableDV is equal to 0, find DV from DCP neighboring block, and once DV is found availableDV is set to 1.

- For 2 spatial neighboring blocks (A1 and B1 blocks), if availableDV is equal to 0, find disparity vector from MCP neighboring block, and once DV is found availableDV is set to 1.

- If available DV is equal to 0, DV is set to the disparity vector corresponding to the depth value of 128 by default and availableDV is set to 1.

# Experimental results

In JCT3V-E0124, there are two tests as follows:

- Test #1: under CTC.

- Test #2: Non-CTC, the CTC modifed by setting "ViewSynthesisPred"/"DepthRefinement" to '0'.

The experimental results of test#1 and test#2 have been verified by running the source code provided by the proponent. The RD performance shown in Table 1 and Table 2 matched those provided by the proponent exactly.

Table 1.  The results of the proposed method under CTC

Table 2.  The results of the proposed method under Non-CTC

# Conclusion

This contribution verified the test#1 and test#2 in JCT3V-F0143 proposed by Samsung.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).