| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0237 |
| --- | --- |

| Title: | CE3: Crosscheck on sub-PU level inter-view motion prediction (JCT3V-F0110) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao 5855 Pacific Center Blvd. San Diego, CA 92121 (USA) | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the cross check results of JCT3V-F0110 “3D-CE3: Sub-PU level inter-view motion prediction”, proposed by MediaTek Inc. The coding performance results obtained in this cross-verification match those provided by the proponents.

# Introduction

In JCT3V-F0110 [1], a sub-PU level inter-view motion prediction method is proposed. The proposed method targets at improving the inter-view motion prediction using a smaller sub-PU block size instead of the full PU size. As a cross-checker, we received the source code from MediaTek Inc. The source code is compiled and tested on Linux 64-bit OS with below macro settings,

#define MTK_SPIVMP               1

#define MD5_SEI_FIX              1

With the proponents' source code, a new parameter is added in the encoder configuration to specify the sub-PU size: SubPULog2Size.

Simulations are performed under common test condition (CTC), and three tests are performed depending on the setting of the sub-PU size, as listed below.

Test #1 (sub-PU size equals to 4x4):

SubPULog2Size :               2

Test #2 (sub-PU size equals to 8x8):

SubPULog2Size :               3

Test #3 (sub-PU size equals to 16x16):

SubPULog2Size :               4

The detailed results for the above three tests are listed in below Table 1, Table 2 and Table 3, respectively.

Table 1: Simulation results of Test #1 using 4x4 sub-PU under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -7.9% | -7.9% | -2.6% | -2.3% | -2.1% | 121% | 110% | 122% |
| Kendo | -4.6% | -4.8% | -1.4% | -1.1% | -0.9% | 92% | 111% | 91% |
| Newspapercc | -4.0% | -3.5% | -1.2% | -1.1% | -0.9% | 102% | 113% | 100% |
| GhostTownFly | -12.0% | -12.2% | -2.4% | -2.2% | -1.8% | 90% | 111% | 91% |
| PoznanHall2 | -6.8% | -6.7% | -2.4% | -2.1% | -1.9% | 102% | 111% | 99% |
| PoznanStreet | -4.5% | -4.8% | -1.1% | -1.0% | -1.0% | 86% | 113% | 84% |
| UndoDancer | -4.8% | -5.2% | -1.2% | -1.1% | -1.0% | 119% | 111% | 112% |
| 1024x768 | -5.5% | -5.4% | -1.8% | -1.5% | -1.3% | 105% | 111% | 104% |
| 1920x1088 | -7.0% | -7.2% | -1.8% | -1.6% | -1.4% | 99% | 112% | 97% |
| average | -6.4% | -6.4% | -1.8% | -1.5% | -1.4% | 102% | 112% | 100% |
| Shark | -8.2% | -8.5% | -1.4% | -1.3% | -1.1% | 108% | 111% | 103% |

Table 2: Simulation results of Test #2 using 8x8 sub-PU under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -7.2% | -7.1% | -2.4% | -2.1% | -1.9% | 113% | 105% | 115% |
| Kendo | -4.5% | -4.2% | -1.4% | -1.1% | -0.9% | 83% | 105% | 83% |
| Newspapercc | -3.9% | -3.4% | -1.2% | -1.1% | -0.9% | 88% | 106% | 87% |
| GhostTownFly | -11.1% | -11.5% | -2.2% | -2.0% | -1.7% | 98% | 106% | 99% |
| PoznanHall2 | -6.3% | -6.4% | -2.3% | -2.0% | -1.8% | 114% | 105% | 107% |
| PoznanStreet | -4.2% | -4.6% | -1.0% | -1.0% | -0.9% | 79% | 106% | 78% |
| UndoDancer | -4.2% | -4.6% | -1.0% | -0.9% | -0.9% | 100% | 106% | 99% |
| 1024x768 | -5.2% | -4.9% | -1.7% | -1.4% | -1.2% | 95% | 105% | 95% |
| 1920x1088 | -6.5% | -6.8% | -1.6% | -1.5% | -1.3% | 98% | 106% | 96% |
| average | -5.9% | -6.0% | -1.7% | -1.4% | -1.3% | 97% | 106% | 95% |
| Shark | -7.8% | -8.0% | -1.3% | -1.2% | -1.1% | 91% | 105% | 92% |

Table 3: Simulation results of Test #3 using 16x16 sub-PU under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -5.4% | -5.5% | -1.8% | -1.6% | -1.4% | 116% | 103% | 115% |
| Kendo | -3.1% | -3.0% | -1.0% | -0.8% | -0.6% | 96% | 104% | 96% |
| Newspapercc | -2.3% | -2.0% | -0.7% | -0.6% | -0.5% | 94% | 104% | 92% |
| GhostTownFly | -7.4% | -8.0% | -1.5% | -1.3% | -1.1% | 89% | 104% | 89% |
| PoznanHall2 | -5.2% | -4.9% | -1.8% | -1.6% | -1.5% | 110% | 104% | 106% |
| PoznanStreet | -2.6% | -2.7% | -0.6% | -0.6% | -0.5% | 96% | 105% | 95% |
| UndoDancer | -2.5% | -2.7% | -0.6% | -0.5% | -0.5% | 106% | 104% | 100% |
| 1024x768 | -3.6% | -3.5% | -1.2% | -1.0% | -0.8% | 102% | 104% | 101% |
| 1920x1088 | -4.4% | -4.6% | -1.1% | -1.0% | -0.9% | 100% | 104% | 97% |
| average | -4.1% | -4.1% | -1.1% | -1.0% | -0.9% | 101% | 104% | 99% |
| Shark | -5.0% | -5.2% | -0.8% | -0.7% | -0.7% | 96% | 104% | 95% |

The encoding and rendering time estimates are calculated on the Linux cluster, these estimates may not be accurate due to parallel simulations. The decoding time estimates are calculated on a single PC with Windows 64-bit OS and 6.00 GB RAM. The deviation on the decoder runtime measurements is probably caused by different test environments.

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei, " CE5: Test result on simplification of DMM pattern generation and signaling", JCT3V-F0110, Geneva, CH, 25 Oct. – 1 Nov. 2013.