| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0241 |
| --- | --- |

| Title: | 3D-CE5 related: Cross-check on DMM simplification and signalling (JCT3V-F0147) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Lin Chang +886-3-5670766 ext. 25314 {yulin.chang, yiwen.chen} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution crosschecked the method proposed in JCT3V-F0147 DMM simplification and signaling proposed by Samsung. The algorithm description and simulation results of JCT3V-F0147 are verified.

# Introduction

Three methods are proposed in JCT3V-F0147 [1] to remove DMM3 and RBC.

The first method removed DMM3, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 0 | - | 0 |
| INTRA_DEP_NONE | 10 | 0 | 1 |
| INTRA_DEP_SDC_DMM_WFULL | 110 | - | - |
| INTRA_DEP_DMM_WFULL | 1110 | 10 | - |
| INTRA_DEP_DMM_CPREDTEX | 11110 | - | - |
| INTRA_DEP_CHAIN | 11111 | 11 | - |

The second method removes RBC, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 0 | - | 0 |
| INTRA_DEP_NONE | 10 | 0 | 1 |
| INTRA_DEP_SDC_DMM_WFULL | 110 | - | - |
| INTRA_DEP_DMM_WFULL | 1110 | 10 | - |
| INTRA_DEP_DMM_CPREDTEX | 11110 | - | - |
| INTRA_DEP_DMM_WPREDTEX | 11111 | 11 | - |

The third method removes both DMM3 and RBC, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 0 | - | 0 |
| INTRA_DEP_NONE | 10 | 0 | 1 |
| INTRA_DEP_SDC_DMM_WFULL | 110 | - | - |
| INTRA_DEP_DMM_WFULL | 1110 | 1 | - |
| INTRA_DEP_DMM_CPREDTEX | 1111 | - | - |

There are nine tests for these three methods. Both CTC and all intra configuration are tested for each method. CTC configurations with bug-fixed version HTM are also tested for each method

The proposed algorithm described in JCT3V-F0147 matched the provided source code. And the simulation results provided by the proponent are the same as those rerun by us.

# Conclusion

The algorithm description and simulation results of JCT3V-F0147 are verified.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Y. Lee, M. W. Park, H.-C. Wey, C. Kim, “3D-CE5 related: DMM simplification and signalling”, Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0147, Oct., 2013.