| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0242 |
| --- | --- |

| Title: | CE3 related: cross-check of Qualcomm's HEVC compatible deblocking for sub-PUs F0135 by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution is a cross-verification of proposal F0135 on deblocking filter for 3D-HEVC, which is related to CE3. The cross-checker confirms perfect match of bitrates and PSNR for the coded video sequences. The synthesized views PSNR mostly matches, with several cases of 0.0001 difference, which possibly happens because of different systems used to run the simulations.

# Introduction

The cross-checked document JCT3C-F0135 [1] proposes changes to the deblocking filtering process applied on top of the proposal JCT3C-F0110 on sub-PU level inter-view motion prediction [2]. The proposal suggests setting “virtual TU boundaries” inside PUs that use sub-PU level inter-view motion prediction, so that the deblocking filtering be applied to those boundaries inside the PU.

# Cross-verification results

The cross-verifier reports a match of BD-rate results on the common test conditions [3]. The coded video sequences show perfect match. The synthesized views PSNR also matches, with several cases of 0.0001 difference, which probably happens because of different systems used to run the simulations. The results for the anchor have not been run and have been taken instead from JCT3V-F0110 (F0135 uses F0110 results for 8x8 sub-PUs as the anchor).  The decoding and encoding times could not be verified for this reason (and also since the software has been run on a heterogeneous computational cluster).

Table 1 3-view test case comparison of F0135 against the F0110 8x8 sub-PU anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.89% | -0.62% | -0.38% | -0.37% | -0.30% |
| Kendo | 0.00% | -0.48% | -0.83% | -0.37% | -0.39% | -0.20% |
| Newspaper_CC | 0.00% | -0.45% | -0.32% | -0.19% | -0.16% | -0.07% |
| GT_Fly | 0.00% | -0.12% | -0.21% | -0.13% | -0.16% | -0.13% |
| Poznan_Hall2 | 0.00% | -0.42% | -0.77% | -0.30% | -0.30% | -0.29% |
| Poznan_Street | 0.00% | -0.50% | -0.36% | -0.23% | -0.23% | -0.15% |
| Undo_Dancer | 0.00% | -0.60% | -0.38% | -0.24% | -0.23% | -0.11% |
| 1024x768 | 0.00% | -0.61% | -0.59% | -0.31% | -0.31% | -0.19% |
| 1920x1088 | 0.00% | -0.41% | -0.43% | -0.22% | -0.23% | -0.17% |
| average | 0.00% | -0.49% | -0.50% | -0.26% | -0.26% | -0.18% |
| Shark | 0.00% | -0.25% | -0.27% | -0.20% | -0.20% | -0.07% |

Table 2 2-view test case comparison of F0135 against the F0110 8x8 sub-PU anchor

|  | video 0 | video 1 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.9% | -0.3% | -0.3% | -0.3% |
| Kendo | 0.0% | -0.5% | -0.2% | -0.2% | -0.2% |
| Newspaper_CC | 0.0% | -0.4% | -0.1% | -0.2% | -0.1% |
| GT_Fly | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Poznan_Hall2 | 0.0% | -0.4% | -0.2% | -0.2% | -0.3% |
| Poznan_Street | 0.0% | -0.5% | -0.2% | -0.2% | -0.2% |
| Undo_Dancer | 0.0% | -0.6% | -0.2% | -0.2% | -0.1% |
| 1024x768 | 0.0% | -0.6% | -0.2% | -0.2% | -0.2% |
| 1920x1088 | 0.0% | -0.4% | -0.2% | -0.2% | -0.1% |
| average | 0.0% | -0.5% | -0.2% | -0.2% | -0.1% |
| Shark | 0.0% | -0.2% | -0.1% | -0.1% | -0.1% |

# Conclusions

The cross-checker confirms perfect match of bitrates and PSNR of the coded video sequences. The synthesized views PSNR mostly matches, with several cases of 0.0001 difference.

# References

- H. Liu, Y. Chen, G. V. D. Auwera, L. Zhang, "CE3 related: HEVC compatible de-blocking for sub-PUs", JCT3V-F0135, Geneva, Switzerland, Oct.-Nov. 2013.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0110, Geneva, Switzerland, Oct.– Nov., 2013.

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-E1100, Vienna, Austria, July-Aug., 2013.