| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0343r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0244r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Summary of contributions on POC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides a summary of the seven contributions in the agenda category “POC alignment and derivation”, section 6.4.2 of the JCT-VC meeting notes, and item 3 of JCTVC-O0223.

Contributions JCTVC-O0176 and JCTVC-O0275 both propose to remove the current POC resetting mechansim and replace it with a different mechanism. The other five contributions try to improve the current POC resetting mechansim.

It is therefore suggested to first discuss JCTVC-O0176 and JCTVC-O0275. If one of the two (main proposals) can be agreed, then there is no need to discuss the other five contributions.

Among the six contributions based on the existing POC resetting mechanism, proposals in JCTVC-O0117, JCTVC-O0211, and item 3 of JCTVC-O0223 are indenpendent from each other and of other proposals. And decsions for these proposals should be relatively easy. Thus, these three are suggested to be discussed before the other three based on the existing POC resetting mechanism.

Proposals in JCTVC-O0128, JCTVC-O0140, and JCTVC-O0213 are related to each other, as both JCTVC-O0128 and JCTVC-O0213 identify a bug of the existing POC resetting mechanism and proposes a bug fix, and both JCTVC-O0140 and JCTVC-O0213 propose a POC MSB reset flag.

# Detailed summary of the contributions

JCTVC-O0176 HLS: Error robust POC alignment [R. Sjöberg, J. Samuelsson, R. Yu (Ericsson)]

Asserted problems: The current POC resetting approach has problems with lost pictures, missing temporally co-located pictures, and temporal layers.

Proposal 1:

- Remove poc_reset_flag and the associated POC resetting mechanism.

- Signal a 16-bit poc_lsb_idr in the slice header extension of base laye IDR pictures when cross_layer_irap_aligned_flag is equal to 0.

- Change the POC derivation process as follows (for those parts not mentioned, the process remains unchanged):

  - When the "POC anchor picture" (the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture) is not present in the DPB, prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

  - When cross_layer_irap_aligned_flag is equal to 0, the following applies:

    - When the current picture is a base layer IDR picture, the following applies:

      - Instead of being setting equal to 0, PicOrderCntMsb is derived using poc_lsb_idr to replace slice_pic_order_cnt_lsb in Eqn. F-29.

      - Decrement POC of each picture (of any layer) by PicOrderCntMsb + poc_lsb_idr.

    - When the current picture is an enhancement layer IDR picture and the base layer picture in the same AU is an IDR picture, the POC value is set equal to that of the base layer IDR picture in the same AU.

Proposal 2: Change the note as follows:

NOTE 1 – All IDR pictures with nuh_layer_id equal to 0 will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for those pictures IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

JCTVC-O0275 MV-HEVC/SHVC HLS: on POC value derivation [M. M. Hannuksela (Nokia)]

Asserted problems: The error resilience issue, changing the POC values of the base layer, the POC values can be dynamic during their persistence.

Proposal 1:

- Remove poc_reset_flag and the associated POC resetting mechanism.

- PicOrderCntMsb is set equal to 0 for an enhancement-layer picture in the same AU as a base-layer IDR picture (otherwise the POC derivation process remain unchanged).

- Slice headers of enhancement-layer slices contain a layer-wise delta POC value (layer_delta_pic_order_cnt_minus1), the presence of which is gated by a slice header flag and a PPS flag. The layer-wise delta POC value is intended to be included in the first picture of an enhancement layer after the base-layer included an IDR picture. The layer-wise delta POC value specifies the POC difference of the "POC anchor picture" and the current picture as if the POC values were not reset in between them.

- A layer-wise POC value, LayerPicOrderCntVal is maintained for each enhancement layer picture. If layer_delta_pic_order_cnt_minus1 is present, it is added to LayerPicOrderCntVal of the "POC anchor picture" to form the LayerPicOrderCntVal of the current picture. Otherwise, LayerPicOrderCntVal is incremented based on the PicOrderCntVal difference of the current picture and "the POC anchor picture".

- The function DiffPicOrderCnt( ) is specified to use LayerPicOrderCntVal values.

Proposal 2: Reset the PicOrderCntMsb value for the first picture of each layer after the layer-wise start-up process has been initiated. This ensures a “clean” reset of the POC values for each layer after a splice point triggering the layer-wise start-up process.

JCTVC-O0117 On POC Alignment [S. Deshpande (Sharp)]

Asserted problems:

- The "POC anchor picture" may be asbsent in the DPB, thus the variable prevPicOrderCnt used in the POC derivation needs to be updated when POC is reset.

- The use of slice_pic_order_cnt_lsb in Eqn. 8-6 instead of the derived POC LSB, i.e. PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ), can results in incorrect RPS.

Proposal 1: When poc_reset_flag is equal to 1, the value of the variable prevPicOrderCnt, i.e. the PicOrderCntVal of prevTid0Pic, is updated. This updated value is used in POC derivation for subsequent pictures in decoding order.

Proposal 2: Modification to the RPS derivation process to utilize the derived POC LSB, i.e. PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ), instead of slice_pic_order_cnt_lsb (changing Eqn. 8-6).

JCTVC-O0211 MV-HEVC/SHVC HLS: On cross-layer POC alignment for layer-wise startup [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

Asserted problem: The value of the variable prevPicOrderCnt cannot be derived in the current POC derivation process for non-IRAP pictures in an initial IRAP access unit that is the first AU in the bitstream, although the POC vlaues of these pictures, which must have poc_reset_flag equal to 1, are equal to 0.

Three alternative solutions are proposed to avoid referring to the variable prevPicOrderCnt:

- Alternative 1: Directly set the PicOrderCntVal of a non-IRAP picture with poc_reset_flag = 1 and nuh_layer_id >0 to 0 when FirstPicLayerDecodedFlag[ nuh_layer_id ] is equal to 0.

- Alternative 2: Directly set the PicOrderCntMsb of a non-IRAP picture with poc_reset_flag = 1 and nuh_layer_id >0 to 0 when FirstPicLayerDecodedFlag[ nuh_layer_id ] is equal to 0.

- Alternative 3: Directly set prevPicOrderCnt of the of a non-IRAP picture with poc_reset_flag = 1 and nuh_layer_id >0 to 0 when FirstPicLayerDecodedFlag[nuh_layer_id] is equal to 0.

JCTVC-O0223 MV-HEVC/SHVC HLS: Comments on latest MV-HEVC and SHVC draft specs [K. Rapaka, Y. Chen, A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late]

Only item 3 is related.

Asserted problem: Missing temporally co-located pictures (same as problem 2 in JCTVC-O0176, see the 2nd figure above under JCTVC-O0176).

Proposal: A definition and a constraint:

POC-anchor picture: A picture that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

It is a requirement of bitstream conformance that in each access unit that contains at least one picture with poc_reset_flag equal to 1, the pictures of all layers in the bitstream shall be present in that access unit and shall be POC-anchor pictures.

JCTVC-O0128 MV-HEVC/SHVC HLS: On poc reset for long-term reference pictures [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

Asserted problem: In the RPS decoding process, the PocLtCurr and PocLtFoll values may be incorrectly derived when poc_reset_flag is set equal to 1.

Two alternative fixes are proposed (to Eqn. 8-5):

Alternative 1:

...
for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt  +=  PicOrderCntVal − 	DeltaPocMsbCycleLt[ i ] * MaxPicOrderCntLsb −
				PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) slice_pic_order_cnt_lsb
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
...

Alternative 2:

...
if ( poc_reset_flag ) {

for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {

delta_poc_lsb = PocLstLt[i] – slice_pic_order_cnt_lsb

pocLt = delta_poc_lsb & (MaxPicOrderCntLsb-1)

if( delta_poc_msb_present_flag[ i ] )
	if (delta_poc_lsb < 0)

pocLt += -( DeltaPocMsbCycleLt[ i ]+1)*MaxPicOrderCntLsb

else

pocLt += -( DeltaPocMsbCycleLt[ i ])*MaxPicOrderCntLsb

if( UsedByCurrPicLt[ i ] ) {

PocLtCurr[ j ] = pocLt
	  CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]

} else {
	PocLtFoll[ j ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]

}

}

} else {

for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {

pocLt = PocLsbLt[i]

if( delta_poc_msb_present_flag[ i ] )
      pocLt += PicOrderCntVal – DeltaPocMsbCycleLt[i]*MaxPicOrderCntLsb –

PicOrderCntVal&(MaxPicOrderCntLsb-1)

if( UsedByCurrPicLt[ i ] ) {

PocLtCurr[ j ] = pocLt
CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]

} else {
	PocLtFoll[ j ] = pocLt
	FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]

}

}

}
...

JCTVC-O0140 MV-HEVC/SHVC HLS: Alignment of picture order counts [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Asserted problems:

- When a bitstream is spliced and a CRA picture type is changed to a BLA picture type at that point, the POC value alignment cannot be ensured, because a POC MSB of a layer will be reset to 0 while POC MSBs of other layers are not reset.

- It may complicate the POC derivation of the following pictures that poc_reset_flag values is reset by a system for splicing, due to the misalignment of POC LSB values.

Proposal 1: Add poc_msb_reset_flag, when equal to 1 the POC MSB is reset to 0.

Proposal 2: Add the following constraints:

- The following constraints are proposed.

  - If the poc_reset_flag value of a picture in an access unit is equal to 1, the poc_reset_flag values of the other pictures in the same access unit shall be equal to 1.

    - If layers have different poc_reset_flag values, the POC values cannot be the same.

  - If a picture in an access unit is an IDR picture, the poc_reset_flag values of the other pictures in the same access unit shall be equal to 1.

    - Since the POC value of an IDR picture is equal to 0, the POC values of the other pictures shall be reset equal to 0.

  - An IDR picture and a BLA picture cannot be present in the same access unit.

- If poc_msb_reset_flag is adopted,

  - If the poc_msb_reset_flag value of a picture in an access unit is equal to 1, the poc_msb_reset_flag values of the other pictures in the same access unit shall be equal to 1.

    - If layers have different poc_msb_reset_flag values, the POC MSB values cannot be the same.

  - If a picture in an access unit is a BLA picture or a CRA picture with HandleCraAsBlaFlag equal to 1, the poc_msb_reset_flag values of the other pictures in the same access unit shall be equal to 1.

    - Since the POC MSB value of a BLA picture is equal to 0, the POC MSB values of the other pictures shall be reset equal to 0.

Proposal 3: Change the POC derivation process as follows:

...

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb − PicOrderCntOffset

When poc_reset_flag is equal to 1, the following steps apply in the order listed:

- PicOrderCntOffset is set equal to PicOrderCntVal.

- The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntValPicOrderCntOffset.

- PicOrderCntVal is set equal to 0.

...

Proposal 4: Remove the signalling of POC LSB in the slice header for IDR pictures with nuh_layer_id greater than 0, or correct the note as follows:

NOTE 1 – All IDR pictures nuh_layer_id equal to 0 will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

JCTVC-O0213 On picture order count [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang, Hendry (Qualcomm)]

Asserted problems:

- A bug in the current POC design: under certain conditions, when the base layer of bitstream is decoded by a legacy (version 1) HEVC decoder, the POC values derived for the BL pictures can be incorrect in their relative ordering with other pictures.

- The error resilience problem.

Proposal 1:

- Instead of signal a POC reset flag, signal two flags, one for POC MSB reset, and the other for POC LSB reset, and change the POC derivation process accordingly.

- As part of the above item, some restrictions are proposed:

When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 and at least one picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1, poc_msb_reset_flag shall be present and the value shall be equal to 1.

When the current picture is not an IDR picture or slice_pic_order_cnt_lsb is not equal to 0, and the picture with nuh_layer_id equal to 0 in the current access unit is an IDR picture, poc_lsb_reset_flag shall be present and the value shall be equal to 1.

When the value of poc_msb_reset_flag is equal to 0, the value of poc_lsb_reset_flag shall be equal to 0.

It is a requirement of bitstream conformance that, when there is an IRAP picture with NoRaslOutputFlag equal to 1 in the current access unit, the current picture shall have discardable_flag equal to 0 and TemporalId greater than 0, and shall not be a sub-layer non-reference picture, a RASL picture, or a RADL picture.

Proposal 2: A POC reset information SEI message is proposed to provide information for POC derivation when the previous picture in decoding order has either POC LSB or POC MSB reset is lost. The syntax of the SEI message is as follows:

| poc_reset_info( payloadSize ) { | Descriptor |
| --- | --- |
| poc_reset_type_flag | u(1) |
| poc_reset_value | u(32) |
| poc_reset_id | u(7) |
| } |  |