| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0247 | Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul. – 2nd Aug. 2013 | Document: JCT3V-E0194 |
| --- | --- | --- | --- |

| Title: | CE4: Crosscheck on Further improvements on advanced residual prediction (JCT3V-F0123) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu, Seoul 137-130 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution provides a cross-check evaluation for the proposal of further improvements on advanced residual prediction, JCT3V-F0123. The verification was finished successfully, and we have confirmed that our results completely match those provided by Qualcomm[1].

# Introduction

JCT3V-F0123 proposes further improvements on advanced residual prediction in 3D-HEVC. From the document, it is reported that the proposed method includes : 1) the ARP is extended for inter-view residue, when the current block utilizes inter-view prediction from an inter-view reference picture, in a similar way as current design; 2) a disparity motion vector associated with the reference block of current block, if available, is used during ARP, to replace the derived disparity vector; 3) the signalling of Illumination Compensation (IC) flag and ARP weighting factors are jointly considered wherein the ic_flag is not signalled when the ARP weighting factor is unequal to 0; 4) In addition to the above items, block-level ARP is proposed by splitting one prediction unit into several blocks to perform ARP separately. The proposed method is verified with different combinations of schemes described above.

- Test #1: the full proposal

- Test #2: combination of the first three aspects, 1), 2), and 3)

In this crosscheck, test #1 and test #2 have been verified. We have performed the crosscheck for this proposal and found out that the results are identical to ours.

# Experimental results

The result of the proposal from us exactly matches the result provided by the proponent. There is no mismatch between encoder and decoder. The algorithms have been simulated on the common test conditions Error: Reference source not found.

Table 1 HTM8.0 vs. Test #1

Table 2 HTM8.0 vs. Test #2

# Conclusion

The experimental results (completely) match from each other. The cross-check is confirmed.

# Reference

- L. Zhang, Y. Chen, M. Karczewicz, “Further improvements on advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0123, 6th Meeting: Oct. 25 - Nov. 1, 2013, Geneva, CH.

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: July 27–Aug. 2, 2013, Vienna, AT.