| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0349_v5 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0248_v5 |
| --- | --- |

| Title: | BoG report on SHVC/MV-HEVC HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met Oct 25 to review the following contributions, some of which contain additional topics not yet reviewed.

- JCTVC-O0059, JCTVC-O0092, JCTVC-O0096, JCTVC-O0252, JCTVC-O0109, JCTVC-O0179

The BoG recommends the following:

- Restrict sharing of SPS and PPS across layers to avoid creating problems during sub-bitstream extraction, based on modification of proposals in JCTVC-O0059 and JCTVC-O0092, reflected in the v2 version of O0092.

- Add flag in rep_format( ) syntax structure to control sending of chroma and bit depth related parameters, as proposed in the v2 version of JCTVC-O0179.

- Refers to the editors a problem identified with the integration of JCTVC-N0092.

The v2 version of this document captures the Oct 26 meeting of this BoG, where the following contributions were reviewed:

- JCTVC-O0096, JCTVC-O0125,  JCTVC-O0141,  JCTVC-O0093,  JCTVC-O0223,  JCTVC-O0058,  JCTVC-O0109,  JCTVC-O0111,  JCTVC-O0179,  JCTVC-O0214,  JCTVC-O226, JCTVC-O0118

All documents originally categorized to section 6.4.4. Parameter Sets were given a review.  Some additional documents related to topics in this section were also reviewed.  Several revisits within the BoG are still required.

The BoG recommends the following:

- Modify the SPS syntax for layers with nuh_layer_id > 0 to signal a reference to a rep_format index in the VPS, rather than signaling explicit representation format data in the SPS, from the v2 version of JCTVC-O0096.

- Add a gating flag in VPS extension to condition the presence of direct dependency type, with a default type signaled, from JCTVC-O0096

- Modify the VPS extension syntax and semantics to replace view_id_len_minus1 with view_id_len, always signal that syntax element, add a constraint that (1<<view_id_len) >= NumViews, and modify view_id_val semantics to infer value of 0 when not present, from discussion of JCTVC-O0109

- Modify the semantics of profile_ref_minus1[ i ] to replace “shall be less than i” with “shall be less than or equal to i”, from discussion of JCTVC-O0109

- Move the vps_vui_present_flag to precede vps_vui_offset, and make vps_vui_offset conditional on that flag, from JCTVC-O0109

- Change default_one_target_output_layer_flag to a 2-bit default_one_target_output_layer_idc, and reserve values of 2 and 3, from JCTVC-O0109 3rd proposal

- Revisit in track proposal 4 from JCTVC-0214, regarding constraints on never activated parameter sets.  The identified issue applies to version 1 and extensions

The v3 version reflects the BoG meeting on Oct 26 to review the following contributions:

- JCTVC-O0353, JCTVC-O0138, JCTVC-O0120, JCTVC-O0225, JCTVC-O0254, JCTVC-O0271 JCTVC-O0061, JCTVC-O0060, JCTVC-O0174, JCTVC-O0062, JCTVC-O0175, JCTVC-O0260 JCTVC-O0273, JCTVC-O0110, JCTVC-O00116, JCTVC-O0119, JCTVC-O0153, JCTVC-O0223

The BoG recommends the following:

- Add syntax elements to signal max temporal sub-layers for each layer in the VPS, with a gating flag, from JCTVC-O0120 option 2.

- Change derivation of NumActiveRefLayerPics to consider max_tid_il_ref_pics, from JCTVC-O0225.

- Revisit 2nd proposal of JCTVC-O0225 in track regarding signaling of max_tid_il_ref_pics per layer, based upon relation to SCE2 on single loop decoding

- Revisit JCTVC-O0062 in track, regarding extraction/rewriting of independent non-base layer

- Assign to a different category: JCTVC-O0056, JCTVC-O0057

- Revisit JCTVC-O0273 in track, regarding multi-mode bitstream extraction

- Add a flag in the VPS to indicate for decoding process to output the highest available layer if the target output layer is not available, from JCTVC-O0153

- Review in track JCTVC-O0137 , JCTVC-O0200, and JCTVC-O0223 proposal 4, regarding expanding the number of layers

- Add a flag in VPS VUI to indicate cross layer pic type alignment.  Move cross_layer_irap_aligned_flag to VPS VUI and make presence condition on added flag.  From JCTVC-O0223

The v4 version reflects the BoG meeting on Oct 27 to review or revisit the following contributions:

- JCTVC-O0059, JCTVC-O0214, JCTVC-O0118, JCTVC-O0120, JCTVC-O0262, JCTVC-O0153, JCTVC-O0343, JCTVC-O0176, JCTVC-O0275

The BoG recommends the following:

- Add visual signal information (video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs) per layer to the VPS VUI, from v2 version of JCTVC-O0118.

- Modify inter-layer reference picture list default construction to incorporate max temporal sub-layers per layer syntax elements in VPS extension, from r2 version of JCTVC-0120.

The v5 version reflects the BoG meetings on Oct 28 and 29 to review the following contributions:

- JCTVC-O0117, JCTVC-O0128, JCTVC-O0140, JCTVC-O0176, JCTVC-O0211, JCTVC-O0213, JCTVC-O0223, JCTVC-O0275, JCTVC-O0098, JCTVC-O0063, JCTVC-O0255, JCTVC-O0177, JCTVC-O0224, JCTVC-O0197, JCTVC-O0358

The BoG recommends the following:

- Modification of the PicOrderCntVal of prevTid0Pic and modification to the decoding process for reference picture set, to address problems found for cross-layer POC alignment, from JCTVC-O0117.

- Modify signaling of scaled reference layer offsets to allow signaling of any lower layer, rather than just a direct reference layer, in order to enable alignment of auxiliary pictures, from JCTVC-O0098.

- Revisit JCTVC-O0358 in a joint JCT-VC/JCT-3V session,  which proposes coding of individually controllable overlays

The v6 version of the notes reflect the Oct 30 meeting of the BoG, to revisit JCTVC-O226,  JCTVC-O211, and POC derivation and alignment issues.

The BoG recommends the following:

- Modifications to the VUI indicators of tile and WPP alignment related syntax elements, from the r1 version of JCTVC-O0266.

- Modify POC derivation to correct an ambiguity in the spec, from JCTVC-O0211.

- AHG study of POC derivation and alignment issues

# Review Notes

## Parameter sets

### Parameter set sharing across layers

JCTVC-O0059 MV-HEVC/SHVC HLS: On sharing parameter sets across layers [T. Tsukuba, T. Ikai, T. Yamamoto (Sharp)]

The first proposal in this contribution: Restrict that sharing parameter sets (SPS/PPS) shall be used in the condition the parameter sets is direct or indirect reference layer of the layer.

It is asserted that the proposed changes can provide the following benefit; Avoid the improper removal of sharing parameter set in bitstream extraction.

A consequence of this proposal is to disallow simulcast layers to share an SPS.  Auxiliary pictures would also be similarly restricted, since they won’t be dependent on the primary picture.

A suggestion was made to enforce this proposed restriction (which was also proposed in JCTVC-O0092) only if the layer id of the parameter set is greater than 0. A slice with layer id value CurLayerId can refer to a parameter set with a different layer id, if the layer id of the parameter set is 0 or if the parameter set is referred to by a layer that is a direct or indirect reference layer of the first layer.  Agreed of specification language for the restriction.

The 2nd proposal is: Add a new direct_depenency_type to show sharing parameter set dependency in VPS

The 3rd proposal is:  Signal scaled reference layer offsets for multiple layers which refer the same SPS or Move scaled reference layer offsets into VPS like JCTVC-N0089.  This would enable sharing of SPS’s when different scaled reference offsets are used, which could save some bits in this case.  For the 3rd proposal, no action.

JCTVC-O0092 MV-HEVC/SHVC HLS: On nuh_layer_id of SPS and PPS [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

This contribution proposes constraint on the value of nuh_layer_id of SPS/PPS NAL unit to ensure the availability of the parameter sets after sub-bitstream extraction.

Two options proposed.  The 1st option is to restrict sharing to dependent layers (either direct or independent).  The 2nd option is to require all parameter sets to have layer id of 0.

The 1st option is very similar to JCTVC-O0059.

The 2nd option would cause the base layer bitstream to include all parameter sets for all layers, which might be large.

A suggestion was made to consider layer sets in the restriction.

See notes for JCTVC-O0059.

### Representation format

JCTVC-O0096 AHG9: On high level syntax [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

This 2nd aspect of this contribution proposes: (2) An indicator in SPS is proposed to index into the representation format list to avoid explicit signaling of all representation parameters in SPS when possible.

In the current design, a list of rep_format( ) are signalled in the VPS. If the rep format is updated in the SPS, it may refer to an index to the list of formats that were signalled in the VPS.  Allows to save bits in the SPS when updated ref formats are used. The number of bits saved depends upon the resolution, it was suggested to be approx. in the range of 40-50 bits.

It was remarked that a restriction is needed to avoid the new rep format signaled to have a higher resolution than that originally signaled in the VPS.

It was suggested that if the contribution were adopted, the else condition to explicitly signal the new format may not be needed.

This was further discused after drafting the syntax for the  suggested alternative with the else condition removed.

(Revisit chaired by GJS Fri 25th pm.)

A modified syntax was reviewed, in which the format in the SPS would be identified by using an index into the list of formats sent at the VPS level (without the ability to alternatively send all the individual syntax elements that specify the format individually). This removes some of the flexibility of the original proposal, but provides the potential benefit of being able to identify all possible formats from information available in the VPS. It would still be possible to change beyond that, but only by sending IRAPs in all layers.

Agreed to recommend this modified version for adoption.

JCTVC-O0252 SHVC / MV-HEVC HLS: On signalling of representation format [Hendry, Y.-K Wang, Y. Chen (Qualcomm)]

This contribution discusses some issues of the current representation format signaling and proposes some modifications to address the issues. The current signaling of the representation format in VPS extension assumes the same default values for the syntax elements rep_format_idx_present_flag and vps_rep_format_idx when they are not present. However, in order to make the signalling optimal for both typical cases in MV-HEVC and SHVC, such syntax elements are proposed to be set with different default values based on the scalability dimensions.

Proposes to remove the condition for the presence of vps_rep_format_idx[ i ].  It was suggested to move the condition above the for loop rather than removing it.

Would save 4 bits in the VPS for MV-HEVC, by allowing the default to match the expected MV-HEVC case.

It was noted that there was an editorial problem with the integration of JCTVC-N0092 to MV-HEVC draft, where a section of the inference for vps_rep_format_idx[ i ] was omitted. Action: editors to correct the integration of JCTVC-N0092.

JCTVC-O0109 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

It is proposed to add single_rep_format_flag syntax element to target the MV-HEVC case, as the current design is optimized for SHVC. Proposal eliminates the need to explicitly signal that one rep format will be sent, which would use 4 bits in that case, per VPS.

It was questioned if the proposed syntax is really very different than the existing syntax. Some details were questioned about the changing vps_num_rep_formats_minus1 to vps_num_rep_formats_minus2.

JCTVC-O0179 MV-HEVC/SHVC HLS: Improvements of parameter sets [Jung Won Kang (ETRI), Jinho Lee, Hahyun Lee, Jin Soo Choi, Truong Cong Thang (UoA)]

The second proposal is the description of representation formats in VPS, including the index signaling and format signaling. Correspondingly, a revised updating mechanism of representation format in PPS is presented.

The proposed restructuring of the syntax table for the second proposal is an editorial change with the same behavior.  This is referred to the editors.

The third proposal proposes adding two syntax elements to the rep_format( ) syntax structure in the VPS, so that chroma and size can be sent without sending the bit depth. Motivation is that bit depth would be less likely to change.

It was suggested to restructure to include a single flag in rep_format( ) to control sending of chroma and bit depth. When chroma and bit depth not present, their values are inferred to be those from the previous rep_format( ) structure.  Revised text was reviewed  Agreed,.  .

An SPS modification was also proposed.    Not necessary given adoption of JCTVC-O0096.

### Dimension id signalling

JCTVC-O0125 SHVC/MV-HEVC HLS: On use of splitting_flag with combinations of scalability dimensions [A. Norkin, T. Rusert (Ericsson)]

In HEVC extensions, use of splitting_flag (signalling scalability identifiers in the layer_id_in_nuh) is constrained by the order of scalability dimensions in the scalability_mask.  This restriction does not allow using the splitting_flag with different combinations of scalability extensions which do not follow the order of scalability dimensions in the scalability_mask. The document proposes two solutions that enable different coding orders of scalability layers and therefore provide compatibility of the splitting_flag syntax and semantics with the future scalability extensions. Solution 1 is capable of supporting any coding order of scalability layers with the splitting_flag while at the same time simplifying the syntax and semantics of the working draft (see Solution 1, Syntax variant 2). Solution 2 enables more efficient use of nuh_layer_id bits in some cases and can be combined with Solution 1.

A suggestion was made to not change the syntax table, but to add the Reordering entry to the scalability mask table.  The language elsewhere in the spec needs to be checked to ensure that setting this mask to a non-zero value doesn’t cause problems, especially in profile definitions.  Version 2 of the document includes the suggested change.  Reserved entry in the scalability mask table is utilized, with a note added to describe its use for ordering.  Profile definitions updated to reflect this change, to allow this Reordering field to have any value.  Some concern with design cleanliness expressed.

JCTVC-O0141 MV-HEVC/SHVC HLS: Clean-ups of parameter sets [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

The contribution proposes:

When splitting_flag is equal to 1, vps_nuh_layer_id_present_flag shall be equal to 1.

- When i is greater than 0 and splitting_flag is equal to 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

No action.

### Direct dependency type signaling

JCTVC-O0141 MV-HEVC/SHVC HLS: Clean-ups of parameter sets [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

- all_layer_independent_flag is proposed.

No action

- direct_dep_type_present_flag is proposed.

Proposing that if this flag is set, infer direct dep type is equal to 3, which means both texture and motion prediction.  Semantic language problem of where inference rule is in the wrong place.

See notes under JCTVC-O0096.

JCTVC-O0096 AHG9: On high level syntax [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

(1) A default gating flag in VPS extension is proposed to condition the presence of direct dependency type;  The default would be that both sample prediction and motion prediction were used.

Suggestion to add explicit signaling of a default value, using ue(v).

Agreed.  Revised text to be provided in update of document, with co-authors from JCTVC-O0141.

JCTVC-O0093 AHG9: On inter layer dependency [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

This contribution proposes constraint and syntax modifications to simplify the high level signaling of inter-layer dependency type. The constraint is to mandate all direct dependent layers are used for inter-layer sample prediction. Based on this constraint, two options are proposed to simplify the current layer dependency signaling: (1) removing syntax element indicating dependency type length and merging the dependency flag syntax and the dependency type syntax together; (2) removing dependency type syntax and signaling inter-layer motion prediction using a new flag.

JCTVC-O0223 MV-HEVC/SHVC HLS: Comments on latest MV-HEVC and SHVC draft specs [K. Rapaka, Y. Chen, A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late]

The 6th proposal relates to coding of direct_dep_type_len_minus2.  Proposes to use u(3) coding instead of ue(v).

It was noted that a new adoption JCTVC-O0217 adds ue(v) coded syntax elements just prior to this syntax element.

### Other topics

JCTVC-O0058 MV-HEVC/SHVC HLS: On profile, tier, and level information [T. Tsukuba, T. Ikai, T. Yamamoto (Sharp)]

This contribution proposes to 1) remove profile_ref_minus1, 2) add inference rules for sub-layer profile and level information and 3) byte alignment for profile_tier_level.

No action on proposal 1.

For proposal 2, it proposes handling VPS profile/tier/level inference  for sub-layers differently than what is done for version 1.

For proposal 3, proposes adding an offset for profile tier level location.

No action.

JCTVC-O0096 AHG9: On high level syntax [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

This 3rd aspect of this contribution proposes for the slice segment header, including: (3) Conditions on two syntax elements are proposed to remove signaling redundancy.

Move contribution to section 6.4.5 inter-layer dependency signaling and derivation

JCTVC-O0109 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

Additional topics.

The 1st proposal, has identified a problem where when NumViews equals 1, the length of the view_id_len_minus1 is not known.

It was suggested to always send view_id_len, not use minus1 in the view_id_len, and infer a value for view_id_val if view_id_len is equal to 0.

| view_id_len | u(4) |
| --- | --- |
| if (view_id_len > 0) |  |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| } |  |

Semantics for view_id_val when not present to infer value of 0.

Restrict that (1<<view_id_len) >= NumViews

Agreed to the above suggested modified syntax and semantics for view_id_len.  Editors to verify that it is correct.

In the 2nd proposal, it is asserted that there is a problem with the profile_ref_minus1[i] semantics.  Proposes to change profile_ref_minus1 to profile_ref.

It was suggested to change the semantics so that the “less than i” becomes “less than or equal to i”. Agreed to this suggestion to modify semantics.

In the 3rd proposal, it is proposed to change default_one_target_output_layer_flag to a 2-bit default_one_target_output_layer_idc, and reserve values of 2 and 3.  Agreed.

The 4th proposal, proposes to remove cross_layer_irap_aligned_flag.  No action.

The 5th proposal suggests removing single_layer_for_non_irap_flag.  It was suggested to check the parameter set activation process when this flag is set. It was suggested that this flag could be in the VUI. Defer decision to when JCTVC-O0251 has been reviewed.

The 6th proposal proposes to move vps_vui_present_flag to precede vps_vui_offset, and make vps_vui_offset conditional.  Agreed.

JCTVC-O0111 MV-HEVC/SHVC HLS: On VPS extension syntax element arrangement [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

This contribution proposes to group the syntax elements of the vps extension into those necessary for decoding (sample reconstruction), and those for session negotiation. Also, it is proposed to have a presence flag and an offset to a group of side information, such that information unnecessary for sample reconstruction can be optional.

It would be useful to have an analysis of how many bits which would be in each section, and further consideration of which syntax elements should go in each category.  For further study.

JCTVC-O0179 MV-HEVC/SHVC HLS: Improvements of parameter sets [Jung Won Kang (ETRI), Jinho Lee, Hahyun Lee, Jin Soo Choi, Truong Cong Thang (UoA)]

Remaining topics: The first proposal is providing a flag in VPS that signals the presence of tiles in a CVS

A semantic constraint would be needed for the SPS tile enable flag to not conflict.

The constraint for when the flag is set is overly restrictive since it requires that more than one tile be present.  Should use “may” language instead.

Related to JCTVC-O0226.  There was support for the proposal, see notes under JCTVC-O0226.

JCTVC-O0214 On parameter sets [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, V. Seregin (Qualcomm)]

See also section Error: Reference source not found. This document appears twice in the notes, since it touches multiple topics.

Remaining topics.

This document provides an analysis of VPS size, discusses a few parameter set related issues, and proposes solutions for the issues. These issues include: 1) repetition of parameter sets within one AU, 3) scaling matrix prediction, 4) bitstream conformance restrictions on parameter sets that are never activated, 5) repetition of same SPS VUI in SPSs, and 6) signaling of MV-HEVC SPS extension syntax elements.

A problem has been identified with repeating a parameter set in more than one layer because of sub-bitstream extraction.  It was suggested that a note be added to clarify.

A potential error resiliency issue was raised regarding scaling matrix prediction, when the referred to SPS has not been activated because of picture loss. Further study of this issue is recommended.

An issue has been identified regarding constraints on never activated parameter sets.  This issue applies to be version 1 and the extensions.  Revisit in track.

Proposal aspect 5 proposes to allow sending of entire SPS VUI in VPS, and optionally not sending in SPS for nuh_layer_id > 0.  Related to JCTVC-O0118.  Defer decision until it is reviewed.  Was discussed with revision of O0118, no action.

Proposal aspect 6 proposes to move the inter_view_mv_vert_constraint_flag from the SPS to the VPS, to avoid the need to use the SPS extension for MV-HEVC.  An editorial issue was identified for the semantics of inter_view_mv_vert_constraint_flag.  Some participants preferred that the flag remain in the SPS because it could differ for different views.

JCTVC-O0226 MV-HEVC/SHVC HLS: On early indication of parallel processing tools in HEVC extensions [K. Rapaka, Hendry, Y.-K. Wang (Qualcomm)]

This contribution proposes to indicate the configuration of parallel processing tools (tiles and WPP) across layers in the VPS VUI.

The semantic constraints in the contribution require that the flags indicate that tiles_enabled_flag are equal to 0 for all PPSs or equal to 1 for PPSs, disallowing a combination.  It was suggested to have one state be “may” be 0 or 1 in PPS.

It was suggested that an additional condition for the presence of the the tile_boundaries_aligned_flag[ i ][ j ] should be added to require that the reference layer also had tiles enabled.

It was suggested that it might be useful to have a bitstream constraint flag in the VPS VUI, similar to what exists in the SPS.

There was support for the proposal, subject to implementing the above suggestions.   The r1 version of the document implemented the suggestions.

Agreed, as expressed in the r1 version.

JCTVC-O0118 On Video Signal Information in VPS [S. Deshpande (Sharp)]

This document proposes syntax and semantics for signalling video signal information including video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs regarding layers in VPS extension.  It is asserted that this is useful for session negotiation. Additionally a mechanism is proposed to share this information with SPS and to also enable update of the video signal information in the SPS.

It was questioned if putting these parameters in the VPS is helpful for session negotiation if updating of the values in the SPS was allowed.

There was some support for the proposal.  It was suggested to explore combination with JCTVC-O0214 proposal item 5.Inter-layer dependency signaling and derivation

Agreed to add visual signal information to VPS VUI, from v2 of the contribution.

The second aspect in combination with JCTVC-O0214 related to sharing VUI from the VPS to the SPS, allows bitrate savings when the VUI could be shared but the SPS could not be shared. No action on the combination with O0214.

- JCTVC-O0353 MV-HEVC/SHVC HLS: Summary of contributions on inter-layer dependency signalling and derivation [A. K. Ramasubramonian (Qualcomm)]

Additional early error detection within an access unit may be beneficial, but isn’t seen as essential because errors would be detected at a higher dependent layer in the current access unit, or at the next access unit.   Slice level bitrate overhead would be undesirable.

JCTVC-O0138 MV-HEVC/SHVC HLS: On inter-layer dependency signalling [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Some clean-ups or improvements of syntax related to inter-layer RPS or other layer dependency indication are proposed.

- On inter-layer reference picture set

  - Removal of the inter-layer RPS

  - Or, improvements of the inter-layer RPS ( inter-layer DPB status instead of direct inter-layer references)

Other related contributions provide similar error loss benefits.

Signaling of max_one_active_ref_layer and all_ref_layers_active_flag on a per layer basis, rather than for all layers, in the VPS.  Allows more efficient use of shortcuts of slice header, under more conditions, especially for potentially future scalability combinations.  Would add a subscript to the conditions in the slice header syntax table. Change to derivation when those flags were used. No analysis of bitrate savings provided. Proponents asked to provide savings, and describe conditions under which savings are achieved, along with applicable use case.  Further study.

Make signaling of max_one_active_ref_layer conditional on value of all_ref_layers_active_flag. Could have implications for profile restrictions for SHVC.  No action.  Also in JCTVC-O0138, proposal 2, item 1.

Proposes to add high_ref_layers_active_flag in slice segment header,  Requires adding a syntax element to potentially save bits in the slice header when unusual inter-layer referencing is used  No action on this aspect.

JCTVC-O0120 On Inter-layer Reference Picture Set [S. Deshpande (Sharp)]

It is asserted that when different layers have different frame rates and when max_one_active_ref_layer_flag is equal to 1 or  NumDirectRefLayers[ nuh_layer_id ]  is equal to 1 or  all_ref_layers_active_flag is equal to 1 the decoder is not able to distinguish between the case that an AU had no coded picture for a direct reference layer of a current layer due to that picture not existing in the bitstream (e.g. due to the reference layer having different frame rate) versus the case that the coded picture for the direct reference layer of a current layer was lost during transmission. This document proposes solution to distinguish between these cases. Two alternative solutions are proposed.

- A slice segment header signalling and modification to the decoding process for inter-layer reference picture set is proposed.

- A VPS signalling and other semantics change is proposed

Option #1, directed at higher frame rates for higher enhancement layers.  More relevant to MV-HEVC than SHVC, because SHVC is expected to allow only one active reference layer.   Adds one new syntax element to slice header, which is conditionally present.  Would enable all_ref_layers_active_flag to be set under more conditions with different frame rates, leading to more efficient slice header signaling. No analysis provided of bitrate savings.

Option #2, send max temporal sub layers for each layer in VPS.  Allows a decoder or middle box to determine if a lower layer picture of a higher temporal sub-layer access unit is intentionally missing vs. lost.

It was suggested to have a gating flag to be able to avoid sending these syntax elements. Agree to add the VPS syntax elements, from Option 2, with a gating flag.

Decoding process change, would affect all_ref_layers_active_flag semantics and meaning.  Further discussed decoding process change after experts had opportunity to view semantics text further.    Agreed, as reflected in version r2 of the document, showing combination with JCTVC-O0225.

JCTVC-O0225 MV-HEVC/SHVC HLS: On inter-layer RPS derivation and sub-layer inter-layer dependency [K. Rapaka, Y.-K. Wang, J. Chen, V. Seregin, Hendry, M. Karczewicz (Qualcomm)]

This contribution proposes two changes for SHVC and MV-HEVC related to inter-layer RPS derivation process and signaling of sub-layer inter-layer prediction dependency. The first change targets to improve the derivation process of NumActiveRefLayerPics by considering the sub-layer inter-layer prediction restrictions indicated by max_tid_il_ref_pics_plus1, so that the pictures belong to sub-layers that are disallowed to be used for inter-layer prediction are not included in the inter-layer reference picture set when all_ref_layers_active_flag is equal to 1. The second change targets to have layer-specific signaling of max_tid_il_ref_pics_plus1, where max_tid_il_ref_pics_plus1 is signalled separately for the layers that use the current layer as a direct reference layer.

For 1st proposal, no changes to syntax, but changes semantics and decoding process.  Allows a more efficient slice segment header when max_tid_il_ref_pics_plus1 is used. Bitrate savings numbers are provided, 3 bits per slice for 3 layers, 7 bits per slice for 4 layers. Is related to JCTVC-M0209. Agreed to change semantics and decoding process in proposal #1.

For 2nd proposal, is related to SCE2 single loop decoding. Provides a more flexible signaling of max TiD, by sending on a per layer basis. Revisit in the track, because of relation to SCE activities.

JCTVC-O0254 MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling [Hendry, Y. Chen, Y.-K Wang (Qualcomm)] [late]

This contribution is a follow-up proposal of JCTVC-N0118 with some syntax and semantic changes. It is proposed to signal the candidates inter-layer reference picture sets in VPS and the candidates can be used by each slice. Bit-count analysis of bit-count is also provided to show the effectiveness of the proposed method.

For the different frame rates for different layers case, provides bitrates savings.  Numbers of bitrate savings in the slice header provided.  Savings were small in typical cases.  Could provide savings in slice header in future extensions using combined scalabilities.

Would align inter-layer RPS with regular RPS used for temporal prediction.

Further study, to consider combination with JCTVC-O0138 proposal 1.

JCTVC-O0271 SHVC: On direct_dependency_flag [K. Sato (Sony)]

In SHVC specification, layer sets can be defined by the syntax element “layer_id_included_flag[i][j]” in VPS. Direct dependencies between layers can be defined by “direct_dependency_flag[i][j]” in VPS_extension(), and indirect dependencies between layers can be inferred from the values of this syntax element. If there exists direct or indirect dependency between layerId A and B (A > B), it means that layer A cannot be decoded unless layer B is decoded. Then if a certain layer set contains layer A, it should also contain layer B. However there is no constraint in the semantics of direct_dependency_flag[i][j].

This contribution proposes to add constraint on the semantics of direct_dependency_flag[i][j] associated with layer sets defined by layer_id_included_flag[i][j].

A participant noted that the current sub-bitstream extraction process already imposes such a constraint.

JCTVC-O0061 MV-HEVC/SHVC HLS: Alternative colPic indication [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

The syntax, alt_collocated_indication_flag and collocated_ref_layer_idx, were removed due to possible low-level implementation changes and the lack of coding efficiency improvement ground. This proposal proposes an enhanced alternative indication mechanism which could produce short cut for motion only inter-layer picture without low-level changes and syntax changes. The alternative method could achieve bit saving up to 6 bits per slice (4 bits per slice if the length of RefPicList[] is 4), assuming that the motion only inter-layer picture is stored in the last position of RefPicList and selected for colPic.

Contribution spec language has issue with assigning a value to a syntax element.  This contribution  may give bit savings in the slice header for the MV+D case when motion information is predicted but texture is not predicted. No action.

### Reference picture list construction (2)

JCTVC-O0060 MV-HEVC/SHVC HLS: reference picture list construction for motion only dependent picture [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

This contribution proposes an interlayer reference picture set (RPS) which is used for non-sample dependency reference pictures. It is proposed to define a motion only inter layer RPS as RefPicSetInterLayer2 and insert it not in the temporal reference picture list (RefPicListTempX) but in the extended part of reference picture list (RefPicListX). It is asserted that it is beneficial to avoid reference picture list modification. Additionally, the range of list_entry_LX is confined to the conventional part of reference picture list to exclude the motion only pictures.  The bit amount of list_entry_LX could be reduced. In this contribution, the main part of reference picture list, whose length is defined by num_ref_idx_lX_active_minus1, excludes motion only pictures, which reduces bit amount of ref_idx_LX and weighted prediction values. colPic can be indicated with the extended part of RefPicListX, which includes motion only pictures.

A participant noted that the binarization would be efficiently CABAC coded, so no change is needed.

JCTVC-O0174 MV-HEVC/SHVC HLS: Reference picture set/list derivation independent of ViewId [M. M. Hannuksela (Nokia)]

The contribution attempts to make the inter-layer reference picture set decoding process is independent of scalability dimensions in use and the same in MV-HEVC and SHVC, while initial reference picture lists obtained with the proposal can reportedly be identical to those achieved by the current MV-HEVC and SHVC draft standards.

The following changes are proposed:

- Specify zero or more partitionings of active inter-layer reference pictures in the VPS extension. Each partitioning is assigned an index. Each inter-layer reference picture partitioning specifies the division of active inter-layer reference pictures into two subsets (0 and 1, which correspond to RefPicSetInterLayer0 and RefPicSetInterLayer1, respectively, in the current MV-HEVC/SHVC draft text).

- Include a gating flag, il_rps_partition_enabled_flag, in the PPS. The gating flag controls the presence of the il_rps_partition_idx syntax element in the slice segment header.

- When there are more than one active inter-layer reference picture and when il_rps_partition_enabled_flag is equal to 1, the slice header includes an index of the inter-layer reference picture set partitioning, il_rps_partition_idx, to be used (when the number of partitionings is greater than 1).

Proposal changes default list construction.  List modification is still available.

Proposal would not make common processes depend upon scalability mask values.

Some similarity to an aspect of JCTCV-N0316 which sent similar info in the slice header.

It was remarked that the applications which would benefit from bit savings from the proposal are not clear.

## Miscellaneous HLS topics (8)

JCTVC-O0056 MV-HEVC/SHVC HLS: On conversion to ROI-capable multi-layer bitstream [T. Yamamoto, T. Ikai, T. Tsukuba (Sharp)]

This contribution proposes support of bitstream conversion to ROI-capable multi-layer bitstream. First part of this contribution introduces a conversion process with enhancement-layer picture size modification. Second part of this contribution proposes normative changes to SHVC/MV-HEVC to support the described conversion process. The proposed change includes signalling of the information on phase shift between enhancement-layer luma pixel and reference-layer luma pixel. The proposed changes are asserted to help to keep the phase shift after cropping.

Involves per pixel operation.  Should be reassigned to another category.

JCTVC-O0057 MV-HEVC/SHVC HLS: On support of different luma CTB sizes for different layers [T. Yamamoto, T. Ikai, T. Tsukuba (Sharp)]

Should be reassigned to another category.

JCTVC-O0062 On independent layer [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

This contribution proposes to restrict nuh_layer_id value dependent syntax so that independent layer can conform to HEVC version 1 syntax.  Currently, the version 1 decoder cannot decode an independent layer with nuh_layer_id greater than 0.

Proposal introduces a parsing dependency between SPS and VPS.

Would like to enable being able to extract a simulcast layer with layer_id greater than 0 be able to be decoded by a version 1 decoder, without a change to the slice header.

Update the document to include a new option 3 which is option 2 without modifications to SPS and PPS, only to slice header.  Revisit in the track.

JCTVC-O0175 MV-HEVC/SHVC HLS: Indications related to single-loop decoding [M. M. Hannuksela, H. Roodaki (Nokia)]

Revision of this document combines with JCTVC-O0260 and adds its authors.

It is proposed to enable indicating in the slice header which one of the following types a picture is:

- discardable picture. Specifies that the coded picture is not used as a reference picture for inter prediction and is not used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order

- intra-layer non-reference picture. Specifies that the picture is not used as a reference picture for inter prediction of subsequent pictures in decoding order within the same layer and that the picture may be used as an inter-layer prediction reference.

- non-output-layer skip (NOLS) picture. Specifies that the picture may be used as a reference for inter prediction of subsequent NOLS or discardable pictures in decoding order until the next unconstrained reference picture in decoding order with the same or lower TemporalId value than that of the current NOLS picture. Also specifies that the picture is not used as a reference for inter-layer prediction and not used as a reference for inter prediction of any unconstrained reference picture or intra-layer non-reference picture.

- unconstrained reference picture. Specifies that the picture may be used as a reference for inter prediction of subsequent pictures in decoding order within the same layer and may be used as a reference for inter-layer prediction.

The design already includes discardable flag.  This contribution provides several more attributes of this type.  Seems related to non sub layer reference, and max tid for ilp, and temporal_id_nesting_flag.  This contribution could provide capability to identify attributes with more precision for some coding patterns that don’t follow typical scalability prediction patterns.

Could apply these attributes for the base layer.  Could use num_extra_slice_header_bits.  Option 1 – replace discardable flag with a 2-bit indicator for the 4 states, use 1 addition num extra slice header

Option 2, use only with enhancement layer pics, include in slice segment header dependent on PPS presence flag and discardable flag.

Potential DPB benefits are questioned for a per layer DPB specification.  For a shared DPB specification there may be some savings.

Would be interesting to better understand the bitrate savings when temporal scalability is not used. Further study.

JCTVC-O0260 Inter Prediction Signalling and Picture Marking [K. Misra, S. Deshpande (Sharp)]

See notes under JCTVC-O0175.

JCTVC-O0262 MV-HEVC/SHVC HLS: decoding operation in the case of missing reference pictures [M. M. Hannuksela (Nokia)]

The contribution depends on the capability of specifying alternative output layers (JCTVC-O0153/JCT3V-F0058).

The reference picture set design enables decoders to detect unintentional losses of reference pictures and the layer-wise start-up process enables decoders to increment the number of decoded layers in a step-wise manner. It is proposed that if a missing reference picture in a reference picture set for an enhancement-layer picture is detected and an alternative output layer exists for that enhancement layer, the following applies:

- The layer-wise start-up process is initiated for the enhancement layer.

- Pictures of an alternative output layer are output until the enhancement layer contains an IRAP picture.

It is asserted that the proposed decoder operation provides tolerance to picture losses and careless bitstream manipulation e.g. in middle-boxes.

Some participants questioned if it is desirable to define a conforming behavior on a non-conforming bitstream.

JCTVC-O0273 MV-HEVC/SHVC HLS: On multi-mode bitstream extraction [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

In MV-HEVC/SHVC, it is not necessary for each AU to be "complete", i.e. picture from each layer in the bitstream is present in the AU. This allows use cases like picture rate scalability in SHVC as well as optimization of the size of the bitstream by discarding some pictures not needed for decoding of pictures belong to the target output layers, e.g. pictures with discardable_flag equal to 1 in layers that are not target output layers.

It has been proposed in JCTVC-N0267/JCT3V-E0087 to enable bitstream extraction with different modes. In this contribution, multi-mode bitstream extraction is re-proposed, and furthermore, it is proposed to require sub-bitstreams extracted under all the defined extraction modes to be conforming bitstreams, and different levels of conformance are specified.

A participant questioned if the coarseness of our level definitions would make it likely that a conformance point would have a lower level than the operation point from which it was extracted.

A bitstream encoded for single loop decoding for some pictures could benefit from this.

A participant expressed support for the contribution.  Another participant expressed support for the first aspect to define a sub-bitstream extraction process to create a conformance point, which is considered to be conforming, but didn’t consider the profile/tier/level signaling for a conformance point to be useful.

A question was raised about version 1 compliance for a conformance point containing only base layer pictures.

## Generic HLS issues

JCTVC-O0110 MV-HEVC/SHVC HLS: On picture output flag marking process [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

This contribution proposes PicOutputFlag marking process based on a signalled output layer set in the vps extension.

Some participants expressed an opinion that the current specification is functionally correct.  Perhaps editorial improvement could be made.  Suggestion for contributor to confer offline with the editors or submit a ticket for editorial improvements.

JCTVC-O0116 Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

This document proposes based on SHVC Draft 3 and MV-HEVC Draft Text 5 the following:

- The restriction regarding cross-layer alignment of TemporalId values in the same access unit is proposed to be relaxed with respect to IRAP pictures.

- The restriction regarding cross-layer alignment of TSA and STSA pictures in the same access unit is proposed to be relaxed to allow IDR pictures in the same access unit.

- A modification to the syntax related to restrictions on inter-layer prediction signalled in vps_extension is proposed.

The first proposal could cause problems with sub-bitstream extraction if the lower layer had a higher value of temporal_id than a higher layer.  The other direction would not have that problem, but would change the access unit definition previously agreed upon .  No action.

The 2nd proposal depends upon the 1st proposal.

For the 3rd proposal, it was questioned if the optimization applies to the typical case.   No action.

JCTVC-O0119 On Highest Temporal Sub-layer [S. Deshpande (Sharp)]

In this document :

- It is proposed to derive HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList.

It is proposed to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found when TemporalId is equal to HighestTemporalIdList[nuh_layer_id] during the  decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.2.

This potential benefit is uncertain with a per layer DPB operation.  Further study.

JCTVC-O0153 MV-HEVC/SHVC HLS: On output layer sets [M. M. Hannuksela (Nokia)]

The MV-HEVC/SHVC design allows access units in which the pictures may represent a subset of the layers present in the bitstream. In a typical SHVC use, only one picture per access unit (from the highest layer) is desired to be output. However, output layer sets are specified to be static for the duration that a VPS is active and the same output layer set is used for an entire coded video sequence. Hence, a mechanism is reportedly needed to specify which picture is to be output, if any, when no picture is present at an output layer.

It is proposed to specify a minimum alternative output layer in the VPS extension for each output layer. When a picture at the output layer is not present in an access unit, a candidate list of alternative output pictures includes pictures that are present in the access unit and have nuh_layer_id that is equal to a nuh_layer_id of a direct or indirect reference layer of the output layer and that is greater than or equal to nuh_layer_id of the minimum alternative output layer. Among the candidate list, the picture has the greatest nuh_layer_id value is output.

A problem was identified with the current specification. It could be addressed either by assuming a default behavior, or by sending some syntax elements to describe the behavior.  The best default behavior probably differs for MV-HEVC and SHVC.  A  min output layer was proposed which to be signaled with a syntax element in an [i ][ j ] loop, with a decoding process to use these syntax elements.  It was questioned if less syntax and a simpler decoding process could be used to address the problem.

A suggestion was made to use a simple global flag to solve the problem for now, with further study suggested for a more flexible solution. Agreed

For further study, DPB allocation impacts and allowing  implementation specific approaches.

JCTVC-O0137 MV-HEVC/SHVC HLS: Extended layer identifier [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Review in track.

JCTVC-O0200 3D/MV-HEVC HLS: Study and proposal of methods for extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

Review in track.

JCTVC-O0223 MV-HEVC/SHVC HLS: Comments on latest MV-HEVC and SHVC draft specs [K. Rapaka, Y. Chen, A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late]

This document discusses the following issues in MV-HEVC Draft Text 5 and SHVC Draft Text 3 and proposes corresponding solutions: 1) cross-layer alignment of picture type, 2) unnecessary restrictions on the semantics of poc_reset_flag, 3) POC resetting at an AU that does not have all pictures present, 4) support of larger numbers of views, 5) persistence scope of an active SPS, and 6) coding of direct_dep_type_len_minus2.

For the 1st proposal, a cross_layer_pic_type_aligned_flag  was proposed. A suggestion was made to move this and the cross_layer_irap_aligned_flag to VUI.   Agreed.

3rd proposal to be allocated to the POC category.

4th proposal deferred to the track.

5th and 6th proposals already discussed.

## POC alignment and derivation

There are two new overall design proposals, and smaller changes proposed to the current design.

There are two new overall design proposals, and smaller changes proposed to the current design.

For the existing design:

A problem has been identified with incorrect POC derivation for version 1 decoders under some conditions. (related to POC MSB reset)

A problem has been identified if a picture in one layer with POC reset flag set while there is no picture in another layer.

Error resilience when a layer picture with POC reset flag set is lost, detection of loss may be delayed, which affects the DPB state, which could impact decoding of future pictures in the CVS.

Base layer POC derivation process is modified from version 1 when an SHVC/MV-HEVC decoder is decoding a bitstream.

POC values of past pictures are no longer static, which may affect SEI message semantic persistence which refer to POC delta.  (progressive refinement SEI message, others?)

For JCTVC-O0176:

Enhancement layer POC derivation relies upon the base layer POC when IRAP in enhancement layer.  It is asserted that DPB state may be intact for decoding of future pictures if the base layer picture is lost.

Impose a restriction that if enhan layer has an IRAP, base layer pic must be present in same AU.

POC values of past pictures are no longer static, which may affect SEI message semantic persistence which refer to POC delta.

For JCTVC-O0275:

Vulnerable to loss of enhancement layer TL0 picture preceding the base layer IDR picture.

Some possible restrictions as to selection of reference pictures for enhan layer in the case where the same POC value for an enhan layer picture occurs prior to the base layer IDR

Bitrate tradeoffs:

JCTVC-O0176, IDR in base layer and cross layer irap pic equal to 0, 2 bytes in slice segment header extension, plus 3 bits for extension length

Existing design, POC reset flag uses 1 bit per layer in every slice, when needed

JCTVC-O0275:

- If 1 PPS used, 1 bit in every enhan layer slice, and also a ue(v) when the base layer has IDR.  If 2 PPS used, 1 bit plus ue(v)in enhan layer slice only when base layer has an IDR.

- When there is an IDR/BLA in the base layer but not in the enhan layer, the POC values expressed in the RPS are higher, and would take more bits.

The discussion has been almost exclusively among the proponents.

Suggested evaluation criteria:

- Solution does not cause any problems in the no loss case, including sub-bitstream extraction and layer switching

- Understanding of what picture coding patterns were disallowed

- Error resiliency aspects: prioritize good POC behavior for typical use cases, and prioritizing when the decoded pictures themselves were likely to be useful

  - POC corruption caused by picture loss should be detectable

  - POC problems shouldn’t persist beyond sample accuracy problems, after a loss

- Specification consistency

Additional evaluation criteria:

- POC values can be determined for feedback based error recovery

Discussion regarding these criteria for the contributions:

- Understanding of what picture coding patterns were disallowed

  - For modification of existing design:

    - POC reset can only occur for TL0 pics, other proposals do not have that restriction

    - There must be a sub-layer reference picture in each enhan layer when there is an IDR pic in the base layer, which would disable discarding of CL-RAS pictures in layer-wise startup procedure or restrict sub-sequence extraction.  (This restriction may be difficult to specify.) POC MSB reset may solve this problem.

  - O0176:

    - When an IRAP is in the enhan layer (which only occurs for TL0 pics), there must be a base layer pic in the same AU, which is not RADL, RASL, or sub-layer non reference, and does not have discardable_flag set.  (It was suggested that this restriction may be undesirable for use with interlaced field coding)

    - Also, some restrictions are placed on adaptive resolution using the single_layer_for_non_irap_flag use case, which may make the feature unusable with the current proposal.  Proponents believe this ARC problem is solvable with a modification to the proposal.

    - No BLA pictures in the enhan layer when cross_layer_irap_flag is not set

  - O0275:

    - Switching must occur within the POC LSB cycle.  Use with ARC may be problematic, but proponent asserts that the problem is solvable.

    - Either a restriction needs to be made for leading picture following a base layer IDR w/o a enhan layer picture, or the proposal needs to be modified, perhaps by adding an additional constraint or by changing the process to address

- Error resiliency aspects: prioritize good POC behavior for typical use cases, and prioritizing when the decoded pictures themselves were likely to be useful

  - POC corruption caused by picture loss should be detectable

  - POC problems shouldn’t persist beyond sample accuracy problems, after a loss

    - Proponents made assertions, but time constraints limited discussion to explore the accuracy of the assertions

    - O0176: Proponent asserts that POC recovery would not persist beyond sample accuracy being corrected.  (further clarification)

    - O0275: Proponent asserts that as long as reference picture predictions preceding the AU containing the base layer IDR are not used, the POC recovery occurs at the same time as the sample recovery.   Proponent asserts that if the enhan layer allowed to use reference pic prediction preceding the AU containing the base layer IDR, both the sample and POC values could be corrupt, the layerwise POC would be accurate at the same time the sample POC would become accurate.

Further study encouraged in an AHG, considering these evaluation criteria.

- JCTVC-O0343 MV-HEVC/SHVC HLS: Summary of contributions on POC [Y.-K. Wang (Qualcomm)]

This document provides a summary of the seven contributions in the agenda category “POC alignment and derivation”, section 6.4.2 of the JCT-VC meeting notes, and item 3 of JCTVC-O0223.

Contributions JCTVC-O0176 and JCTVC-O0275 both propose to remove the current POC resetting mechansim and replace it with a different mechanism. The other five contributions try to improve the current POC resetting mechansim. It is therefore suggested to first discuss JCTVC-O0176 and JCTVC-O0275. If one of the two (main proposals) can be agreed, then there is no need to discuss the other five contributions.

Among the six contributions based on the existing POC resetting mechanism, proposals in JCTVC-O0117, JCTVC-O0211, and item 3 of JCTVC-O0223 are indenpendent from each other and of other proposals. And decsions for these proposals should be relatively easy. Thus, these three are suggested to be discussed before the other three based on the existing POC resetting mechanism.

Proposals in JCTVC-O0128, JCTVC-O0140, and JCTVC-O0213 are related to each other, as both JCTVC-O0128 and JCTVC-O0213 identify a bug of the existing POC resetting mechanism and proposes a bug fix, and both JCTVC-O0140 and JCTVC-O0213 propose a POC MSB reset flag.

Several contributions are concerned about error resilience associated with POC reset.

JCTVC-O0117 On POC Alignment [S. Deshpande (Sharp)]

This document proposes refinements for the cross layer POC alignment. Following refinements are proposed.

- Modification of the PicOrderCntVal of prevTid0Pic which is the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture when poc_reset_flag is equal to 1 for the current picture.

- Modification to the decoding process for reference picture set to utilize derived PicOrderCntVal & (MaxPicOrderCntLsb-1) instead of slice_pic_order_cnt_lsb.

It is asserted that the proposed refinements are necessary for the correct cross-layer POC alignment.

In the revision of this document it is shown how the proposed refinement related to PicOrderCntlVal of prevTid0Pic works when combined with other JCTVC proposals related to POC at the Geneva meeting.

Agreed, for both proposals.,

JCTVC-O0128 MV-HEVC/SHVC HLS: On poc reset for long-term reference pictures [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

When poc_reset_flag is set equal to 1, the picOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal of current picture and then PicOrderCntVal of current picture is set equal to 0. It is asserted that, in the decoding process for reference picture set, the picture order count values, PocLtCurr and PocLtFoll, are incorrectly derived when poc_reset_flag is set equal to 1. This contribution proposes two alternative solutions to derive the correct PocLtCurr and PocLtFoll values for deriving the RPS when poc_reset_flag is equal to 1.

The proposed solution reverts to the design before it was changed at the last meeting, and can still calculate the POC value incorrectly under some situations.

JCTVC-O0140 MV-HEVC/SHVC HLS: Alignment of picture order counts [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

- Improvement of POC value derivation process for poc_reset_flag

  - PicOrderCntOffset is used for POC value derivation, for cases that poc_reset_flag is reset by systems.

- On POC values of IDR pictures with nuh_layer_id greater than 0

  - Signalling slice_picture_order_cnt_lsb for IDR pictures with nuh_layer_id greater than 0 is removed.

JCTVC-O0176 HLS: Error robust POC alignment [R. Sjöberg, J. Samuelsson, R. Yu (Ericsson)]

Move the POC reset functionality from the enhancement layer to base layer IDR pictures.  Use the slice extension for IDRs in the base layer.

The case where the enhancement layer pic has an IDR and a base layer was not present would be disallowed.

POC of an AU with an enhan layer IDR pic is not automatically reset to 0, but is set equal to the POC of the base layer.

It was questioned if the rewriting process would be impacted by this.

Proposal 2 is an editorial issue, which is referred to the editors.

A problem has been identified with the current design when sub-bitstream extraction is done.

Question about access unit boundary detection with this proposal.  When the base layer has an IDR, if the enhan layer derives its POC from the base layer, how will the decoder know if the enhancement layer picture is part of the same access unit or the next access unit?

JCTVC-O0211 MV-HEVC/SHVC HLS: On cross-layer POC alignment for layer-wise startup [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

According to the current SHVC and MV-HEVC draft text, in layer-wise startup decoding process when poc_reset_flag of non-IRAP picture in the initial IRAP access unit is equal to 1, PicOrderCntVal of the non-IRAP picture in the initial IRAP access unit has to be derived before setting PicOrderCntVal of the non-IRAP picture to 0. This proposal asserts that there is a problem to derive PicOrderCntVal of the non-IRAP picture in the initial IRAP access unit because prevPicOrderCnt does not exist. Three alternative solutions are proposed to solve the problem.

Proposal corrects an ambiguity in the spec.

Have identified a problem, and we intend to fix it in some way, but need to decide upon which solution alternative to do.

Agreed, with exact editorial expression delegated to editors.

JCTVC-O0213 On picture order count [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang, Hendry (Qualcomm)]

This contribution presents an improved design for the signalling and derivation of picture order count in SHVC and MV-HEVC. It is proposed that two flags indicating reset of POC LSB and POC MSB be signalled in the slice header, in place of the one flag that is currently signalled for the POC reset. Additionally, a POC reset information SEI message is proposed to help recover the POC derivation process when pictures containing POC reset information is lost.

Proposes to add another POC reset flag in the num extra slice header bits. We have already used 3, so this would be the 4th of the  bits.  The bits could instead be moved to the slice header extension, but that would be less convenient for a splicer.   Defer decision until error resiliency issue decision made.

2nd proposal proposes an SEI message to indicate POC reset.

A poc_reset_value syntax element indicates the absolute POC value to which the POC reset applies.  There may be random access issues in which the encoder’s POC MSB is not known by the decoder if the encoder not required to use POC reset flag for all CRA pictures. Depends on whether the CRA picture is required to set the POC MSB to 0.

JCTVC-O0223 MV-HEVC/SHVC HLS: Comments on latest MV-HEVC and SHVC draft specs [K. Rapaka, Y. Chen, A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late]

This document discusses the following issues in MV-HEVC Draft Text 5 and SHVC Draft Text 3 and proposes corresponding solutions: 1) cross-layer alignment of picture type, 2) unnecessary restrictions on the semantics of poc_reset_flag, 3) POC resetting at an AU that does not have all pictures present, 4) support of larger numbers of views, 5) persistence scope of an active SPS, and 6) coding of direct_dep_type_len_minus2.

Proposes to define a POC anchor picture, and make a constraint that POC reset can only be done when all layers are present. A consequence of this proposal would be to disallow a POC reset in a base layer when a corresponding enhancement layer picture is not present. There may be interaction with sub-bitstream extraction or layer-wise startup, e.g. when not all of the original layers are present in the bitstream.

The VPS is not rewritten when sub-bitstream extraction is used,

It was suggested that other parts of the specification may have problems with sub-bitstream extraction, in that there may be a question about how the decoder determines which layers are present in the bitstream.

Further study encouraged.

JCTVC-O0275 MV-HEVC/SHVC HLS: on POC value derivation [M. M. Hannuksela (Nokia)]

It is asserted that the contribution presents a design that enables non-aligned IRAP pictures across layers, is error-resilient, does not change the POC values of the base layer and keeps the POC values static during their persistence. The proposal includes the following ingredients:

- Slice headers of enhancement-layer slices optionally contain a layer-wise delta POC value (layer_delta_pic_order_cnt_minus1), which is intended to be used in the first picture of each layer after a base-layer IDR picture.

- A layer-wise POC value, LayerPicOrderCntVal is derived for each enhancement-layer picture based on layer_delta_pic_order_cnt_minus1, if present, and based on PicOrderCntVal differences, otherwise.

- The function DiffPicOrderCnt( ) is specified to use LayerPicOrderCntVal values.

An independent aspect of the proposal is to reset the PicOrderCntMsb value for the first picture of each layer after the layer-wise start-up process has been initiated. This ensures a “clean” reset of the POC values for each layer after a splice point triggering the layer-wise start-up process.

Version 2 of the contribution includes an illustrative figure and updates the specification text to handle properly an access containing a base-layer IDR picture while some enhancement layer pictures are absent as well as enhancement-layer IRAP picture POC derivation.

If cross layer irap alignment flag not set, would either need to use different PPSs within the CVS, or would require that extra bits be present in all slice headers to send the syntax.

Has some inter-layer dependency.

Treats IDR in the enhancement layer as a normal enhancement layer, in terms of POC derivation.  If rewriting of simulcast enhancement layer, constraints are imposed on the base layer POC values.

Asserted benefits: base layer POC unchanged, improved error resiliency although the exact conditions under which it would be improved is unclear.

Asserted that current design forces the output process to consider POC reset.

A question about how enhan layer POC is derived when IRAP in the enhan layer.

## Signalling of cropped inter-layer reference

JCTVC-O0098 AHG9: On signaling of scaled reference layer offsets [K. Ugur, M. M. Hannuksela (Nokia)]

Scaled reference layer offsets enable enhancing only a region of the base layer picture with the enhancement layer. In the current SHVC working draft, the scaled reference layer offsets are signalled only for direct reference layers (i.e. layers that may be used for inter-layer prediction). In order to more efficiently support additional ROI use-cases, we propose to change the SHVC design so that scaled reference layer offsets could be signalled for any layer.

Allows to align layers that are not directly dependent upon each other for post-processing/display operations, especially for auxiliary pictures.

We might want to make an editorial change to the variable names if the other layer is no longer a ref layer.  Agreed.

## Motion and prediction constrained SEI messages

JCTVC-O0063 HLS: Extensions to Temporal Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami, T.Suzuki (Sony)]

This contribution proposes an extension to a temporal motion-constrained tile sets SEI message to indicate the level information for a decoder to decode each defined motion-constrained tile set. The proposal provides flexibility for HEVC tile structure to be applied for various applications such as in interactive UHDTV application, dynamic high-quality zoom-in application and interactive on-demand e-learning etc. The idea is initially proposed in JCTVC-N0117. This contribution further clarifies the specification text on the definitions of level constraints for motion-constrained tile sets.

Proposal applies to single layer design (version 1).

Proposal sends a level indicator for each tile set.

It is noted that the existing definition of a motion constrained tile set is not required to represent a rectangular region, although it contains rectangular tiles.  This flexibility could be useful to ROI applications. There is also no constraint about position within slices of tiles in a tile set.  Hence a tile set could include tiles from more than one slice.

The contribution proposes using the bounding rectangle for the width and height calculations used for the level size constraint.  Language is improved from JCTVC-N0117, but the contribution does not provide language to impose a bitrate constraint for the tile set, as expressed in Table A-2.

Further study, especially regarding how to express the bitrate level constraint on the tile set.

JCTVC-O0255 SHVC / MV-HEVC HLS: On motion and inter-layer constrained tile set SEI messages [Hendry, K. Rapaka, Y.-K Wang (Qualcomm)]

This contribution discusses some issues of the current design of the motion constrained tile set (MCTS) SEI message and the inter-layer constrained tile set (ILCTS) SEI message, and proposes three bitstream restrictions to be incorporated into the semantics. The proposed restrictions are as follows:

- For both the MCTS and the ILCTS SEI messages, any two tiles in each tile set, shall either be neighboring to each other or the region between them belong to other tiles in the same tile set.

It is noted that the tiles in a tile set are not required to be contiguous.  No action on this point.  There is not a clear benefit of restricting this flexibility.

- For both the MCTS and the ILCTS SEI messages, when there is more than one rectangular region in a tile set, the rectangular regions shall be signalled in increasing order of top_left_tile_index[ i ][ j ].

No action on this point. There is not a clear benefit of restricting this flexibility.

- The ILCTS SEI message shall not be present when extended scalability is used.

It is asserted that the existing design has a problem with extended spatial scalability, but a participant questioned if the current design has a problem for some use cases with extended spatial scalability.

## Other SEI messages

JCTVC-O0177 MV-HEVC/SHVC HLS: On Layers Not Present SEI message [Jung Won Kang (ETRI), Jinho Lee, Hahyun Lee, Jin Soo Choi, Truong Cong Thang (UoA)]

Currently the layer not present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. This contribution proposes to describe the (temporal) sub-layers which are not present in a CVS. The usefulness of the proposal is shown with some practical use cases.

Same concept was previously proposed last meeting, but additional use cases were described.

Assumes profile/tier/level will signaled for multiple operation point corresponding to different values of highest temporal id.

The proposal sends a revised maximum temporal sub-layers.  A participant noted that the actual number of temporal sub-layers present may be useful information, but would be difficult to derive and specify.

JCTVC-O0224 MV-HEVC/SHVC HLS: On Signalling of random accessibility for IRAP pictures in non-IRAP AUs [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian, Y. Chen, J. Chen, M. Karczewicz (Qualcomm)]

This contribution proposes two aspects related to random accessibility of IRAP pictures in non-IRAP access units. In particular, the first aspect proposes to indicate the presence of IRAP pictures in an access unit and the second aspect proposes to indicate which pictures in previous access units need to decoded in order to random access to the layer of the IRAP picture from the access unit.

Two SEI messages are proposed.

One proposed SEI message  provides an indication of an upswitch point.  The proposed syntax only includes a flag to indicate that an IRAP exists in a layer.  It was suggested that it might be preferable to indicate which layer or layers contains an IRAP.  The use case for the SEI message was unclear.

The second proposed SEI message indicates how many pictures from a lower layer (pre-roll) need to be decoded before being able to decode the higher layer.

It was suggested that this message describes a systems functionality, and that the file format may already provide the indications of the second proposed SEI message, so it is unnecessary.  The usefulness was questioned without a definition of the corresponding decoding process of the pre-roll pictures in the non-conforming bitstream.  The question was raised if we want to provide hints to a file format writer with an SEI message.  No action on this point.

JCTVC-O0197 HLS: Non-significant tile set for tiled streaming with single layer HEVC extensions [C. Auyeung (Sony)]

The temporal motion constrained tile sets SEI message in JCTVC-N0383 was adopted for HEVC range extension text specification. This SEI message can be used to facilitate tiled streaming where different clients may request the server to send only the requested regions of interest to the clients with minimal overhead after the video is encoded. This contribution proposes to modify the temporal motion constrained tile sets SEI message to signal to the client that the client decoder does not need to decode and display the pixels outside the regions of interest so that the area outside the regions of interest can be coded efficiently to reduce network bandwidth and to reduce decoder computation and memory requirements.

The semantic language in the contribution is unclear.  It is noted that significant tile sets indication is already included in the SEI message.

Perhaps the semantics for the existing syntax could be revised, without need for a change to syntax.

Further study to consider if signaling through external means may be appropriate or semantics changes to the existing syntax.

JCTVC-O0358 Proposal for Supporting Optional Overlays with MV-HEVC [N. Stefanoski, O. Wang, A. Smolic, T. Szypulski] [late]

Previously submitted to JCT-3V as JCT3V-F0057.

In this document, it is proposed to realize with MV-HEVC a new functionality called Optional Overlays. Optional Overlays is a functionality which enables viewers to turn on and off single overlay elements, like score bars, information panels, news tickers, subtitles etc. on their screens. To realize this functionality, it is proposed to represent 2D video and overlay data as separate views with MV-HEVC and to specify an SEI message which (i) signals the type of data represented with MV-HEVC, and which (ii) provides information that is required to identify singe overlay elements. Syntax and semantics of such an SEI message are proposed in this document. It is proposed to adopt the SEI message in the MV-HEVC specification to enable the functionality of Optional Overlays.

Proposes to code the entire picture, even when only a small region is contained within the overlay.

Need 3 different auxiliary pictures.

- Overlay picture: Actual overlay picture, containing composite of all overlay components

- Label: Mask to indicate positions of each of N masks, e.g. value of 1 for all pixels in the 1st individual overlay component.

- Alpha (optional)

SEI message to describe the overlays.  Includes a label offset, because label picture is compressed. The encoder can control these parameters.

May want to allow an overlay per frames.

It is asserted to be advantageous to have the overlay data temporal synchronized with the video.

MPEG LASeR has some relationship.  A participant noted that MPEG LASeR is used in Korea for mobile television.

Change to existing design required to support: define 3 auxiliary picture types, and add an SEI message.

Revisit in joint VC/3V session to determine if sufficient interest for feature, and consider possible overlap with existing MPEG standards (especially LASeR).