| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0353r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0253r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Summary of Contributions on inter-layer dependency signalling and derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document presents a summary of proposals related to inter-layer prediction. There are 7 contributions in this section, listed below:

- JCTVC-O0061 MV-HEVC/SHVC HLS: Alternative colPic indication [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

- JCTVC-O0093 AHG9: On inter layer dependency [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

- JCTVC-O0120 On Inter-layer Reference Picture Set [S. Deshpande (Sharp)]

- JCTVC-O0138 MV-HEVC/SHVC HLS: On inter-layer dependency signalling [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

- JCTVC-O0225 MV-HEVC/SHVC HLS: On inter-layer RPS derivation and sub-layer inter-layer dependency [K. Rapaka, Y.-K. Wang, J. Chen, V. Seregin, Hendry, M. Karczewicz (Qualcomm)]

- JCTVC-O0254 MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling [Hendry, Y. Chen, Y.-K Wang (Qualcomm)] [late]

- JCTVC-O0271 SHVC: On direct_dependency_flag [K. Sato (Sony)]

- JCTVC-O0096 AHG9: On high level syntax [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

- The following order is suggested for discussion of topics.

- Motion-only inter-layer prediction

  - JCTVC-O0093 (proposes to disallow the option of Motion-only inter-layer prediction and change syntax elements)

  - JCTVC-O0061 (proposes improved signaling of collocated_ref_idx when colPic is a motion-only inter-layer prediction picture)

  - Item 6 of JCTVC-O0223 is related to fixed length signalling of direct_dep_type_len_minus2

- Inter-layer reference picture set definition

  - JCTVC-O0138 Proposal 1 proposes to change the IL-RPS to indicate all pictures that are used for reference for the current picture or future pictures in the same AU.

- Improvements to current signaling of inter-layer reference picture signaling

  - JCTVC-O0120 (Related to indicating reference layer in current AU flag under some conditions)

  - JCTVC-O138 Proposal 2 (Item 1condition signaling of max_one_active_ref_layer and all_ref_layers_active_flag) and JCTVC-O0096 Proposal 3.

    - The inference value of max_one_active_ref_layer is different in these two proposals.

  - JCTVC-O138 Proposal 2 Item 2 (signal max_one_active_ref_layer and all_ref_layers_active_flag for all layer)

  - JCTVC-O138 Proposal 3 (Related to highest_ref_layer_active_flag)

  - JCTVC-O0225 Proposal 1 (Related to semantics of all_ref_layers_active_flag)

  - JCTVC-O0225 Proposal 2 (Signaling max_tid_il_ref_pics for each layer and each layer that refers to this layer)

  - JCTVC-O0254 (IL-RPS candidates)

- direct_dependency_flag and layer sets

  - JCTVC-O0271 ( related to semantic constraint on layer_id_included_flag).

- Revision 1 of this document includes proposal 3 from JCTVC-O0096.

- Revision 2 of this document includes table containing bit count results from JCTVC-O0225 and motivation for the proposal items in JCTVC-O138.

# Summary of each proposal

## JCTVC-O0061 MV-HEVC/SHVC HLS: Alternative colPic indication [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

  - Abstract: The syntax, alt_collocated_indication_flag and collocated_ref_layer_idx, were removed due to possible low-level implementation changes and the lack of coding efficiency improvement ground. This proposal proposes an enhanced alternative indication mechanism which could produce short cut for motion only inter-layer picture without low-level changes and syntax changes. The alternative method could achieve bit saving up to 6 bits per slice (4 bits per slice if the length of RefPicList[] is 4), assuming that the motion only inter-layer picture is stored in the last position of RefPicList and selected for colPic.

  - Motivation/Problem: When motion-only inter-layer reference picture is used as the collocated picture, it is most likely included at the end of the ref-pic list. Signalling collocated_ref_idx to indicate this collocated picture is claimed to be made more efficient.

  - Proposal: Consider list of motion-only inter-layer reference pictures (ILRPs). When collocated_ref_idx is used to index motion-only ILRP, index it to the list of motion-only ILRPs rather than the reference picture list and infer the correct value of collocated_ref_idx; other cases signalling of collocated_ref_idx remains the same.

    - Bit count analysis given for example where motion-only reference picture is present at the end of the RPL and it used as the collocated picture.

| Length of RefPicList[] | collocated_ref_idx by conventional definition [ref_idx for RefPicListX] | collocated_ref_idx by proposed definition [ref_idx for MotionRefLayerId] | bits saving |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 0 (1 bit – 1bit) |
| 2 | 1 | 0 | 2 (3 bit – 1bit) |
| 3 | 2 | 0 | 2 (3 bit – 1bit) |
| 4 | 3 | 0 | 4 (5 bit – 1bit) |
| 5 | 4 | 0 | 4 (5 bit – 1bit) |
| 6 | 5 | 0 | 4 (5 bit – 1bit) |
| 7 | 6 | 0 | 4 (5 bit – 1bit) |
| 8 | 7 | 0 | 6 (7 bit – 1bit) |
| 9 | 8 | 0 | 6 (7 bit – 1bit) |
| 10 | 9 | 0 | 6 (7 bit – 1bit) |
| 11 | 10 | 0 | 6 (7 bit – 1bit) |
| 12 | 11 | 0 | 6 (7 bit – 1bit) |
| 13 | 12 | 0 | 6 (7 bit – 1bit) |
| 14 | 13 | 0 | 6 (7 bit – 1bit) |
| 15 | 14 | 0 | 6 (7 bit – 1bit) |

## JCTVC-O0093 AHG9: On inter layer dependency [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

- Abstract: This contribution proposes constraint and syntax modifications to simplify the high level signaling of inter-layer dependency type. The constraint is to mandate all direct dependent layers are used for inter-layer sample prediction. Based on this constraint, two options are proposed to simplify the current layer dependency signaling: (1) removing syntax element indicating dependency type length and merging the dependency flag syntax and the dependency type syntax together; (2) removing dependency type syntax and signaling inter-layer motion prediction using a new flag.

- Motivation/Problem: It is asserted that motion-only pred without sample pred is not useful. The value of direct_dependency_type can be used to indicate (a) sample-only prediction, (b) motion-only prediction, and (c) sample and motion prediction from a direct reference layer.

- Proposal

- Remove the option to do motion-only prediction from a reference layer. Only permit sample-only prediction, or both sample and motion prediction.

- Two ways to realize the above removal are proposed:

      - Remove direct_dep_type_len_minus2, and combine signalling of direct_dependency_flag and direct_dependency_type into one ue(v) coded syntax element direct_dependency_type to indicate no dependency, sample-only prediction, and sample and motion prediction.

      - Remove direct_dep_type_len_minus2 , and replace signalling of direct_dependency_type by il_motion_prediction_flag that is conditioned on direct_dependency_flag.

- Item 6 of JCTVC-O0223 is related to fixed length signalling of direct_dep_type_len_minus2.
JCTVC-O0059 and JCTVC-O0092 had an item related to direct_dependency_type.

## JCTVC-O0120 On Inter-layer Reference Picture Set [S. Deshpande (Sharp)]

- Abstract: It is asserted that when different layers have different frame rates and when max_one_active_ref_layer_flag is equal to 1 or  NumDirectRefLayers[ nuh_layer_id ]  is equal to 1 or  all_ref_layers_active_flag is equal to 1 the decoder is not able to distinguish between the case that an AU had no coded picture for a direct reference layer of a current layer due to that picture not existing in the bitstream (e.g. due to the reference layer having different frame rate) versus the case that the coded picture for the direct reference layer of a current layer was lost during transmission. This document proposes solution to distinguish between these cases. Two alternative solutions are proposed.

    - A slice segment header signalling and modification to the decoding process for inter-layer reference picture set is proposed.

    - A VPS signalling and other semantics change is proposed

- Proposal: Two options (Proponent prefers Option 1)

- Option 1: When one of the above conditions is true and current picture is EL, then, in addition to current signalling, (A) signal a flag for each “active reference layer” to indicate whether that picture is present in AU or not, and (B) change the decoding process of inter-layer RPS to use this flag.

- Option 2: In addition to the current signalling , (A) signal the maximum number of sub-layers in each layer of the bitstream in the VPS, and (B) If (maximum number of sub-layers – 1) in each reference layer is less than the TemporalId of the current picture (i.e. reference picture not present in that layer), do not include the reference layer picture in the inter-layer RPS for the current picture – includes changes to derivation of NumActiveRefLayers and semantics of inter_layer_pred_layer_idc[ ].

## JCTVC-O0138 MV-HEVC/SHVC HLS: On inter-layer dependency signalling [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

- Abstract: Some clean-ups or improvements of syntax related to inter-layer RPS or other layer dependency indication are proposed.

    - On inter-layer reference picture set

      - Removal of the inter-layer RPS

      - Or, improvements of the inter-layer RPS ( inter-layer DPB status instead of direct inter-layer references)

    - On all_ref_layers_active_flag and max_one_active_ref_layer_flag

      - Signalling those syntax for each layer

      - Optional signalling of max_one_active_ref_layer_flag depending on the value of all_ref_layers_active_flag

    - A new flag high_ref_layers_active_flag

      - Indication whether closer dependent layers are used for inter-layer prediction

- Three proposals included:

- Proposal 1: Related to ILP sets. The motivation is that this design is claimed that the proposed design would be useful for early picture marking, early detection of picture loss, and aligning the inter-layer reference picture set with the concept of the RPS.

      - Identify “indirect dependent layers” (to be clarified if it means all layers that may be used by current and/or future layers) for each layer.

      - Signal syntax element num_dependent_layers (specifying number of “indirect ependent layers”).

      - For each of num_dependent_layers, signal idc from the list of indirect dependent layers and used_by_curr_layer_flag (with some optimization/inference based on inter_layer_pred_enabled_flag and all_ref_layers_active_flag)

      - Changes are made to early picture marking process for sub-layer non-reference pictures based on whether picture is included in IL RPS..

- Proposal 2 has two aspects. The motivation for the first aspect is that it is claimed to be unnecessary for signaling both all_ref_layers_active_flag and max_one_active_ref_layer_flag always ,and that in come cases, the value of max_one_active_ref_layer_flag can be inferred from all_ref_layers_active_flag. For the second aspect, it is claimed that there are cases where it would be beneficial to signal all_ref_layers_active_flag and max_one_active_ref_layer_flag for each layer.

      - Signal flags all_ref_layers_active_flag and max_one_active_ref_layer_flag for each layer, in the VPS.

      - Condition the signalling of max_one_active_ref_layer_flag to be only when all_ref_layers_active_flag is equal to 0. When all all_ref_layers_active_flag equal to 1, max_one_active_ref_layer_flag is inferred to be equal to 0.

- Proposal 3: The motivation of this aspect is that in cases where the only the highest reference layers are used, current signaling is claimed to be inefficient.

      - Signal highest_ref_layer_active_flag that specifies default values of interlayer_pred_layer_idc to layer IDs of the highest direct reference layers.

## JCTVC-O0225 MV-HEVC/SHVC HLS: On inter-layer RPS derivation and sub-layer inter-layer dependency [K. Rapaka, Y.-K. Wang, J. Chen, V. Seregin, Hendry, M. Karczewicz (Qualcomm)]

- Abstract: This contribution proposes two changes for SHVC and MV-HEVC related to inter-layer RPS derivation process and signaling of sub-layer inter-layer prediction dependency. The first change targets to improve the derivation process of NumActiveRefLayerPics by considering the sub-layer inter-layer prediction restrictions indicated by max_tid_il_ref_pics_plus1, so that the pictures belong to sub-layers that are disallowed to be used for inter-layer prediction are not included in the inter-layer reference picture set when all_ref_layers_active_flag is equal to 1. The second change targets to have layer-specific signaling of max_tid_il_ref_pics_plus1, where max_tid_il_ref_pics_plus1 is signalled separately for the layers that use the current layer as a direct reference layer.

- Proposal 1

- Motivation: When max_tid_il_ref_pics_plus1 syntax element is used to specify for a particular layer that the pictures in that layer with TemporalId > max_tid_il_ref_pics_plus1 cannot be used for inter-layer prediction, all_ref_layers_active_flag (in the VPS) has to be set equal to 0 for the entire CVS and the inter-layer RPS may need to be explicitly signaled in the slice segment header for all pictures.

- Change semantics of all_ref_layers_active_flag  and the derivation process of NumActiveRefLayerPics so that the pictures belong to sub-layers that are disallowed to be used for inter-layer prediction are not included in the inter-layer reference picture set

- Bit count analysis provided for examples where one of the lower layer pictures is not allowed to be used for inter-layer prediction.

| Number of layers | Slice header bit saving |
| --- | --- |
| 2 | 1 |
| 3 | 3 |
| 4 | 7 |
| 5 | 9 |
| 6 | 16 |
| 7 | 19 |
| 8 | 22 |
| … | … |
| 63 | 373 |
| 64 | 379 |

- Proposal 2

- Motivation/Problem: Current signaling of max_tid_il_ref_pics_plus1 only allows to signal a picture that belongs to particular layer and particular range of value of TemporalId is not used for inter-layer prediction by all layers. This is claimed to be restrictive in some use-cases.

- Change signaling of max_tid_il_ref_pics_plus1, where max_tid_il_ref_pics_plus1 is signalled separately for the layers that use the current layer as a direct reference layer.

## JCTVC-O0254 MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling [Hendry, Y. Chen, Y.-K Wang (Qualcomm)] [late]

- Abstract: This contribution is a follow-up proposal of JCTVC-N0118 with some syntax and semantic changes. It is proposed to signal the candidates inter-layer reference picture sets in VPS and the candidates can be used by each slice. Bit-count Analysis of bit-count is also provided to show the effectiveness of the proposed method.

- Motivation/problem: In some use cases, all_ref_layers_active_flag cannot be set to 1 and ILP-RPS has to be signaled explicitly in slice segment header and this is claimed to be inefficient.

- Proposal: It is proposed to signal the candidates inter-layer reference picture sets in VPS and the candidates can be referred to by each slice. The changes in include the following

- In VPS, signal ILP RPS candidates (loop of flags for each direct dependent layer) in the VPS for each layer that has NumDirectRefLayers[ ] > 1, with signaling optimizations based on max_one_active_ref_layer_flag  and all_ref_layers_active_flag.

- In slice segment header, signal flag to indicate if ILP RPS taken from the VPS or explicitly signalled and remove signaling of num_inter_layer_ref_pics_minus1 and inter_layer_pred_layer_idc.

- Bit count analysis given for two cases:

      - CASE 1: Layer i has twice the frame rate as Layer i-1,

|  | Bits in VPS |  | Bits in SH |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Current | Proposed | Current | Proposed | Saving |
| EL 2 (per 4 picture) | 0 | 7 | 9 | 8 | 11.11% |
| EL 3 (per 8 picture) | 0 | 14 | 30 | 20 | 33.33% |
| EL 4 (per 16 picture) | 0 | 25 | 52 | 40 | 23.08% |

      - CASE 2: Layer i uses one active reference layer as layer i-1, and alternatively use layer i-2 and i-3 as the second active reference layer.

|  | Bits in VPS |  | Bits in SH |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Current | Proposed | Current | Proposed | Saving |
| EL 3 | 0 | 9 | 7 | 3 | 57.14% |
| EL 4 | 0 | 17 | 7 | 4 | 42.86% |
| EL 5 | 0 | 25 | 10 | 4 | 60.00% |

## JCTVC-O0271 SHVC: On direct_dependency_flag [K. Sato (Sony)]

- Abstract: In SHVC specification, layer sets can be defined by the syntax element “layer_id_included_flag[i][j]” in VPS. Direct dependencies between layers can be defined by “direct_dependency_flag[i][j]” in VPS_extension(), and indirect dependencies between layers can be inferred from the values of this syntax element. If there exists direct or indirect dependency between layerId A and B (A > B), it means that layer A cannot be decoded unless layer B is decoded. Then if a certain layer set contains layer A, it should also contain layer B. However there is no constraint in the semantics of direct_dependency_flag[i][j].

- This contribution proposes to add constraint on the semantics of direct_dependency_flag[i][j] associated with layer sets defined by layer_id_included_flag[i][j].

- Motivation/problem: The signaling of layer set information in the VPS allows cases where a particular layer is included in a layer, and one or more of its direct or indirect reference layers is not included in that layer set.

- Proposal:

- Identify (direct/indirect) dependency among layers, deriving the information from direct_dependency_flag.

- Add constraint in the semantics of layer_id_included_flag such that if one layer is included in the layer set, all its direct and indirect reference layer shall also be included in that layer set.

## JCTVC-O0096 AHG9: On high level syntax [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

- Proposal 3 is included in this section.

- Abstract: This contribution proposes a few syntax element modifications in VPS, SPS and slice segment header, including: (1) A default gating flag in VPS extension is proposed to condition the presence of direct dependency type; (2) An indicator in SPS is proposed to index into the representation format list to avoid explicit signaling of all representation parameters in SPS when possible; (3) Conditions on two syntax elements are proposed to remove signaling redundancy.

Based on the BoG discussion, a new section (section 3) is added to propose a simplified signaling method to update the representation format syntax in SPS.

- Motivation/Problem: (Proposal 3) The value of max_one_active_ref_layer_flag can be inferred based on the value of all_ref_layers_active_flag.

- Proposal: Condition the signaling of max_one_active_ref_layer_flag on the value of all_ref_layers_active_flag.