| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0255 |
| --- | --- |

| Title: | 3D-CE5 related: Cross check of bug-fix for DLT coding in 3D-HEVC (JCT3V-F0201) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yichen Zhang | Tel: Email: | +86-571-87953107 felixzyc@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal JCT3V-F0201. The source code was implemented based on HTM-8.0. We compiled, and ran the code of the cases listed in JCT3V-F0201 with only all-intra configurations. The cross-check has been completed successfully and the results match with those provided by proponents of JCT3V- F0201.

# Simulation description and results

This document reports the cross-check results of JCT3V-F0201 [1]. No mismatch was found during the crosscheck. Simulations are done under CTC [2] except for the configuration on DMM, SDC and DLT.

Only all-intra cases are tested for JCT3V-F0201 and there are three test cases for this proposal as follows:

- Test1: (A_DMM0_SDC0_DLT0) HTM8.0 with DMM, SDC, and DLT disabled;

- Test2: (A_DMM0_SDC0_DLT1) HTM8.0 with DMM, and SDC disabled;

- Test3: (P_DMM0_SDC0_DLT1) Proposed method with DMM, and SDC disabled.

The RD performance of Test 2 vs Test 1 is shown in Table 1.

The RD performance of Test 3 vs Test 1 is shown in Table 2.

Table 1: A_DMM0_SDC0_DLT0 vs A_DMM0_SDC0_DLT1 (All-intra config)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 151.7% | 99.2% |
| Kendo | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 134.3% | 97.5% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 77.3% | 98.9% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 84.7% | 100.2% |
| Poznan_Hall2 | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 138.7% | 98.7% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 88.9% | 99.2% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 85.6% | 100.4% |
| 1024x768 | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 121.1% | 98.5% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99.5% | 99.6% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 108.8% | 99.2% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 112.8% | 101.1% |

Table 2: A_DMM0_SDC0_DLT0 vs P_DMM0_SDC0_DLT1 (All-intra config)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 119.5% | 103.3% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100.7% | 102.3% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 83.2% | 98.9% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 83.4% | 99.8% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 118.5% | 99.8% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 85.6% | 99.5% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 84.0% | 102.7% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101.2% | 101.5% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 92.9% | 100.4% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96.4% | 100.9% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99.0% | 99.9% |

The simulation results match with those provided by the proponents of JCT3V- F0201. Please note that the encoding run time is not reliable for that the simulations are conducted on machines with different hardware. The source code is not inspected.

# References

- X. Chen, X. Zheng, Y. Lin, J. Zheng, “3D-CE5 related: Bug-fix for DLT coding in 3D-HEVC”, Geneva, CH, October, 2013.

- D. Rusanovskyy, K. Muller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-E1100, Vienna, AT, July. 2013.