| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0256 |
| --- | --- |

| Title: | Overview of tool parameters in 3D-HEVC Draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gerhard Tech | Email: | gerhard.tech@hhi.fraunhofer.de |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document provides an overview, in which syntax structures the coding tools in 3D-HEVC Draft 1 can be enabled and where related parameters are present.

# Abbreviations

T: For texture only.

D: For depth only.

X: For texture and depth.

(L): Per layer.

# Overview

| Coding tool | Syntax element(s) | VPS | SPS | PPS | SH | CU |
| --- | --- | --- | --- | --- | --- | --- |
| Camera parameters | cp_present_flag | X(L) |  |  |  |  |
|  | cp_in_slice_segment_header_flag | X |  |  |  |  |
|  | cp_precision | X |  |  |  |  |
|  | cp_scale, cp_off, ... | X(L) |  |  | X |  |
| Inter-view motion prediction | iv_mv_pred_flag | T(L) |  |  |  |  |
| Inter-view residual prediction | iv_res_pred_flag | T(L) |  |  |  |  |
|  | iv_res_pred_weight_idx |  |  |  |  | T |
| DoNBDV | depth_refinement_flag | T(L) |  |  |  |  |
| VSP | view_synthesis_pred_flag | T(L) |  |  |  |  |
| Depth Intra Modes | vps_depth_modes_flag | D(L) |  |  |  |  |
|  | depth_mode_parameters( ) |  |  |  |  | D |
| QT prediction | lim_qt_pred_flag | D(L) |  |  |  |  |
| DLT | dlt_flag, | D(L) |  |  |  |  |
|  | num_depth_values_in_dlt, dlt_depth_value | D(L) |  |  |  |  |
| Inter-view motion vector scaling | iv_mv_scaling_flag | X |  |  |  |  |
| Illumination compensation | slice_ic_enable |  |  |  | X |  |
|  | slice_ic_disable_merge_zero_idx_flag |  |  |  | X |  |
|  | ic_flag |  |  |  |  | X |
| Inter SDC | inter_sdc_flag |  |  |  |  | D |
|  | inter_sdc_resi_abs_minus1, inter_sdc_resi_sign_flag, ... |  |  |  |  | D |