| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0259 |
| --- | --- |

| Title: | BoG report on inter-view advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the implementation differences of inter-view advanced residual prediction from the two CE4 proposals, i.e., JCT3V-F0108 and JCT3V-F0123 [1].

# Introduction

This section describes the differences between these two proposals, as shown in Table 1.

It is noted that the aspect on ‘availability of temporal motion information’ doesn’t have any impact on the coding efficiency under the common test conditions.

Table 1: Difference between JCT3V-F0108 and JCT3V-F0123

|  | Blocks to be checked | Default temporal motion information | Availability of temporal motion inforamtion |
| --- | --- | --- | --- |
| JCT3V-F0108 | Two blocks: Top-left of the center position and bottom-right of the corresponding block | Motion vector: (0, 0) Reference index: 0 | There should be one picture in the current picture lists that is in the same AU of the reference picture |
| JCT3V-F0123 | One block: bottom-right of the center position of the corresponding block | N/A | No need to check |

# Recommendations

Based on the discussion from the BoG, the following harmonized method, as shown in Table 2, is recommended.

Table 2: Difference between JCT3V-F0108 and JCT3V-F0123

| Blocks to be checked | Default temporal motion information | Availability of temporal motion inforamtion |
| --- | --- | --- |
| One block: bottom-right of the center position of the corresponding block (as in JCT3V-F0123) | MV: (0, 0) (as in JCT3V-F0108) Reference index: the first available temporal reference picture index | No Check (as in JCT3V-F0123) |

# Participants

Table 3: Participants

| Participant | Contact | Email |
| --- | --- | --- |
| MediaTek | Jian-Liang Lin Kai Zhang Yu-Lin Chang | jl.lin@mediatek.com Kai.zhang@mediatek.com yulin.chang@mediatek.com |
| NTT | Shiori Sugimoto Shinya Shimizu | sugimoto.shiori@lab.ntt.co.jp shimizu.shinya@lab.ntt.co.jp |
| Qualcomm | Li Zhang | lizhang@qti.qualcomm.com |

# References

- K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei, “3D-CE4: Results on improved advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0108, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0123, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.