| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0261 |
| --- | --- |

| Title: | CE3-related: Crosscheck on removal of redundancy on VSP, ARP and IC (JCT3V-F0104) Test 5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao 5855 Pacific Center Blvd. San Diego, CA 92121 (USA) | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the cross check results of Test 5 in JCT3V-F0104 “CE3-related: Removal of redundancy on VSP, ARP and IC”, proposed by Sharp Corporation. The coding performance results obtained in this cross-verification match those provided by the proponents.

# Introduction

In JCT3V-F0104 [1], it is proposed to insert VSP merge candidate on the condition that ARP and IC is not enabled in the CU, and IC flag is also not signalled when ARP is enabled. Upon request, we received the source code from Sharp Corporation and performed crosscheck on the Test 5 of JCT3V-F0104. The source code is compiled and tested on Linux 64-bit OS with below macro setting,

#define ARP_DEP_MERGE             1

#define IC_DEP_MERGE              1

#define ARP_IC_DEP_FLAG           0

#define VSP_TO_DCP_CONVERT        1

Simulations are performed under common test condition (CTC), and detailed results are listed in below Table 1. The encoding, decoding and rendering time estimates are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Table 1: Simulation results of JCT3V-F0104 Test 5 under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.8% | -0.6% | -0.1% | -0.1% | -0.2% | 116% | 109% | 112% |
| Kendo | -0.8% | -0.9% | -0.2% | -0.1% | -0.2% | 84% | 87% | 84% |
| Newspapercc | -0.6% | -0.4% | -0.1% | -0.1% | -0.1% | 85% | 84% | 82% |
| GhostTownFly | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 86% | 84% | 85% |
| PoznanHall2 | -0.5% | -0.4% | -0.1% | -0.1% | -0.2% | 104% | 99% | 100% |
| PoznanStreet | -0.5% | -0.6% | -0.1% | -0.1% | -0.2% | 94% | 92% | 92% |
| UndoDancer | -0.1% | -0.3% | 0.0% | 0.0% | -0.1% | 96% | 91% | 92% |
| 1024x768 | -0.7% | -0.6% | -0.1% | -0.1% | -0.2% | 95% | 93% | 93% |
| 1920x1088 | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 95% | 91% | 92% |
| average | -0.5% | -0.5% | -0.1% | -0.1% | -0.1% | 95% | 92% | 92% |
| Shark | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% | 96% | 89% | 92% |

# References

- T. Ikai, Y. Yamamoto, T. Kim, S. Yea, "CE3-related: Removal of redundancy on VSP, ARP and IC", JCT3V-F0104, Geneva, CH, 25 Oct. – 1 Nov. 2013.