| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0262 |
| --- | --- |

| Title: | A fix to the mismatch of ATM and text for the disparity-based Skip and Direct modes in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-AVC, there exist a mismatch between the reference software and working draft (WD) which is related to the disparity-based Skip and Direct mode. In the software, the central point of the current block in the dependent view and the derived disparity vector are used to find the corresponding block in the base view. However, in the WD, the description to derive the position is incorrect. Therefore, in this contribution, it is proposed to fix the mismatch.

# Problem statement

A mismatch related to the disparity-based Skip and Direct mode could be found between the reference software, ATM-9.0 [1], and the test model of 3D-AVC [2]. The inter-view MV candidate derivation in the software is shown in Figure 1. The central point of the current block in the dependent view and its disparity vector are used to find the corresponding point in the base view. In the WD, however, the description to derive the position is incorrect. The modified text proposed to fix this mismatch is illustrated in the next section.

Figure 1.  The disparity-based inter-view MVP derivation in the software

# Draft text modification

The added/modified parts are highlighted in yellow color.

The removed parts are highlighted in purple color.

J.8.2.1.5 Derivation process for the motion vector in inter-view reference

Inputs to this process are mbPartIdx, subMbPartIdx, and listSuffixFlag.

Outputs of this process are the motion vector mvCorrespond and the reference index refIdxCorrespond.

Inter-view reference picture InterViewPic and an offset vector dV are derived as follows:

–	If TextureFirstFlag is equal to 0, the following ordered steps apply:

–	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

–	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

–	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

–	The following applies to derive an inter-view reference picture or inter-view only reference picture InterViewPic and to set the variable interViewAvailable:

interViewAvailable = 0
for( cIdx = 0; cIdx <= num_ref_idx_l0_active_minus1  &&  !interViewAvailable; cIdx++ )
	if ( view order index of RefPicList0[ cIdx ] is equal to 0 ) {
		InterViewPic = RefPicList0[ cIdx ]		(J-1)
		interViewAvailable = 1
	}

–	When interViewAvailable is equal to 1, the depth-based disparity value derivation process in subclause Error: Reference source not found is invoked with depthPic equal to DepthCurrPic, textureX equal to x1 + dx1 + dx2, textureY equal to y1 + dy1 + dy2, tBlWidth equal to the width the sub-macroblock partition CurrMbAddr\mbPartIdx\subMbPartIdx, tBlHeight equal to the height the sub-macroblock partition CurrMbAddr\mbPartIdx\subMbPartIdx, srcViewId equal to view_id and refViewId equal to the view_id of InterViewPic as inputs and the output assigned to dV[ 0 ] and dV[ 1 ] is set to 0.

–	Otherwise (TextureFirstFlag is equal to 1), the following ordered steps apply:

–	dV is set to (DvMBX, 0) and interViewAvailable is set to InterViewRefAvailable.

–	When interViewAvailable is equal to 1, InterViewPic is set to be the texture view component of the base view.

The refIdxCorrespond and mvCorrespond are set as follows.

–	If interViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond[ 0 ] and mvCorrespond[ 1 ] are both set to 0.

–	Otherwise, the following step applies in order.

–	The variable luma4x4BlkIdx is derived as  (4 * mbPartIdx + subMbPartIdx).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x1 + 7 + ( dV[ 0 ] >> 2 ), y1+ 7 + ( dV[ 1 ] >> 2 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dV[ 1 ] >> 6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dV[ 0 ] >> 6 ), and ( xRelative, yRelative ) is set equal to ( xCorrespond – (( xCorrespond>>4)<<4), yCorrespond  - (( yCorrespond>>4)<<4) ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ) ( xRelative, yRelative ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

# Conclusion

In this proposal, a fix for the mismatch between the software and WD for the disparity-based skip and direct modes in 3D-AVC is proposed.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. M. Hannuksela, Y. Chen, T. Suzuki, “3D-AVC Draft Text 6,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1002, April, 2013.

- D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki, “3D-AVC Test Model 7,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1003, July, 2013.

- J.-L. Lin, Y.-W. Chen, Y.-L. Chang, Y.-P. Tsai, Y.-W. Huang, S. Lei, “3D-CE5.a results on motion vector competition-based Skip/Direct mode with explicit signaling,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-A0045, July, 2012.