| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0365 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0264 |
| --- | --- |

| Title: | Study on future extension the maximum number of supported layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karsten Suehring Gerhard Tech Robert Skupin Yago Sanchez Thomas Schierl | Email: | karsten.suehring@hhi.fraunhofer.de gerhard.tech@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de yago.sanchez@hhi-extern.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | FhG HHI |  |  |

_____________________________

# Abstract

This contribution contains a study of possible future extensions of additional layers and the necessary hooks that are desirable to include in the first version of SHVC and MV-HEVC.

# Introduction

In track discussion a version of the syntax related to document JCTVC-O0200, JCTVC-O0137 and JCTVC-O0224 proposal 4 was requested that contains only minimal changes to a first version and some possible changes in a future extension.

# Syntax and semantics

In the following subsections syntax and semantics changes are shown for version one of SHVC and MC-HEVC to allow later extension of the supported number of layers. Section 2.1 contains common changes, section 2.2 contains changes that would be required for a modified VPS extension, section 2.3 contains a possible new VPS extension that would be an alternative to the syntax shown in section 2.2.

The following marking has been used:

changes in version 1

anticipated changes in future version

## NAL Unit header and VPS

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| if (nuh_layer_id == 63) |  |
| nuh_layer_id_extension | u(16) |
| } |  |

nuh_layer_id specifies the identifier of the layer.

The value of nuh_layer_id shall be in the range of 0 to 62. The value of 63 for nuh_layer_id is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 63 for nuh_layer_id in a NAL unit.

If nuh_layer_id is not equal to 63, the variable LayerId is set equal to nuh_layer_id. Otherwise (nuh_layer_id is equal to 63) the variable LayerId is determined as follows:

LayerID =  nuh_layer_id + nuh_layer_id_extension

When nal_unit_type is equal to AUD_NUT, the value of nuh_layer_id shall be equal to the minimum of the nuh_layer_id values of all VCL NAL units in the access unit.

When nal_unit_type is equal to VPS_NUT, the value of nuh_layer_id shall be equal to 0. Decoder shall ignore NAL units with nal_unit_type equal to VPS_NUT and nuh_layer_id greater than 0.

nuh_layer_id_extension specifies and additional value that can be used to detemine the value of the variable LayerID.

- Video parameter set RBSP

vps_max_layers_minus1 
[Ed. (KJS): semantics are missing in the extension for vps_max_layers_minus1. The value of 63 needs to be reserved for allowing the extension of nuh_layer_id].

## Modified VPS extension

- Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| if( vps_max_layers_minus1 == 63 ) |  |
| vps_ext_max_layers | u(16) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(34) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= VpsMaxNumLayers − 1vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nalu[ i ] layer_id_in_nuh[ i ] | u(6v) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= VpsMaxNumLayers − 1vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < VpsMaxNumLayers − 1vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= VpsMaxNumLayers − 1vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= VpsMaxNumLayers − 1vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

vps_ext_max_layers specifies the additional number of layers when vps_ max_layers is equal to 63. When not present vps_ext_num_layers is inferred to be equal to 0.

The variable VpsMaxNumLayers specifying the maximum number of layers in the extended layer range is set equal to ( vps_max_layers_minus1 + vps_ext_max_layers + 1 ).

## New VPS extension

- Video parameter set extension2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| vps_ext_max_layers | u(16) |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(4) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = vps_max_layers_minus1; i <= VpsMaxNumLayers − 1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nalu[ i ] | u(16) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( ExtNumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < ExtNumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = vps_max_layers_minus1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| vps_ext_number_layer_sets_minus1 | u(10) |
| vps_ext_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps__ext_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_ext_profile_present_flag[ i ] | u(1) |
| if( !vps_ext_profile_present_flag[ i ] ) |  |
| vps_ext_profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numExtOutputLayerSets = vps_ext_number_layer_sets_minus1 + 1 |  |
| vps_ext_more_output_layer_sets_than_default_flag | u(1) |
| if(vps_ext_more_output_layer_sets_than_default_flag ) { |  |
| vps_ext_num_add_output_layer_sets_minus1 | u(10) |
| numExtOutputLayerSets += vps_ext_num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numExtOutputLayerSets > 1 ) |  |
| vps_ext_default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numExtOutputLayerSets; i++ ) { |  |
| if( i > vps_ext_number_layer_sets_minus1 ) { |  |
| vps_ext_output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = vps_ext_output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumExtLayersInIdList[ lsIdx ] − 1; j++) |  |
| vps_ext_output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| vps_ext_profile_level_tier_idx[ i ] | u(v) |
| } |  |
| vps_ext_rep_format_idx_present_flag | u(1) |
| if(vps_ext_rep_format_idx_present_flag ) |  |
| vps_ext_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_ext_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = vps_max_layers_minus1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| if( vps_ext_num_rep_formats_minus1 > 0 ) |  |
| vps_ext_rep_format_idx[ i ] | u(4) |
| vps_ext_direct_dep_type_len_minus2 | ue(v) |
| for( i = vps_max_layers_minus1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| } |  |

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).