| - Joint Collaborative Team on 3D Video Coding Extensions - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0265 |
| --- | --- |

| Title: | Editorial improvements for texture and depth view packing SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Takanori Senoh | Tel: Email: | cheny@qti.qualcomm.com senoh@nict.go.jp |
| Source: | Qualcomm Incorporated and NICT |  |  |

_____________________________

# Abstract

This document provides editorial improvements for the depth view information SEI message in 3D-AVC.

# Relevant WD text changes in 3D-AVC

Texutre and depth view packing SEI message semantics

The texture and depth view packing SEI message informs the decoder that some view components of the output views contain samples of multiple distinct spatially packed constituent residual pictures. The output views are not suitable to be displayed directly and may be converted to the natural views that can be appropriate for direct display as indicated by the depth_type syntax element. This information can be used after the decoder output to appropriately rearrange the samples to produce natural views that are appropriate for display or to server for other purposes (which are outside the scope of this Recommendation | International Standard).

The depth view information SEI message, when present, shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence. This SEI message conveys information associated with the coded video sequence that outputs views that may be further processed to generate the natural views that are to be displayed.

In such a coded video sequence, there are at least two output views, wherein the base view is a natural view and a non-base view is a residual view. A residual view is not a natural view, instead it contains a sequence of difference pictures which are decoded view components of the residual view. Each difference picture is created by frame packing up to four constituent residual pictures.

Each constituent residual picture represents a picture generated by 2:1 decimation from the residual picture horizontally and vertically, starting from the top-left most sample.

Each constituent residual picture is considered as produced by pictures of two natural views, wherein one belongs to the view that is coded as the base view and used to predict the other picture in the other view of view A. The residual picture of such a process is considered as the residual picture associated with view A. The residual picture consists of the pixels which are not predictable by the base view A, namely occluded pixels and out-of-picture pixels in the base picture A. After decimation from the residual picture associated with view A, a constituent residual picture associated with view A is generated.

depth_type specifies the arrangement how distinct spatially packed constituent residual pictures are packed into view components. The values of depth_type greater than 0 are reserved for future use.

num_residual_texture_views_minus1 +1 specifies the number of constituent residual pictures packed to composite a difference picture which is a non-base texture view component. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that the number of constituent residual pictures packed to composite a difference picture which is a depth view component of a non-base view is equal to num_residual_texture_views_minus1 +1. residual_depth_flag equal to 0 specifies there is only one depth view.

When depth_type is equal to 0, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be equal to 0 in the active SPS RBSP for the coded video sequence and avc_3d_extension_flag shall be 0 in all VCL NAL units of the coded video sequence.

NOTE: These constraints disable tools for inter-view and inter-component prediction.

When depth_type is equal to 0, the coded video sequence shall contain two texture views, wherein the non-base texture view  is the only residual view containing texture information and is composited by spatially packed residual pictures for texture.

When depth_type is equal to 0, the number of coded depth views in coded video sequence shall be equal to 1 + residual_depth_flag.

Let nR be equal to num_residual_texture_views_minus1 + 2.

Base texture view (B) is the input from the central camera (camera ID equal to 3) as shown in Figure J-1. In the case nR equal to 2, camera ID=2 and 3 are used. In the case nR equal to 3, camera ID=2, 3 and 4 are used. In the case nR equal to 4, camera ID=1, 2, 3 and 4 are used. In the case nR equal to 5, camera ID=1, 2, 3, 4 and 5 are used. The other N-1 input texture views except for the base view are converted to nR – 1 horizontally and vertically half-size constituent residual pictures (R-x) and packed in the non-base texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. Their top-left co-ordinates of the constituent residual pictures in the difference picture of the residual view of width=W and height=H is shown in Table J-1 where W and H shall be even number. Each constituent residual picture represents an occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views by the GVD process. The GVD process is summarized in the following paragraph.

1     2     3     4     5

R-1   R-2    B    R-4   R-5

camera ID

GVD view

- Figure J-1 – Relation between camera ID and GVD texture views

- Table J-1 – Top-left corner co-ordinates of GVD residual views packed in a Residual view of width=W and height=H.

| R-1 | (W/2, 0) |
| --- | --- |
| R-2 | (0, 0) |
| R-4 | (0, H/2) |
| R-5 | (W/2, H/2) |

Base depth view (B) is a sample-wise median of the same coordinates of all input depth views and is associated with the base texture view (camera ID=3) as shown in Figure J-2. The other nR – 1 input depth views are converted to nR – 1 constituent residual pictures for depth of the non-base depth view component as specified in the next paragraph. They are packed in the non-base depth view in the same positions as the constituent residual texture pictures shown in Table J-1. Each constituent residual picture of the non-base depth view component associates with a constituent residual picture of the non-base texture view component in the same relative location.

When residual_depth_flag is equal to 0 and the number of input views is even (2 or 4), base depth view (B) is the center of input views (camera ID equal to 2.5).

Each constituent residual picture for depth represents the occluded area or the out-of- pictures area of the base depth view when it is projected to the N-1 input depth views.

Within an access unit, given a decoded view component of the base view and the decoded view component of the non-base view, which is a difference picture, the picture of the natural view, view A of the same time instance is generated as follows.

- A constituent residual picture associated with view A from a difference picture is firstly extracted out and 1:2 upsampled horizontally and vertically to a residual picture associated with view A.

- The decoded view component of the base view is used to produce the prediction picture associated with view A.

- A residual picture and the prediction picture of view A are composited to produce the picture of the view A.

Each constituent residual picture is 1:2 upsampled horizontally and vertically. Decoded texture picture of base view is directly used to generate a texture picture of target view. Upsampled constituent texture pictre in the residual view is used to fill the pixels in texture picture of target view, which are not filled by the texture picture of base view, namely occluded pixels and out-of-picture pixels in the base view.

[Ed. (MH/CY): camera parameters to be associated with the constituent residual pictures. The current camera parameters are associated ONLY for the coded views. The current design might need camera parameters to be transmitted in other metadata that is not supported in the current specification.]

Camera parameters for the constituent residual pictures in residual view is generated from the decoded camera parameters for the base view and the residual view.

1     2     3     4     5

(R-1)  (R-2)  B   (R-4)  (R-5)

camera ID

GVD depth

1       2       3       4

2.5

B

camera ID

(even)

GVD depth

wo/res.

- Figure J-2 – Relation between camera ID and GVD depth views