| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0268_r1 |
| --- | --- |

| Title: | BoG report on BVSP related issues in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the BVSP related issues in 3D-AVC.

# BVSP related issues in 3D-ATM

The original design of BVSP is to derive one disparity motion vector for each NxN block (e.g., N = 8), as shown in the following figure.

However, in the current ATM, the following problems are identified:

  - All the 8x8 blocks within one MB or one MB partition shall the same disparity motion vector which is derived from MB-level disparity vector derivation method.

  - The 8x8-level BVSP was disabled due to the integration of JCT3V-D0162, and the corresponding macro is named as ‘NOKIA_USTC_IVP_VSP_D0126’.

Therefore, we suggest making the following change:

  - Change the software to support 8x8-level BVSP

# BVSP related issues in 3D-AVC working draft

The following table summarizes the problems, suggested solutions and actions need to be taken.

Table 1: Issues and solutions related to 3D-AVC working draft

| Item | Problems | Suggested solution | Actions |
| --- | --- | --- | --- |
| 1 | In J.7.3.6.2, braces are missing | In J.7.3.6.2, the highlighted part should be added. if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) { ref_idx_l1[ mbPartIdx ] if( VspRefL1Flag[ mbPartIdx ] && slice_vsp_flag ) bvsp_flag_l1[ mbPartIdx ] } |  |
| 2 | The width and height of one MB partition is used in J.8.2.1.6. However, it is not clearly defined in J.8.2 which may invoke J.8.2.1.6. | In J.8.2, the highlighted part should be added. For each value of mbPartIdx, the variables partWidth and partHeight for the width and height of each macroblock partition or sub-macroblock partition in the macroblock are derived as follows: ... |  |
| 3 | In J.8.2, the depth information are specified only when DMVP is enabled without considering BVSP mode. | In J.8.2, the highlighted part should be added. When DepthFlag is equal to 0 and dmvp_flag or slice_vsp_flag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. ... |  |
| 4 | If DMVP is disabled while BVSP is enabled, 8.4.1 is invoked instead of J.8.2.1. | In J.8.2, the highlighted part should be added. – If nal_unit_type is equal to 21 and DepthFlag is equal to 0, the following applies: – If mb_alc_skip_flag is equal to 1 or mb_alc_flag is equal to 1, subclause Error: Reference source not found is invoked. – Otherwise, if dmvp_flag or slice_vsp_flag is equal to 1, subclause Error: Reference source not found is invoked. – Otherwise, subclause 8.4.1 is invoked. – Otherwise, ... |  |
| 5 | To decode one VSP PSkip mode, Sub-clause J.8.2.1.3 (i.e., Derivation process for luma motion vectors for VSP skipped macroblocks in P and SP slices) is invoked four times, however, it always return the same disparity vector. | The depth-based disparity value derivation process in subclause Error: Reference source not found is invoked with depthPic equal to DepthCurrPic, textureX equal to x1 + dx1+ shiftedX, textureY equal to y1 + dy1, tBlWidth equal to 168, tBlHeight equal to 168, ... |  |
| 6 | BVSP B_Direct_8x8 mode is not supported in 3D-AVC. | In J.8.2.1, the following changes should be made: vspFlag = !( sub_type[ mbPartIdx ] = = B_Direct_8x8 ¦ ¦ ( mb_type = = B_Skip && MbVSSkipFlag = = 0 ) ¦ ¦ ( ( mb_type = = B_Direct_16x16 ¦ ¦ sub_type[ mbPartIdx ] = = B_Direct_8x8 ) && !mb_direct_type_flag ) ) |  |
| 7 | To decode one VSP B_Direct_16x16, the MB is set to have four partitions and for each partition, sub-partition size is set to 4x4. However, 8x8 BVSP should be applied. There is no need to invoke the motion vector derivation process 4*4 times. | Set the sub-partition size equal to 8x8, by the following modified text: In subclause J.8.2, the highlighted part should be added. – Otherwise (mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8), the following apply: – If either MbVSSkipFlag or mb_direct_type_flag is equal to 1, subMbPartIdx is set to 0 and partWidth and partHeight are derived as: partWidth = 8 partHeight = 8 – Otherwise (both MbVSSkipFlag and mb_direct_type_flag is equal to 0), subMbPartIdx proceeds over values 0..3, and partWidth and partHeight are derived as: partWidth = 4 partHeight = 4 |  |
| 8 | For BVSP coded 16x8/8x16/8x8 (excluding B_direct_8x8), the whole partition share the same motion vector. | In subclasue J.8.2, the highlighted part should be added. The range of the macroblock partition index mbPartIdx is derived as follows: – If mb_type is equal to B_Skip or B_Direct_16x16 or if both mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, mbPartIdx proceeds over values 0..3. – Otherwise (mb_type is not equal to B_Skip or B_Direct_16x16), set variable vspPartFlag equal to 0, and the following apply: - if one of bvsp_flag_l0[ mbPartIdx ] or bvsp_flag_l1[ mbPartIdx ] is equal to 1, vspPartFlag is set to 1 and mbPartIdx proceeds over values 0..3; - otherwise, mbPartIdx proceeds over values 0..NumMbPart( mb_type ) − 1. For each value of mbPartIdx, the variables partWidth and partHeight for the width and height of each macroblock partition or sub-macroblock partition in the macroblock are derived as follows: – If mb_type is not equal to P_8x8, P_8x8ref0, B_Skip, B_Direct_16x16, or B_8x8, subMbPartIdx is set equal to 0 and the following applies: – If mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, or vspPartFlag is equal to 1, partWidth and partHeight are derived as: partWidth = 8 (J-1) partHeight = 8 (J-2) – Otherwise, partWidth and partHeight are derived as: partWidth = MbPartWidth( mb_type ) (J-3) partHeight = MbPartHeight( mb_type ) (J-4) | To be confirmed |

# Participants

Table 2: Participants

| Participant | Contact | Email |
| --- | --- | --- |
| MediaTek | Jian-Liang Lin Yu-Lin Chang | jl.lin@mediatek.com yulin.chang@mediatek.com |
| Nokia | Miska Hannuksela | miska.hannuksela@nokia.com |
| Qualcomm | Li Zhang | lizhang@qti.qualcomm.com |
| Samsung | JinYoung Lee | jinyoung79.lee@samsung.com |