| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F1012 |
| --- | --- |

| Title: | Draft 2 of MFC Conformance |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft of MFC conformance |  |  |
| Author(s) or Contact(s): | Peng Yin Dolby Laboratories, Inc. 432 Lakeside Dr. Sunnyvale CA 94085 USA Dong Tian MERL 201 Broadway Cambridge, MA 02139 | Email: | pyin@dolby.com tian@merl.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is draft 2 of new amendment to ITU-T Rec. H.264.1 | ISO/IEC 14496-4 adding conformance test of Multi-resolution Frame Compatible Stereo Coding extension of AVC.

In 10.6.5.7, add the following text at the end of the subclause:

A decoder that conforms to the MFC High profile at a specific level shall be capable of decoding all bitstreams in which all active MVC sequence parameter sets have any of the following:

- profile_idc equal to 134

- profile_idc equal to 128

- profile_idc is equal to 118 and constraint_set5_flag is equal to 1

- profile_idc equal to 100 or 77

- profile_idc equal to 88 and constraint_set1_flag equal to 1

- profile_idc equal to 66 and constraint_set1_flag equal to 1

and in which level_idc or the combination of level_idc and constraint_set3_flag for all active MVC sequence parameter sets represent a level less than or equal to the specific level. In addition to the bitstreams defined in Table-AMDxx-1, a decoder that conforms to the MFC High profile shall be capable of decoding the Main profile bitstreams specified in Table AMD6-1, and the High profile bitstreams specified in Table AMD 9-2.

After 10.6.6.xx. x, add the following text:

10.6.6.36 Test bitstreams – MFC High Profile

10.6.6.36.1 Test bitstream #MFCRFT-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with RPU Filter mode, using SbS base layer and default grid position.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views  with RPU filter mode.

10.6.6.36.2 Test bitstream #MFCRFT-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 0. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with RPU DC mode, using SbS base layer and default grid position.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views with RPU DC mode.

10.6.6.36.3 Test bitstream #MFCFLD-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. field_pic_flag is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with RPU field processing, using SbS base layer, default grid position, RPU filter mode, and each view component is coded as a field picture.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views with RPU field processing for field pictures.

10.6.6.36.4 Test bitstream #MFCFLD-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mb_adaptive_frame_field_flag  is equal to 1. field_pic_flag is equal to 0. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with RPU field processing, using SbS base layer, default grid position, RPU filter mode, and each view component is coded as an mbaff frame picture.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views with RPU field processing for mbaff frame pictures.

10.6.6.36.5 Test bitstream #MFCFLD-3

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mb_adaptive_frame_field_flag  is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with RPU field processing, using SbS base layer, default grid position, RPU filter mode, and each view component is coded as an mbaff frame or a field picture.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views with RPU field processing for mbaff frame or a field picture.

10.6.6.36.6 Test bitstream #MFCMFM-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 1. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with TaB base layer MFC format configuration, using default grid position and RPU filter mode.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views with TaB MFC format configuration.

10.6.6.36.7 Test bitstream #MFCGRD-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 0. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement view components and reconstruction of the enhanced resolution stereo views with non-default grid position configuration, using SbS base layer and RPU filter mode.

Purpose: Check that the decoder can properly decode the base and enhancement view components and reconstruct the enhanced resolution stereo views with non-default grid positions.

Add the following after Table AMD41.1:

Table AMD42.1 – Bitstreams for the MFC High profile

| Categories | Bitstream | Donated by | File Name | MFC High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| RPU Operational Modes | MFCRFT-1 | Dolby | MFCRFT-1 | X | 4.1 | 24p |
|  | MFCRFT-2 | Dolby | MFCRFT-2 | X | 4.1 | 24p |
| RPU Interlaced Coding Tools | MFCFLD-1 | Dolby | MFCFLD-1 | X | 4.1 | 50i |
|  | MFCFLD-2 | Dolby | MFCFLD-2 | X | 4.1 | 50i |
|  | MFCFLD-3 | Dolby | MFCFLD-3 | X | 4.1 | 50i |
| MFC Format | MFCMFM-1 | Dolby | MFCMFM-1 | X | 4.1 | 24p |
| Grid Position | MFCGRD-1 | Dolby | MFCGRD-1 | X | 4.1 | 24p |